AA4RE BBS now supports the HAL-4000 CLOVER card.  A typical port

would look like:

port

port_type               = HAL4000-CLOVER

addr                    = $360

max_connect             = 1

max_channel             = 2

call_set                = AA4RE

no_busy_fwd             = yes

max_pac                 = 230

speed                   = 9600

max_pending             = 1

monitor                 = yes

time_out                = 180

fwd_min                 = 0

minimum_class_allowed   = new_user

remote_sysop            = Y

upload_download         = Y

broadcast               = n

port_name               = Clover

monitor_color           = 121

disconnect_delay        = 10

default_forward_order   = TA

new_user_display_format = 0

remove_ssid_addresses   = y

endport

The following commands are available..

-- These commands take a parameter of a callsign

C  xxxxx  -- Connect normal to xxxxx

CN xxxxx  -- Same as C

CR xxxxx  -- Connect robust to xxxxx

I xxxxx   -- Set callsign

-- No parameters allowed on these commands

D         -- Disconnect

CQ        -- Start ARQ CQ

FEC       -- Begin FEC mode

ANSWER    -- Answer ARQ CQ

ID        -- Send CW ID

-- Query Commands (no parameters)

Q CSDN    -- Query channel spectral data (narrow)

Q CSDW    -- Query channel spectral data (wide)

Q CSTAT   -- Query channel statistics

Q LS      -- Query link state

Q VSB     -- Query voice subband

Q WAVE    -- Query clover waveform

Q DSPV    -- Query DSP software version

Q CSWV    -- Query controller software version

Q EPROMV  -- Query EPROM version

Q EPROMC  -- Query EPROM checksum

-- Monitor -- special parameters

M N       -- Monitor off

M xxxx    -- Monitor on.   xxxx can be one or more letters from list below:

L -- Link status

S -- Channel status

I -- Data from other QSOs but only when not connected

-- Waveform setting -- special parameters

WAVE CWID     -- These set the waveform

WAVE 2DFSM

WAVE 4DFSM

WAVE FSM

WAVE 2DPSM

WAVE BPSM

WAVE QPSM

WAVE 8PSM

WAVE 8P2A

WAVE 16PSM

WAVE 16P4A

WAVE SIZE nn  -- Set the blocksize to 17, 51, 85, or 255

WAVE FEC  nn  -- Set the error correction to 60, 75, 90, or 100 %

-- These commands set a numeric parameter

P n       -- Set chat threshold

RLRM n    -- Set robust link retry maximum

NLRM n    -- Set normal link retry maximum

CCBRM n   -- Set CCB retry maximum

VSB  n    -- Set Voice subband

BIAS n    -- Set bias

SCAN n    -- Set scan mode

-- These are on/off commands.  b = 1 or Y or YES or ON

b = 0 or N or NO or OFF

Y b       -- Allow connect

RCS b     -- Report channel stats

ARPA b    -- Automatic Remote Power Adjust

ACWF b    -- Adaptive Clover Waveform

CWID b    -- CW ID every ten minutes

LISTEN b  -- Enable monitor of FEC

EMS b     -- Enable Monitor Status

TEST b    -- Start/stop Clover test

*** Note to testers.  CLOVER support is still very very new.  This doc

only outlines the base set so I can get it out to you testers.  More

will come later.

*** The PCC.S28 and PCC.LOD files must be in the C:\PCC subdirectory!!!!!!