The answer to the question "What is the capacity of BB" is very complex

and depends on:

1) Processor speed

2) DOS Memory size (EMS doesn't count)

3) Number and type of ports.

4) Number of messages kept in BBS

The latter two eat up memory which pressures #2.  As far as I can tell,

here are some guidelines: 4.77 Mhz XT - 3 ports, about 6 connects. 10

Mhz AT - 4 ports, about 10 connects.  You can trade ports for connects.

No one has run more than 4 ports so am not sure what happends then.

I assumed full 640K.

The DRSI PC*PA cards are a special case because the act like one port.

I would say a regular XT could run 4 ports and 8 total connects before

resource exhaustion.

Some memory numbers:

Each port requires about 15-18K (Not true for PC*PA)

Each connect requires about 12K

Each BID requires 4 bytes

Each message needs 270 bytes