KISSINIT Version 1.00  (20. November 1993)

by Ren� Stange, DG0FT @DB0KG.DEU.EU

Translation G0KIU

Contents

1. General

2. Program Call and Options

3. Initialisation File

4. Application Example

5. Copyright and Usage Conditions

1. General

With many TNCs (e.g. PK232, KAM) the KISS-Mode must be activated by

special Commands, before they can be used with TFPCX.  TFPCX does

not offer anything for thia purpose. The Program KISSINIT can issue

a chosen Character Sequence via the Serial Port to the TNC and with

that undertake the switch into KISS,  also the Deactivation of KISS,

when the TNC is required for other uses.  Moreove The Usage of

KISSINIT is not restricted to TFPCX und KISS.

The Commands for the activation of the KISS-Mode will be undertaken

by a Text File (Standard  KISS.INI). They are different from TNC to

TNC and should be given in the TNC-Documentation. KISSINIT offers

the Possibility, of multi different Initialisation Sequences to be

contained in a File and the required variant called with the Start.

It would be sensible to create a kind of Library for the KISS-

Initialisations for the entire TNC-Types. The included File KISS.INI

already contains certain Example Sequences, which I have taken from

different Mailbox-Articles, however most could not be self tested.

If you have worked previously with TFPCR, can also directly take over

from it, the employed File AUTOKISS.CFG (renamed to KISS.INI or employ

Option -F. In the case where if it should give Problems, it can lie

with particular meanings a certain Character Sequence, but which should

seldom occur in Commands.

TFPCX may not be loaded at the Caling of KISSINIT, otherwise an Error

Message will be displayed. The KISS-Activation must arise before the

Start of TFPCX. If multi-TNCs are employed in parallel, so KISSINIT

must be called repeatedly for every TNC.

The Parameter for the Serial Port of the TNC must be installed by

KISSINIT as follows:

- 8 Datenbits

- 1 Stopbit

- no parity

2. Program Call and Options

With the Call of KISSINIT particular Parameters can be effected with

the Help of Command-Line-Options. All Options begin with a '-' and

will be separated one from another with a Space. Within an Option no

Empty Characters are allowed. Upper-/Lower- Case Characters will not

be differentiated. The following Options are possible (obtainable

with 'KISSINIT -H'):

Options:                                  Default

-PCOMn[:xxx]    COM port [address]      COM1

-Bnnnn          baud rate               9600

-Dnnn:nnn:nnnn  delay (char/line/exit)  30:300:2000 (ms)

-F<file>        init file               KISS.INI

-S<section>     select section in file

-K              kiss reset

-N              no messages

The Options have the following meanings:

-PCOMn[:xxx]  Selects the Serial Port (n = 1-4)

Here the Serial Port is issued to which the TNC is connected

(Standard  COM1). Normally the Port Address of the Port will be established

automatically, but also by choice can be issued directly in hexadeczimal.

Example:

KISSINIT -PCOM3:338

(TNC on COM3 with Port Address 338H)

-Bnnnn  Baudrate on the Serial Port (decimal)

Possible Values are 2400, 4800, 9600 (Standard), 19200,  38400  and

57600 Baud.

-Dnnn:nnn:nnnn  Safety Time (decimal in Milliseconds)

^   ^   ^

1)  2)  3)

Most TNCs require a certain Time for the processing of the received

Characters, for the execution of Commands (at the Line End) and for

Initialisation purposes. If no regard to this is taken, it can lead

to Character Loss or the TNC is not yet in the KISS-Mode, when TFPCX

is started. For this reason KISSINIT waits an installable Time after

each Character output, up to the next following Output or the

Program is completed.

Saftey Timen          Value Range   Standard

1) after each Character    0-999  ms     30 ms

2) after each Line         0-999  ms    300 ms

3) after the Transfer      0-9999 ms   2000 ms

The Standard Value for 1) and 2) generally should be sufficient,

but Value 3) must be increased in certain circumstances.

Example:

KISSINIT -D30:300:5000

The TNC2 with The Firmware the  KISS-Initialisation lingers a certain

Time, especially by TNCs with low Fequency (Obserev the flashing of the

LEDs. For this reason KISSINIT waits 5s (= 5000 ms) before its ending,

so that with the following Start of TFPCX from a Batch-File the TNC

is also in the KISS-Mode.

-F<File>  Name of the Initialisation File (Standard KISS.INI)

In the case where another File should be used, the Name (also with Path)

can be given with this Option.

-S<Section>  Select Initialisation sequence (Section)

The Initialisation File can select by choice from multiple Sections,

which begin with '[Name]' (see below). With this Option the Section

is selected, which should be used for the Initialisation. As a Parameter

the Name of the Section should be issued (without []),  Upper-/Lower Case

will not be differentiated.

Normally the Initialisation File will be read from the beginning.

-K  KISS-Mode Switch off

If the TNC is required to be reset out of the KISS-Mode, no Initialisation

File will be required. This Option causes the Sending of the KISS-Reset-

Sequence to the TNC. KISS.INI will not be read in this case.

-N  Suppress Reports

This allows the Reports of KISSINIT and the Echo of the sent Characters

to be switched off. But Error Reports are displayed nevertheless.

3. Initialisation File

The File can contain the following special Character Sequences:

\nnn    Issue Control Character with decimal ASCII-Code nnn (0-255)

\e      Issue Escape-Character (\27)

\k      Send KISS-Reset-Sequence (\192\255\192)

\d      1 Second Pause

^c      Issue Control-Character (c = A-Z) (e.g. ^X = Ctrl-X)

Upper and Lower Case will not be differentiated.

If another Character follows on the '\' as indicated above, so an

actual Special Meaning of this Character will be raised and only the

Character issued. If e.g. the '\' should be issued, therefore  '\\'

is to be used ( '\^' with the Character '^'). If a '\' stands at the

end of the Line, the Line End Character (CR = \13) wil not be issued

to the TNC. An ';' at the Line Commencement indicates Commentary. The

Commentary Line will be ignored completely.

Normally the File will be executed from the beginning to the end. If

it is required to save multi-different Initialisation sequences, the

File can be split into Sections, whereby the Option -S (see above)

should be used. A Section begins with the Heading [Name], which must

stand at the beginning of the Line. 'Name' is the Indicator with which

the Option -S is to be issued.

If the Option -S is to be used, the Output begins in the Line, which

follows the Heading of the chosen Section. After  ']'  in the Header

certain Commentary can be included. Dependant on -S  the processing

will be ended always, when the Character '[' appears at the beginning

of a Line, therefore where the next Section begins, or the File end is

reached.

4. Operation Example

A TNC2 with The Firmware (from NORD><LINK) is to be switched into the

KISS-Mode.  The corresponding Command is '@K' and will be preceded with

an Escape-Character. Before the Command Output, any Characters standing

in the Line Buffer of the TNC should be cleared with Control-X. The File

KISS.INI contains the following Line:

^X\e@K

If the TNC is connected on COM1 with 9600 Baud, KISSINIT will be started

e.g. as follows:

KISSINIT -PCOM1 -B9600 -D::5000

Through the Option -D KISSINIT waits for 5 Seconds before the Program

ends,  with it the TNC Initialisation can be performed  kann  (only

required with slow TNCs. The Security Time for Character Output and

new Lines are not issued here (but the ':' must be), here the Standard

Values will be employed. The Options -P and -B certainly could be

omitted in this case, because COM1 and 9600 Baud are Standard. The

Screen Output appears in this example so:

���������������������������������������Ŀ

� KISSINIT v1.00 (Nov 20 1993) by DG0FT �

�     Free for non-commercial usage     �

���������������������������������������Ĵ

� COM1 (3F8),  9600 Bd,  30 300 5000 ms �

�����������������������������������������

Reading ...

\24\27@K

In the lower box the Serial Port will be displayed with Port Address,

the Baudrate and the Security Time (Characters, Line, End). KISSINIT

issues all Characters, which will be sent to the TNC (Echo). Characters

which cannot be displayed, shown as '\nnn' (nnn is the ASCII-Code). In

the Example '\24' satnds for '^X' and '\27' for '\e'. These could be

used also, in the File KISS.INI.

For the switching off of the KISS-Mode and for the following Installation

of the Call Sign in the TNC  e.g. the following File Contents can be used:

\k\d\d\d\d\d\

\eI CALL

First of all a KISS-Reset (\k) occurs, afterwards 5 Seconds (5 times \d)

will be awaited. This Time is required by the TNC before the New Start.

The '\' at the Line End causes the following Linefeed not to be sent to

the TNC. In the following Linee the individual Call Sign will be installed

with the Command 'I'.

Both Initialisation Sequences can follow as in a File combination:

;

; KISS Switch On

;

[KISSON]

^X\e@K

;

; KISS Switch OFF

;

[KISSOFF]

\k\d\d\d\d\d\

\eI CALL

The Calling of KISSINIT then follows with (here without -P und -B):

KISSINIT -SKISSON

respectively

KISSINIT -SKISSOFF

The File contains additional Commentary Lines. To observe is that,

this File can be used no longer without the Option -S, because the

File will be read from the beginning up to the Line '[KISSON]' and

here stands only Commentary. But before '[KISSON]' a 'Standard'-

Sequence inserted, which without -S will be used.

5. Copyright and User Conditions

The Author of the Program KISSINIT is Ren� Stange (DG0FT).  KISSINIT

may be used in Amateur Radio and Copies given to third Parties, so

long as no Charge is made. Particularly the adjunct of KISSINIT to

other Hard- and Software is only permitted, when no charge is made for

the respective Product or by my previous agreement. It is not permitted

to use or circulate the Program Commercially.

A Guarantee for an orderly Function will not be given.

The Author cannot be held responsible for actual Damage, which may occur

from the Usage of KISSINIT  (Responsibility Exclusion).