================================================================

A Z80 Monitor Configured for the TAPR TNC-2 and Clones

================================================================

Paul Newland, P.E.

Post Office Box 205

Holmdel, NJ  07733-0205

------------------------------------------------------------

|       Copyright (c) 1991 by Paul Newland, P.E.           |

|  All Rights Reserved.  This guide may be reproduced      |

|  by individuals for their own personal use, provided     |

|  this copyright notice is included in any reproduction.  |

------------------------------------------------------------

This monitor is configured for use on the TAPR TNC-2 Packet Radio

controller, a Z80 based system.  With minimal effort, it can be

re-configured for other Z80 based systems.  This monitor is like

most other single board, stand alone, monitors.  However, it does

have some unusual features.  They include:

o  Single step through user program executing from RAM

o  Power-On-Vector to run user program in ROM or Battery Protected

RAM

o  DEMO system for outputing text repetitively and/or

interactively

This manual describes how to use the monitor.

Z80 Monitor                              Rev 4.05, September 2, 1991

================================================================

A Z80 Monitor Configured for the TAPR TNC-2 and Clones

================================================================

CONTENTS

1.  Introduction ................................... 1

2.  Overview ....................................... 1

3.  Command Summary ................................ 1

4.  Commands in Detail ............................. 2

4.1   Blast (BI, BO, BT, BF) ..................... 2

4.2   Checksum (C) ............................... 3

4.3   Demo (AE, AP, AI, AV) ...................... 3

4.4   Display (D) ................................ 5

4.5   Examine (X) ................................ 6

4.6   Find (F) ................................... 6

4.7   Go (G) ..................................... 7

4.8   Help (H) ................................... 7

4.9   Hexmath (Y) ................................ 7

4.10  Input (I) .................................. 7

4.11  Jam (J) .................................... 8

4.12  List Stack (L) ............................. 8

4.13  Move (M) ................................... 8

4.14  Next (N) ................................... 9

4.15  Output (O) ................................. 9

4.16  RXintel (R) ................................ 9

4.17  Substitute (S) ............................. 9

4.18  Test (T) .................................. 10

4.19  UpOV (U) .................................. 10

4.20  Verify (V)................................. 10

4.21  ZNext (Z) ................................. 11

5.  Memory and I/O Mapping ........................ 11

6.  System Interrupts ............................. 11

7.  Bugs .......................................... 11

8.  Notes ......................................... 12

- i -

Z80 Monitor                     -1-      Rev 4.05, September 2, 1991

================================================================

A Z80 Monitor for Single Board Computers

================================================================

Paul Newland, P.E.

Post Office Box 205

Holmdel, NJ  07733-0205

------------------------------------------------------------

|       Copyright (c) 1991 by Paul Newland, P.E.           |

|  All Rights Reserved.  This guide may be reproduced      |

|  by individuals for their own personal use, provided     |

|  this copyright notice is included in any reproduction.  |

------------------------------------------------------------

1. Introduction

This monitor is configured for use on the TAPR TNC-2 Packet Radio

controller, a Z80 based system.  With minimal effort, it can be

re-configured for other Z80 based systems.  The monitor is like

most other single board, stand alone, monitors.  However, it does

have some unusual features.  They include:

o  Single step through user program executing from RAM

o  Power-On-Vector to run user program in ROM or Battery

Protected RAM

o  DEMO system for outputing text repetitively

This manual describes how to use the monitor.  However, as with

all instruction manuals, there may be some errors or sections that

do not provide satisfactory explanation.  If you find errors or

insufficient information, please contact me about these problems.

I will attempt to correct them as soon as possible.

2. Overview

The monitor offers many commands to help debug a microprocessor

program.  All commands consist of one or two letter commands.

Commands can be followed by 0 to 16 arguments.  All command line

arguments are in hexadecimal.  With the exception of the GO

command, all arguments are separated from the command and other

arguments by white-space characters.  Arguments shown in angle

brackets <LIKETHIS> are optional.  Specific keyboard presses are

shown in braces.  For example a space would be shown as {sp}, a

return would be shown as {rtn}, an escape would be shown as {esc},

a backspace would be shown as {bs}, a delete would be shown as

{del} and a period would be shown as {.}.  All command input lines

must be ended with a {rtn}.  An example of a command line would

be:

Z80 Monitor                     -2-      Rev 4.05, September 2, 1991

mZ-> m ARG0 ARG1 <ARG2>  {rtn}

3. Command Summary

The table below gives a summary of each command.  The next section

provides details about each command.

-------------------------------------------------------------------

|                  Table 1 -- Command Summary                      |

|                                                                  |

|  Command      Description               Use                      |

|  ----------   ---------------------     -------------------      |

|  BLAST        Blast Data                BI, BO, BF, BT           |

|  CHECKSUM     Calculate Checksum        C start end              |

|  DEMO         Run the Demo Functions    AE, AP, AI, AV           |

|  DISPLAY      Display Memory            D start end              |

|  EXAMINE      Examine Registers         X 'reg                   |

|  FIND         Find Bytes in Memory      F d0 d1 ... d15          |

|  GO           Go with Break Points      Gadr bp1 bp2             |

|  HELP         Print Command Summary     H                        |

|  HEXMATH      Calc Sum and Diff         Y val1 val2; +  -        |

|  INPUT        Show Port Data            I adr                    |

|  JAM          Jam Value to Memory       J start end value        |

|  LIST         List User Stack           L                        |

|  MOVE         Move Block of Data        M start end to           |

|  NEXT         Single Step, All Inst     N                        |

|  OUTPUT       Output Value to Port      O adr value              |

|  RXINTEL      Receive INTEL HEX         R offset                 |

|  SUBSTITUTE   Substitute Data           S adr; SP=next, CR=end   |

|  TESTMEM      Test Memory               T start end              |

|  UPOV         Power On Vector           U adr                    |

|  VERIFY       Verify Memory             V start end to           |

|  ZNEXT        Single Step, skip CALL    Z                        |

-------------------------------------------------------------------

4. Commands in Detail

This section outlines the commands in more detail.

4.1 Blast (BI, BO, BF, BT)

The Blast commands are used to repetitively read and write to I/O

or memory locations.  Blast-In reads from an I/O location while

Blast-Out writes to an I/O location.  Blast-From reads from a

memory location while Blast-To writes to a memory location.

4.1.1 Blast-In (BI)

The Blast-In (BI) command is used to repetitively get data from an

I/O address and display that data to the console.  Command usage

is "BI adr".  Argument "adr" is the address in I/O memory where

the data will be read from.  The data will be displayed on the

console in binary and hex.  To stop this process enter either a

{.} or {esc}.

Z80 Monitor                     -3-      Rev 4.05, September 2, 1991

4.1.2 Blast-Out (BO)

The Blast-Out (BO) command is used to repetitively send data to an

I/O address.  Command usage is "BO adr val".  Argument "adr" is

the address in I/O memory where the "val" data will be written.

To stop this process enter either a {.} or {esc}.

4.1.3 Blast-From (BF)

The Blast-From (BF) command is used to repetitively get data from

a memory address and display that data to the console.  Command

usage is "BF adr".  Argument "adr" is the address in memory where

the data will be read from.  The data will be displayed on the

console in binary and hex.  To stop this process enter either a

{.} or {esc}.

4.1.4 Blast-To (BT)

The Blast-To (BT) command is used to repetitively send data to a

memory address.  Command usage is "BT adr val".  Arguement "adr"

is the address in memory where the "val" data will be written.  To

stop this process enter either a {.} or {esc}.

4.2 Checksum (C)

The Checksum (C) command is used to calculate two types of

checksums across a specified range of memory.  Command usage is "C

start stop".  The monitor responds with checksums SUM and SUM+1 in

hexadecimal.  SUM is simply the 8 bit sum of all data between the

two addresses inclusive.  SUM+1 is equal to SUM plus the number of

bytes added together to determine the value of SUM.

An example would be:

mZ-> c 1000 1fff {rtn}

14DE 24DE

mZ->

4.3 Demo (ae, ap, ai, av)

The Demo commands are used to enter and retrieve text from the

system memory.  They are sometimes used for demonstrations or

displays.  The Demo commands are used use to input and output up

to eight different messages (or screens) to the console.  The

messages can be sent in round-robin fashion with a delay between

each screen or there can be an interactive session where the user

at the console can enter the screen number wanted.  That screen

will then be outputted and, following a delay, the zero screen

will be outputted again.

4.3.1 Demo-Entry (AE)

The Demo-Entry (AE) command is used to enter data into the

system's RAM.  Command usage is "AE adr <opt>".  Text is entered

via the serial point starting at hexadecimal address "adr".  If

the optional argument "opt" is one (1), the text will not be

echoed as it is entered, if "opt" is not included or is zero, then

Z80 Monitor                     -4-      Rev 4.05, September 2, 1991

data will be echoed to the console as it is entered.  Loading

stops when a control D (^D) is transmitted to the monitor.

All LFs and NULLs from the input are ignored.  End-of-lines are

marked by CRs from the console input.  Both the {bs} and {del}

characters are handled the same way.  A {bs} or {del} character

will cause the last character entered to be erased from memory and

will echo {bs} {sp} {bs} to the console.  This will continue for

each {bs} or {del} entered until all characters on the current

line have been erased.  Any characters other than {bs}, {del},

{^D} and {null} can be saved in memory.  Some characters (like the

tilde {~} character) have special meanings depending on the

selected output mode (see AO and AI for more info on this).

4.3.2 Demo-Presentation (AP)

The Demo-Presentation (AP) command is used to output data from the

RAM to the console.  Command usage is "AP adr <opt>".  Argument

"adr" is the address in memory where the text for output begins.

The optional argument "opt" is used to control end-of-line

sequences.  If the B1/B0 bits of "opt" are B1==0/B0==0 then the

end-of-line will be only a CR character.  If the B1/B0 bits of

"opt" are B1==0/B0==1 then the end-of-line will be only a LF

character.  If the B1/B0 bits of "opt" are B1==1/B0==0 then the

end-of-line will be CR and LF.  If the B2 bit of "opt" is B2==0

then any echo from the device receiving the text (console or other

device) will be ignored.  If the B2 bit of "opt" is B2==1 then any

echo from the device receiving the text (console or other device)

will stall output from the monitor until the echo is silent for

about 2 seconds.

Delays can be inserted in-line with the text by including the

string "~Dnnnn".  Where "~" is the tilde character, "D" is the

letter D (either upper or lower case), and "nnnn" is a string of

decimal (0-9) characters.  The string "nnnn" represents a base 10

(not HEX) number for the delay value (this is about the only place

in the monitor where decimal rather than hexadecimal is used).

Delays are in units of (about) 100 mS.  However, delays are

implemented with timing loops and will vary from system to system

depending on clock frequency.  Some scaling of delay values will

be needed for your particular clock frequency.

Special control characters can also be output by the escape

sequence "|x" where "x" is any printable character.  When "|x" is

encountered in the output, bits 8, 7, 6 and 5 of the character are

set to zero before transmission by the UART.  Use "||" to get one

"|" in the output stream.

4.3.3 Demo-Interactive (AI)

The Demo-Interactive (AI) command is used to allow the user at the

console to choose the next screen sent to the console (if no user

control is wanted, use the DEMO PRESENTATION command).

For Demo-Interactive the output the text in memory can be

logically divided into 1 to 8 screens, labled 0 through 7.  Each

screen is separated by a tilde (~) character within the data

Z80 Monitor                     -5-      Rev 4.05, September 2, 1991

loaded into memory.  The 0th screen begins with the first text

entered.  The first tilde (~) entered marks the end of the 0th

screen and the beginning of the 1st screen.  The eighth tilde (~)

marks the end of the 7th screen, which is the last screen.  Any

text entered after the eighth tilde (~) will not be seen using the

AI command.  When entering text with the "ae" command be sure to

include at least 8 tilde (~) characters in the text.  As defensive

programming measure, always enter a line of (~) characters (at

least 10 of them) just before the control D (^D) that marks the

end of the text entry.  That will ensure that you have entered all

8 screens, even if some of them are null.  Null screens are

skipped during output.

Command usage is "AI adr del <opt>".  Argument "adr" is the

address in memory where the text for output begins.  The value

used for "delay" will determine the amount of time delay between

screens.  Note that "delay" in this case, unlike the in-line delay

(~Dnnnn), is a hexadecimal value.  The delay is implemented with

timing loops and the delay will vary from system to system

depending on clock frequency.  The optional argument "opt" is used

to control end-of-line sequences.  See Demo-Presentation for the

function of the optional "opt" argument and in-line escape

sequences.

The user at the console can ask for another screen without waiting

for the delay timer to expire.  If a digit N (from 0 to 7) is

entered from the console, the delay timer will expire and the Nth

screen will be outputed next.  Alternatively, if a space is

entered, the delay timer will expire and the next sequential

screen will be outputed.

4.3.4 Demo-Vector (AV)

The Demo-Vector (AV) command is used with the UpOV command to

create a turn-key demo system.  Via UpOV the user can have the

monitor start the demo output system at power on.  However, UpOV

requires that you provide information about staring address and

options.  Demo-Vector will give you these vectors.  Command usage

is "AV".  The monitor will return with addresses for the argument

list, entry points for the Demo-Presentation and Demo-Interactive

output routines and the start of free RAM and an approximate

length that's safe to use.  Note that all values are in HEX.

4.4 Display (D)

The Display (D) command is used to display memory on the console.

Command usage is "D start <end>".  If only the "start" argument is

used, its value is rounded down to the nearest 16 byte boundary

and the next 16 bytes are displayed on one line.  If both "start"

and "end" arguments are used, the value of "start" is rounded down

to the nearest 16 byte boundary and it becomes the starting

address for the display.  The value "end" is rounded up to the

next 16 byte boundary less one and it becomes the ending address

for the display.  Display can be stopped by pressing the {.} or

{esc} keys on the console.

The output format consists of 16 bytes per line.  Each byte is

Z80 Monitor                     -6-      Rev 4.05, September 2, 1991

printed in hex and the 16 values are followed with the ASCII

representation for each byte.  Values outside the range of 020H

and 07FH are printed as periods (.).

An example would be:

mZ-> d 0023  {rtn}

0020: 00 20 40 41  42 43 44 45  46 47 48 49  50 51 52 53  . @ABCDEFGHIJ

mZ-> d 0023  0038   {rtn}

0020: 00 20 40 41  42 43 44 45  46 47 48 49  50 51 52 53  . @ABCDEFGHIJ

0030: 54 55 56 57  58 59 61 62  63 64 65 66  67 68 69 70  NOPQRSabcdefg

mZ->

4.5 Examine (X)

The Examine command is used to examine and change user registers.

Command usage is "X <reg>".  The Examine command without an

argument will display all the standard registers.

An example would be:

mZ-> x   {rtn}

01  00000001 00 0000 0000 1234 8000 0000 FE00 D03D  00 00 00 00

mZ->  A   SZxHxPNC I  B C  D E  H L   IX   IY   SP   PC  @PC +1 +2 +3

Command usage to change a register would be "X reg".  All

registers accessed by standard naming with the following

exceptions:  Flags are F, Stack Pointer is S, Program Counter is

P, IX is X, IY is Y, and the prime registers are prefaced with an

apostrophe (e.g., 'C).

Some examples would be:

mZ-> x {rtn}

01  00000001 00 8000 0000 1234 8000 0000 FE00 D03D  00 00 00 00

mZ->  A   SZxHxPNC  I B C  D E   HL   IX   IY   SP   PC  @PC +1 +2 +3

mZ-> xa = 01          {rtn}

mZ-> xb = 80 40       {rtn}

mZ-> xp = D03D a000   {rtn}

mZ-> x'c = 33 44      {rtn}

mZ-> x

01  00000001 00 4000 0000 1234 8000 0000 FE00 A000  00 00 00 00

mZ->  A   SZxHxPNC I  B C  D E  H L   IX   IY   SP   PC  @PC +1 +2 +3

4.6 Find (F)

The Find command is used to pattern match data in memory with the

values of the argument list.  The starting address of all memory

that matches will then be printed on the console.  Command usage

is "F d1 ... <d16>".  Up to sixteen arguments may be included.

The monitor then searches for a match between the memory space and

the argument list.  Note that only the least significant 8 bits of

each argument are used for the comparison.  Find can be stopped by

entering a {.} or {esc} from the console.

Z80 Monitor                     -7-      Rev 4.05, September 2, 1991

An example would be:

mZ-> f c3 fd ff  {rtn}

0038:

mZ->

4.7 Go (G)

The Go command is used to begin execution of a user program.

Command usage is "G<start> <bp1> <bp2>".  Note that all arguments

are optional.  If "start" is included this will be the address

that the monitor will begin the user program at.  Note that there

is no white-space character between the command (G) and the

"start" argument.  If "start" is not used, the monitor will jump

to the address shown by the XP command.  "bp1" and "bp2" are

optional breakpoints.  If included, the monitor will replace the

data at these locations with a RST6 instruction and then begin

execution of the user's program.  When the user's program

encounters the RST6 instruction, control will be returned to the

monitor and the monitor will return the original user data to the

break-point address.

The Go command restores all user registers before going to the

user program and it saves all user registers following a

break-point.  Proper operation of the Go command demands that the

user stack value be pointing to RAM.  Failure to ensure this will

cause monitor program to fail.

If the user program doesn't, eventually, get to one of the

break-point addresses, the user may find that the monitor has left

a RST instruction at the break-point address(es).  Following a

hardware reset, the monitor will do its best to restore the

original user data to the break-point address(es), but it isn't

always successful.

4.8 Help (H)

The Help command causes a short list of all the commands to appear

on the screen.  Command usage is "H".

4.9 Hexmath (Y)

The Hexmath is used to calculate two checksums.  Command usage is

"H alpha beta".  The monitor will return with the sum and

difference (alpha - beta) values.

An example would be:

mZ-> h 5678 1234  {rtn}

68AC  4444

mZ->

4.10 Input (I)

The Input command is used to get data from an I/O location and

print it on the console.  Unlike Blast-In, this is a one-time,

non-repetitive operation.  Command usage is "I <adr>".  Argument

Z80 Monitor                     -8-      Rev 4.05, September 2, 1991

"adr" is the address in I/O memory where the data will be read

from.  The data will be displayed on the console in binary and

hex.  If no argument is used, the monitor will use the last

address provided with the previous Input command.

An example would be:

mZ-> i 2f  {rtn}

11001110 CE

mZ-> i     {rtn}

= 11001110 CE

mZ-> i     {rtn}

= 11001110 CE

mZ->

4.11 Jam (J)

The Jam command is used to jam (or fill) an area of memory space

with a constant.  Command usage is "J start end val".  The monitor

with then load the value "val" into memory locations from "start"

to "end", inclusive.

An example would be:

mZ-> j c000 c01f 41  {rtn}

mZ-> d c000          {rtn}

C000: 41 41 41 41  41 41 41 41  41 41 41 41  41 41 41 41  AAAAAAAAAAAAA

mZ->

4.12 List-Stack (L)

The List-Stack command prints out the address of the user stack

pointer and and a few word values on the user stack over a range

of several words above and below the stack pointer address.

Command usage is "L".

An example would be:

mZ-> l

stack pointer

lower             -------------             higher

------------------------------FC03------------------------------

DDE3 138D 93DE 833A 835R 92EB 9831 983E 9399 9300 9318 933D 9DDA

mZ->

Viewing addresses on the stack is sometimes helpful to see where

nested subroutines will return to.

4.13 Move (M)

The Move command is used to move a block of data within memory.

Command usage is "M start end to".  Date from "start" to "end"

inclusive will be moved to memory starting at address "to".

Z80 Monitor                     -9-      Rev 4.05, September 2, 1991

An example would be:

mZ-> m f000 f7fe f001  {rtn}

mZ->

This would fill the memory range F001 to F7FF inclusive with the

value stored at F000.

4.14 Next (N)

The Next command is the most powerful monitor command available to

the user.  It tells the monitor to calculate the next location the

processor will jump to following the instruction pointed to by the

user program counter.  The monitor then uses this address as the

break-point address and operates much the same as the Go command.

When the monitor retains control after the break-point has been

encountered, all registers are displayed along with labels.

Command usage is "N".

As an example, after several Next commands, the screen might look

like this.

00  00000000 00 0000 0000 1234 8000 0000 FE00 D039  00 3E 80 07

00  00000000 00 0000 0000 1234 8000 0000 FE00 D03A  3E 80 07 00

80  00000000 00 0000 0000 1234 8000 0000 FE00 D03C  07 00 00 00

01  00000001 00 0000 0000 1234 8000 0000 FE00 D03D  00 00 00 00

mZ->  A   SZxHxPNC I  B C  D E  H L   IX   IY   SP   PC  @PC +1 +2 +3

4.15 Output (O)

The Output is used to write data to an I/O location.  Command

usage is "O adr val".  Arguement "adr" is the address in I/O

memory where the "val" data will be written.  Unlike Blast-Out,

the Output command causes only one write for every Output command

issued by the user.

For example, to put value 55 at I/O location 7F the command line

would be:

mZ-> o 7F 55  {rtn}

mZ->

4.16 RXintel (R)

The RXintel command is used to load Intel HEX code into memory

space.  Command usage is "R <offset>".  Data is loaded into memory

starting at the address indicated in the record header.  If the

optional argument "offset" is included this will be added to the

record address of every record.  The resulting sum will be used as

the starting address for memory loading.  When a record is

correctly received, a period (.) will be printed on the console.

4.17 Substitute (S)

The Substitute command is used to display and optionally change

bytes stored in memory.  Command usage is "S adr".  The argument

"adr" is the address where substitution will begin.  The address

Z80 Monitor                     -10-     Rev 4.05, September 2, 1991

will be displayed followed by the data at that location.  If no

change is of data is wanted enter a {sp} to look at the next

location or enter a {rtn} to stop the substitute function.  To

change the data enter a new value and type {sp} to continue or

{rtn} to end.

An example would be:

mZ-> s f000    {rtn}

F000: 00       {sp}

F001: 20 30    {sp}

F003: 40       {rtn}

mZ-> d f000    {rtn}

F000:  00 30 40 00  00 00 00 00  00 00 00 00  00 00 00 00  .0@...........

mZ->

4.18 Test (T)

The Test is used to test the system memory.  Command usage is "T

start end".  The monitor will test the memory from "start" to

"end", inclusive.  The memory test is based on a read/write/read

algorithm and should find bad bits as well as double mapping.

TEST can be stopped by entering a {.} or {esc} from the console.

The test will stop after the current test iteration has been

completed.  For large memory blocks it may take several seconds to

complete each test cycle.

An example would be:

mZ-> t d000 f000    {rtn}

passed 0001

passed 0002

passed 0003         {esc} [wait several seconds to complete test 4]

passed 0004

mZ->

Be sure that you don't try to test memory that the monitor needs

for it's functions.  If you do, there's a good chance you will

crash the monitor.

4.19 UpOV (u)

The UpOV command allows the user to tell the monitor to start at

some address other than zero after a reset or power up.  Command

usage is "U adr".  The argument "adr" is the location the user

wants the monitor to go to after a reset or power up.  The

location for start-up is saved in a reserved location in battery

protected RAM (it's complement is also saved as a check).  At

power-on or reset the monitor looks to the console port for a few

seconds to see if a {.} or {esc} has been entered.  If not,

program control is passed to the routine stored at location "adr".

If a {.} or {esc} was entered program control is retained by the

monitor and the user will see the sign-on message and be able to

operate the monitor until the next power-on or reset.  To clear

the vector issue the UpOV command again with argument "adr" equal

to zero (0).  That will clear the vector so that the monitor will

function normally and look only to the console for commands after

Z80 Monitor                     -11-     Rev 4.05, September 2, 1991

a reset or power-up.  The best way to ensure that the monitor will

see the {.} or {esc} is to press the key repetitively during the

power on cycle or reset cycle.

4.20 Verify (V)

The Verify command is used to compare two blocks of memory.

Command usage is "V start end to".  The memory block from "start"

to "end" inclusive is compared with the memory block starting at

"to".  If the blocks contain the exactly the same data no message

is printed.  If any difference is found, the first difference is

printed and the test is stopped.

4.21 ZNext (Z)

The ZNext command works exactly like the Next command except that

when a CALL instruction is found, the monitor will not step

through the subroutine.  It will break at the location the

subroutine should return to.  Command usage is "Z".

5. Memory and I/O Mapping.

In general, the monitor is written so that ROM starts at address

0H and works its way higher in memory while RAM starts at 0FFFFH

and works its way lower.  The monitor uses the top 256 bytes of

RAM (0FF00H to 0FFFFH) for its own needs, including interrupt

vectors.  Console I/O is polled, it is not interrupt driven.

About the only customization is to change the I/O routines to talk

to a different serial port at a different I/O address.  The

monitor is configured to run on a TAPR TNC-2 (or clone) which uses

the SIO B channel to communicate with the console.  The SIO B

control port is assumed to be at I/O location 0DFH and the SIO B

data port is assumed to be at I/O location 0DEH.

6. System Interrupts

Because ROM occupies the first 256 bytes of memory the standard

RSTn and NMI vectors are not directly available to the monitor

user.  In general, this monitor will not function with MODE 0

(8080) interrupts.  MODE 2 (Z80 style) interrupts are no problem

as the Z80 processor can vector to anywhere in memory in this

mode.  MODE 1 (RST7 and NMI) can be used via a double jump system.

The monitor has jump instructions at the RST7 and NMI vector

locations (0038H and 0066H, respectively).  The instruction "JMP

0FFFDH" is stored at location 0038H and the instruction "JMP

0FFFA" is stored at location 0066H.  Note that these instructions

jump to high RAM and that there is sufficient RAM there for

another JMP instruction to any other place in memory.  If the user

wants to run with MODE 1 interrupts he or she should contain code

that will place JMP instructions at RAM locations 0FFFDH and

0FFFAH to vector to the user's interrupt routines.  The loading of

these jump vectors should be included as part of the

initialization routines of the user's program.  The monitor will

never write to these locations on it's own.  Also note that RST6

instruction is used as a trap for the breakpoint feature.  See the

Z80 Monitor                     -12-     Rev 4.05, September 2, 1991

GO, NEXT and ZNEXT commands for more information on this.

The monitor does NOT mess with the interrupt enable flag.  It

doesn't clear it and it doesn't set it.  At power on or reset the

Z80 microprocessor itself disables interrupts.  Other than that,

interrupt control is handled by the user's program.

7. Bugs

If you have the misfortune of encountering a bug in the monitor,

please contact me, in writing, at the address shown on the first

page of this manual.  Please be sure to include a detailed

description of the bug.  I'll do what I can to correct the

problem.

--- END OF DOCUMENT ---