A Collection of

EQUIPMENT MODIFICATIONS FOR 9600 BAUD

From

CompuServe HamNet

Library 9 Digital Operations

File: MODS.ZIP

Downloaded August 17, 1994

PLUS ADDITIONAL MODS

Collected by WÝTN

Rev. 3, 10-17-94

The modifications listed in this document should be considered untested.  Use the information at your own risk.

TABLE OF CONTENTS

(MODZIP File)

Page	Rig	Page	Rig

1	FT790R	19	TR9500

1	TM531	20	FT211

2	C500	20	TM731

3	FT480R (FT280)	21	FT708

3	FT212/FT712	22	FT790R

4	FT726R	23	TM531

5	FT736	24	TM721

7	IC251	25	TNC220

7	IC471	27	IC22A

8	MOCOM 35 UHF	28	IC3220

12	TS700a	30	MFJ 1270/1274

12	TS790	34	FT780/FT480

13	DR1200	34	TM720

14	TM721/731	35	FT767

15	TM721/731	38	FT230

17	IC32	38	TM531

18	IC2400	40	TAPR Modem

18	TR7800

TABLE OF CONTENTS

(WÝTN Additions)

Page	Rig

A-1	MAXON 4020N UHF

A-1	Motorola Mitrek UHF/VHF

(for repeater too)

A-7	FT-708

A-7	GE Master II

A-8	G.E. Exec. II

A-9	Motorola Micor VHF

9600BD @DL       de:G4WFQ  01.10.90 08:48   0   2009 Bytes

FT790R / 9600 bauds.

*** Bulletin-ID: 17239_GB7PET ***

901001/0611z DB0GV , 900929/1554z DB0GE , 900506/1754z DB0IZ

900505/2316z LX0PAC, 900505/0543Z FE1JVI

From: G4WFQ@FE1JVI

To  : AMSAT@EU

R:900505/0311Z 14109 @FE1BJR.#02.FRA.EU [St-Quentin - FBB5.06]

R:900505/0106Z  5566 @F6GJU  Valenciennes -FBB5.06 on atari 1040 pc compatible

R:900505/0032Z 22901 @ON4HU  .#Mous.BEL.EU - SB><LINK - [Mouscron - FBB5.06]

R:900503/2209z 9619@GB7ZAA._3411.GBR.EU [Canterbury, Kent. ] NNA V1.06

R:900503/1419z 11832@GB7SEK._341.GBR.EU [Ashford,Kent] MBL V5.14c

R:900503/1348z 21937@GB7HSN._3212.GBR.EU [Mottingham,London] MBL V5.14c.

R:900503/1328z 8122@GB7SPV._332.GBR.EU [Stevenage, Herts] NNA v1.06

R:900503/1324z 15010@GB7ZPU._211.GBR Sutton, Beds

R:900503/1302z 12729@GB7HXA._222.GBR.EU [Huntingdonshire] NNA V1.06

R:900503/1250z 17239@GB7PET._221.GBR.EU [PETERBOROUGH,CAMBS]

From : G4WFQ @ GB7PET._221.GBR.EU

Audio output from FT790R receiver's Discriminator for 9600bauds

---------------------------------------------------------------

Here is a very simple way of taking audio direct from the discriminator.

Connect a flying lead to pin 9 QO8 (MC3357P). (To save you having to burrow,

just solder a lead on the IC pin itself).

Drill a 1/4 hole in the case between standby and speaker sockets. Insert

phono socket here and connect the flying lead and suitable Gnd.

Thats it, job done.

I am pleased to report that Uosat3 has a huge signal and 9600 bauds is near

perfect rxing with my 790.

Digipeating tests have proved very successful also with just 1watt uplink

power proving useful, and 10watts providing a near 100% link.

73 de Dave G4WFQ @GB7PET (3rd May 1990)

(9600BD) DL2YEN de DB0EAM>

9600B @WW        de:JE1RLA 15.04.91 12:37   3   1955 Bytes

G3RUH modem <-> TM531(to PE1KHX)

*** Bulletin-ID: 38635_JE1RLA ***

910415/1206z DK0MWX, 910415/1204z DB0IZ , 910415/1132z PI8HRL

910414/2017z PI8TCP, 910414/1645Z PI8HWB, 910414/1757  PI8EAE

910414/1534z PA0SCH, 910414/0818  3B8FP , 910413/2157z DU1JMG

910413/2359i JL1ZIJ, 910413/2348i JA2ECD, 910413/2236  JF2PZG

910413/2155I JH0ZDS, 910413/2148  JH0EXA, 910413/2113i JA1WOI

The terminal is connected to slider of VR3(Max. deviation adjast) through

through C55(1uf).

You can discriminate the terminal of VCO from VR3.

TXA ----/\/\/\--II---------- MO VCO

56kohm  0.1uF

If you get weary to remove a shielding plate, you may connect TXA to hotend

of VR3 through R and C as aforesaid.

2. RXA

connect to pin 12 of IC2.

IC2 is a hybrid IC for FM IF amp. and detector.

The IC was not indicated the number of pin.

You must discriminate the pin from granded pin of IC.

2 and 16(last) pin is connected to grand.

I connect 2 TNCs to this tranceiver.

One is 9600bps and oter is Bell 202,1200bps.

Tow modems are running without any troubles.

Sammy  JE1RLA  @je1rla

(9600BD) DL2YEN de DB0EAM>

ALL @EU          de:PA3DVG 19.11.91 06:58  14  27117 Bytes

Mods radio's 9600bd

*** Bulletin-ID: 164_PA3DVG ***

911119/0521z DK0MWX, 911119/0520Z PI8AIR, 911118/0006z PI8UTR

911118/0054  PI8DUY, 911117/2338  PI8JYL, 911105/1741Z PA3DVG

MsgNo: 39715 From: G4WFQ@GB7WNM

Date: 27-Jan/03:17

Subject: C500 Mod Direct Drive

Times Read: 1

From : G4WFQ @ GB7WNM._2231.GBR.EU

Hello,

Klaus (DF5DU) and i have changed the STANDARD C500 for using it with a 9600 bd

modem G3RUH and clones.

For the changging you need only a 2,5 mm connector with swith and a very

calm hand.

The connector is placed on the speaker to the left side of the C500.

There is the most space.

The next step is to place a wire from ic TK10420 pin 11 to the groundpin of the connector. This is one disadvantage to change the C500 in this way : every time is the signal from pin 11 on the connector.

To modulate the C500 you must cut the wire MOD at  J205.

The wire going to the vco is connected to the pin of the connector,that has contact to the top of the plug which will be inserted.

The other side of the connector leads to J205.

The vco is now modulated,when the connector from the modem is plugged in.

You can  use the C500 in normal way.

Klaus and i have tried a connect with the C500 and 9600 bd modem and it worked

with very low signal at once.

Ptt and ground is connected to the normal connector on the top of the handy.

So, much fun in changing the C500.

If there are any questions write to me  or Klaus @ DB0OQ.

73 de Roland

DF3LZ @ DB0OQ

MsgNo: 17403 Type: B Status: $ To: AMSAT  At: GBR    From: GM4JJJ

Date:  08-Jul Time: 02:00

Subject: FT480R 1200 BPSK and 9600 FSK

Times Read: 1

Modifying the FT480R for BPSK and 9600 FSK.

-------------------------------------------

I have modified my FT280 (Japanese home version of FT480R) for direct FM with

G3RUH FO20 modem.

On the FT480R MAIN BOARD, connect a screened lead to FM MOD (XM01) .

On my radio there is a PINK/WHITE wire going to XM01 which is near the centre

front of the MAIN BOARD. This is the point that you want to connect in your

TXAUDIO.

You can connect the screen of the lead to the case of the XM01.

You will require about 2volts p-p at the FM MOD from the FO20 modem for 3.5KHz

deviation.

I modified my G3RUH modem as follows:-

Remove R3 (270K) resistor and fit a wire link (0 ohm).

Remove C9 (10nF) and replace with 1uF foil capacitor.

Remove R5 (1K8) and replace with 22K pot.

Take output from wiper of the pot. (TX Audio).

This has fixed the problem with poor results on FO20 with the Yaesu.

I have also tested it on 9600 FSK terrestrial with good results.

To receive 9600 FSK on the FT280/480R you have to take the O/P direct from

the FM RX discriminator. I picked this signal up from D11 on the MAIN BOARD.

I used a screened lead from D11 to a rewired mic socket:-

On Microphone disconnect orange Lead from pin 5 and solder it to pin 2

leaving pin 5 unused.

On the FT280 mic socket, disconnect link from pin 2 to pin 5 and connect the

screened lead from FM discriminator O/P D11 Anode to pin 5.

73 de David GM4JJJ @ GB7SAN

FT212 & FT712

-------------

I have made the following connections/modifications to the FT212/FT712

transceivers:

For receive, I connected a 10K resistor to pin 11 of Q01 in the IF-unit.

The other end of the 10K resistor is the output.  The resistor is there

to protect the transceiver when you make an error at the external connection...

For transmit, I connected a 10K resistor to the top end of VR03 in the main

unit (near pin 1 of the MIC unit connection).

This point is fed rather directly to the vco varactor.

I have made the external connection with a shielded cable that can run

through a hole near the antenna coax.  There is a small plate on the heatsink

that you can remove and a suitable hole appears.

When you connect the transmit line, it is possible to switchoff the mike

amplifier (at least when you don't have the voice recording unit).

Use 2 diodes, with the kathodes connected together to the external "PTT"

switching lead.  The anode of one diode is connected to the normal PTT point

on the mike connector, the anode of the other diode is connected to pin 10

of Q02 in the MIC unit.  When you now pull the external lead to 0V, the

set will transmit but the microphone does not work.  You can leave it

connected!  Very convenient when you want to use it for fone...

Summary:

IF Q01 pin 11 -----|=====|------ > audio out

10k

MAIN VR03 top -----|=====|------ < audio in

PTT ----------------->|---|

1n4148 |

------ PTT

|

1n4148 |

MIC Q02 pin 10 ------>|---|

(this is a text I sent to someone else before)

I have heard both horror- and success-stories about the TM531 too...

The problem is, that different people refer to the same results as "It works

perfectly, just a few bit errors", or "It's a complete disaster, every

3rd frame I have to send a re-try, killing the throughput".

We are operating a succesful 4800bps link using the TM531 with the HAPN

modem, but that one is a bit less demanding.

73,

Rob  PE1CHL

NO.  DATE   UTC    FROM    TO      SUBJECT

0509 07/09 17:24  IK0HIT  ALL     1280B FT726R PTT Mod P 2 of 2

RE: FT726R Duplex ('Satellite') Op. - PART 2 of 2

IMPLEMENTATION:   Remove top lid.  Remove 'Satellite Unit'.  Remove

lid of 'TX Unit' immediately under 'Sat Unit'. Cut

wire leading to Pin 5 of Connector JO 7 leaving 2 cm on connector side. -

Solder 2 wires of different colour and 30 cm long to each of the cut

ends.  Make a note of colour and connection point. Solder another 30-cm

wire of yet another colour to Pin 1 of Q32 (MC-14069UB). -

Bring the 3 wires neatly out of the box using an existing opening. -

Replace lid of TX Unit taking care not to nick any of the wires. -

Reinstall the 'Sat Unit'. -

Locate small PCB with 4 slide switches near LS. -

Cut all copper lines to the 'BURST' switch. -

Pin 1 of P 46 (the PCB connector) must be soldered to Pin 7, ground.

Complete the circuit shown in Part 1. All the extra components soldered

directly on the small PCB on which the switch is fixed. -

You may also choose to use a separate PCB and switch if you normally use

the 'BURST" function. -

This mod is also very useful for terrestrial high speed file transfer. -

No more clicks, fewer lost packets.

73 de Pesa, IK0HIT

Downloaded from FO-20 de EI6EH

FT736 & 9600 Baud Operation

---------------------------

by James Miller G3RUH

These notes tell you where to get FM RX audio direct from the

discriminator, and where to modulate the FM TX varactor directly.  These

modifications are non-destructive and take no more than a few minutes.

The signals bypass the "DATA SOCKET" for high grade FM operations.

The RX mod is suitable for:

*  UOSAT-D 9600 baud downlink and terrestrial links

*  1200 baud AFSK/FM Standard Packet - BUT IT'S UNSQUELCHED.

The TX mod is suitable for:

*  FO-20/PACSAT uplink (1200 bps Manchester FM)

*  UOSAT-D 9600 baud uplink direct FSK and terrestrial links

*  1200 baud AFSK/FM Standard Packet.

---****---

FT736 -  FM Direct from Discriminator

-------------------------------------

Detected FM direct from the receiver discriminator is available from the

RX UNIT at the junction of R91 and C83.  These components are shown in the

top right-hand corner of the schematic.

Proceed thus:

1. Disconnect FT736 from the mains electricity. (Safety).

2. Remove top cover only.

3. RX Unit is the vertical module on the left.

4. Locate R91 which is about 25mm from the top, 50mm from the radio rear.

the resistor is "on-end", and near a couple of glass diodes.

5. Scrape any paint off R91's free end and wet with solder.

6. Your RXaudio lead should be a fine screened cable;  connect the inner

to R91, and the outer braid to a ground point (e.g. can of TO09)

7. Route the cable out though any convenient aperture in the case.

8. The discriminator sensitivity (FM Normal) as about 6 kHz/volt.

Important note on 9600 Baud Use

-------------------------------

Some FT736 receivers are fitted with an LFH12-S IF filter for FM. (CF01 at

the top front of the RX Unit).  This is a 12 kHz bandwidth filter which is

a little too narrow for 9600 bps FSK operation.  It is recommended you

change this to 15 kHz or better still for UOSAT-D use, 20 kHz bandwidth

which will allow more tolerance for doppler shift, and give a far better

"eye".  Suitable filters are: LFH-15S / CFW455E, and LFH-20S / CFW455D.

---****---

FT736  DIRECT VARACTOR FM MODULATION

------------------------------------

Refer to the circuit diagram; inject your TXaudio at the junction of

R32/C29 on the TX Unit.  The signal level at this point should be 800 mV

peak-peak, and will give +/- 3 kHz deviation.  DO NOT EXCEED THIS LEVEL.

Set Mic Gain to min.

Modulating the FM transmitter this way you get an LF response down to 18

Hz (at which point the associated synthesiser PLL begins to track the

modulation), and an HF response which is flat to some 10 kHz.

Proceed thus:

1. Disconnect FT736 from the mains electricity. (Safety).

2. Remove top cover only.

3. TX Unit is the module flat on the left (not the one tucked down the

side vertically).

4. R32 is just to the left of the rectangular shielded enclosure.  The

resistor is "on end".  Scrape any paint off the free leg.

5. Your TXaudio lead should be a fine screened cable;  connect the inner

to R32, and the outer braid to the adjacent enclosure.

6. Route the cable out though any convenient aperture in the case.

7a. 1200 BAUD PSK MODEM: TXAudio of 800 mV pk-pk can be obtained by

adjusting the components C9= 1uf, R3=47k, R5=infinity (i.e. remove).

C10 stays at 10nf (0.01uf).

7b. 9600 BAUD FSK MODEM:  Adjust TXAudio level with VR1.  Use TXEPROM

selection number 10 (ten) for UO-14, i.e.  JMP1,3 ON, JMP2,4 OFF

Tidy Installation

-----------------

I found it neat to make up a little aluminium bracket to affix under

the rear left lid screw.  This bracket carries a 5-pin DIN socket,

and the TX RX and PTT wires are terminated here.  This saves having

leads dangling out of the interior. (Tnx EI6EH).

Notes compiled by G3RUH @ GB7DDX 1990 Mar 16, rev 1991 Sep 2

From: Andrew G8TZJ

Subj: 9600 Baud Rig Mods.

Hello John, the two rigs that I use for pacsat use are the Icom IC211e and

the IC471e. G3RWL has already told you how he has obtained the 9600 output

from the 471, and I have taken my Rx feed from the same point.

For the 211 I have only modified the rig for transmit. I chose the

slider of the Deviation pot R59 as the input point as it has a low

impedance as recommended for the G3RUH 9600 board. A screened lead can be

soldered to the centre connection of R59 on the component side of the board

if the surrounding components are bent slightly to allow access. The braid

should be connected to ground near to R59. I found that the upper end of

R57 was connected to ground. Check this on your 211 just to make sure that

your 211 has NOT got this resistor in the other way round.

I have adjusted the drive level for a deviation of approximately

2.5 kHz, and this seems OK. The only problem now is the change over time

for the relays, (I have a Mutek front end board fitted which by-passes the

PIN diode switch) but I am working on that.

73 Andrew G8TZJ @ GB7FCI or @ UOSAT3

Hi John de Richard in London. Herewith modification information on my gear.

Rig:

IC-251. Digital modulation applied to top of deviation pot at the junction of

R58, R60 and R61 (just below Q13 on circuit diagram). Modulation

voltage from standard G3RUH 1200 bps modem wasn't high enough but

was boosted by component changes on the modem board. A higher voltage

is required from the G3RUH 9600 bps modem; output level pot set to

about four fifths of maximum and works well.

PTT connection made inside the rig to the appropriate pins on the rear

panel's accessory socket - no complications here, my TNC's PTT comes

from a reed relay.

No modifications done for digital output from receiver (yet).

Have investigated modifying to provide up/down control but checks are

incomplete. Suspect provision of this facility is possible and have

identified (display) DRIVER IC2 pins 3/11 as the likely place to

apply it.

Once AO-21/Rudak starts with higher speeds I'll probably expedite the

provision of receive output and up/down control.

IC-471. Digital output taken from discriminator (IC-14) pin 9. This pin, and

ground, are brought out to J-14 for feeding the optional tone squelch

unit (not fitted on my rig). Rig works well with existing CFW-455E

filter (I think its 15KHz bandwidth) but needs retuning after about

4-5 KHz of doppler; a wider filter may require less frequent tuning

but, with the advent of 12.5 KHz channel spacing on terrestrial uses,

I kept the narrow filter.

Digital modulation would be applied, as with the IC-251, to top of

the deviation pot; thats at the junction of R28 and R29, close to Q1

on the circuit diagram. I haven't actually used this yet (see note

above about RUDAK).

PTT connection made to the appropriate pins on the rear panel's

accessory socket - same story as IC-251.

Connections made to both rigs via phono sockets using screened cable both

inside and outside the rig. The IC-251 has some unused sockets, and the IC-471

has a removable plate which I mounted 3 sockets on.

The IC-471 provides receiver audio on pin 8 (the centre pin) of the mike jack.

I modified the IC-251 to put it on the same (previously unused) pin. This is

used for "ordinary" 1200 bps operations. Pin 4 of the IC-471's mike jack is

the up/down line; that pin is unused on the IC-251 so I'll probably use it

if/when I modify the 251 for up/down.

73 Richard G3RWL

9600 baud UHF Mocom 35 Modification

R:910703/0418z @:W2OY.#WNY.NY.USA.NOAM #:11879 Z:14127 O:#5286 @VE3SNP

R:910702/2253 @:VE3SNP.#NIAG.ON.CAN.NA Welland Ontario #:5286 Z:L3B3A6

Page 1 of 3

GREAT LAKES INTERNATIONAL DIGITAL ASSOCIATION, TECHNICAL BULLETIN ON

MODIFYING THE MOTOROLA MOCOM 35 UHF SERIES FM TRANSCEIVER FOR 9600 BAUD

PACKET

by Ron Kramer VE3MX sysop GLIDA node stack Port Colborne, Ontario

and asistant sysop of VE3SNP BBS Niagara Region.

## NOTE.. Start with a operational tuned up radio on the frequency you are

going to be using it on.

1. Remove DC power cord, Antenna cable, and take the radio out of the case.

2. From the component side of the radio, remove jumper from pin #40 and #41

located with the front of the radio facing you just to the left of the

center partition, about an inch and a half from the back.

NOTE.. keep this jumper.

3. Remove the ends from the jumper in previous step. Obtain a 1.1 uH RFC

and place insulating covers on leads, (note do not cut the leads on the

rfc use full length), solder the jumper ends to the ends of the RFC.

4. Install this RFC on Pins #40 and #3, note #3 is located to the right of

T101 and the left of Variable resistor. on the left side of the radio.

5. Remove capacitor C119 to the right of T102.

6. Remove capacitor C117 to the right of T101.

7. Turn radio over front facing you solder side up.

8. On the right hand side locate T101 and T102 near the rear of the chassis.

9. Cut trace to the right of T101 alignment hole, between the first and second

componet holes on trace going from front to back of set, (this trace has

three component holes) see figure 1 below.

- Add a 4 inch length of hookup wire here on top side of board to xtal

:     ______________

'- > : o           o:       _

:o .-----------'      :o:3

-'         T101    .'.'

_     :o:2 <-- add a 36 pf Mica here to ground

( 0 )   : : <-- cut here

-     :o:1 <-- add a 47 pf Mica here to ground

'-'

Figure # 1

10. Solder a 47 pf Mica capacitor from hole 1 of foil you cut in step 9,

to ground. See figure 1.

11. Solder a 36 pf Mica capacitor from hole 2 of foil you cut in step 9,

to ground. See figure 1.

Page 2 of 3

12. See Figure 2 and cut trace to front right of T102 between hole 2 and 3

of trace.

T102

_

alignmnet hole -> ( o )

-    _ 3

_______:o:

add 36 pf Mica -> .'  o _____.'

here to gnd.    1 :o:'''' ^--Cut trace here

'-'  2

13. Connect power cord, antenna and turn on and retune T101 and T102 before

continuing with modification, once tuned remove power cord and antenna.

14. Locate the Transmit Crystal channel element and remove from board.

Open up the element and remove the fixed capicitor approximately

25 to 32 pf that is in parrallel with the varialble capacitor.

solder a 4 inch lenght of small hookup wire to the junction of the

variable capacitor, crystal and thermisistor, on the foil side of

this pc board. re-install the crystal circuit in the plastic

element case and file a notch on the side of the plastic case for

the jumper wire to exit between the plastic cover and the main

chassis when reinstalled.

15. Reinstall the channel element on the main chassis.

Solder the jumper wire from the crystal element to the rear hole

where C117 was removed next to T101, (see figure 1). This is the

is the junction of CR102 and R112.

16. Locate the mic jack on the left side of the chassis component side

front facing you. There is a 10 pin terminal strip running along the

left side rail, number these terminals from front to back (1 to 10).

17. Disconnect the wire running from the mic connector to terminal strip

pin 5. Solder this wire to blank terminal pin 7 on this strip.

18. Using a length of RG174 or teflon equiv. coax, prepare one end, and

solder the center conductor to pin 7 of terminal strip on left side.

Solder the shield to terminal 9 of same strip.

19. Feed this coax between the face plate and the pcboard near the speaker

from the top to the bottom of the chassis.

20. Turn the radio over with the face toward you and the solder side up.

Locate the metal shield in the shape of a trapizoid, near the front

right side of the pc board (this shield has a 3/8 inch hole in it

for alignment of the Discriminator).

Along the front left angle side of this shield is a jumper wire

connecting the shield to the pcboard. Just to the rear along this

side the very next foil trace is the discriminator output. There

is possibly a printed triangle with the number 15 pointing at this

foil pattern. This is the junction of CR2, R35, C48, R37, R38.

Using the Free end of the RG174 you just installed in step 18,

measrue and cut to length and prepare the end so that it can be

soldered to the above described discriminator trace.

Solder the center conductor to the trace described above and

solder the shield of the cable to the metal shield.

21. Connect the DC power cable and antenna to the radio, align the

transmit crystal to frequency.

Page 3 of 3

22. Pin pinout of terminal strip on top left hand side is:

front pin 1 - PTT

pin 2 - Transmit audio

pin 3 - ground

pin 4 - ground

pin 5 - wire from speaker

pin 6 - unknown

pin 7 - new discriminator output pin to mic connector

pin 8 - jumpered to pin 10

pin 9 - ground

pin 10 - + 12 volt DC input to radio

23. Microphone connector 6 pin Din plug

pin 1 - rx discriminator audio output to tnc

pin 2 - ground

pin 3 - ground

pin 4 - 9600 baud tnc audio input to radio

pin 5 - PTT line

24. That completes the modification. Hook up your 9600 baud PacComm

Tiny 2 Tnc, set the tx audio level in the 9600 baud modem as per

instructions from PacComm, and your away to Happy Packeting.

I hope that this Bulletin is of some help to other's working on System

development, more in this series will follow as modifications are made.

Next will be a writeup on the Mocom 70 series conversion which is very

simple because it is true FM to begin with, Also the Motorola Moxy, and

the Motorola Maxar will be modified, these are also true FM.

73 for now and Happy Packeting from the Niagara Penninsula Ron VE3MX.

Marco - I do not know how many millivolts are required on your radio

to set the deviation to +/- 3 kHz.  But here are two simple ways to

find out.

Method 1.

--------

1. Monitor UO-14 on a scope connected to G3RUH 9600 baud modem TP4.

Measure Pk-Pk voltage.  This corresponds to about +/- 2.5 kHz

2. Set RX to 435.0 MHz, TX to 145.0 MHz.  Power level to min.  Key

up TX.  Reduce pot VR1 on modem until signal (third harmonic) at TP4

is the same size as UOSAT was.  Measure TXAudio voltage pk-pk at

modem output P3-1.

3. Adjust VR1 until this TXAudio voltage is exactly 3x bigger.

You now have the TX deviation the same as UOSAT.

Method 2.

--------

By this method you will calibrate the TS790 RX as a deviation meter.

1. RX = 435.0 MHz, TX=145.0 MHz, Power to min, Modem VR1 to zero.

2. Scope on Discriminator output.  Use 10k/100pf to remove junk if

needed.

3. Key up TX.  Scope output should change from noise to flat DC.

4. Change TX by +/- 1 kHz.  Note change in discriminator voltage.

This gives RX sensitivity in volts per +/-3 kHz.

5. Restore TX to 145.000, increase VR1 until /- peaks of signal are same

as noted in test 4.  Note TXAudio pk-pk voltage at P3-1.

6.  Increase VR1 to give exactly 3x more voltage at P3-1.

7.  You now have TX deviation +/- 3 kHz pk/pk

73 de James G3RUH @ UO14  1991 Sep 19

From      : vk5hi

To        : wd3q

Title     : Reply re TS700

Uploader  : VK5HI

Uploaded  : Fri Mar 01 12:49:52 1991

__________________________

Dear Eric,                                               1st March 1991

Apologies for the delay in answering your message re the TS-700a.

I connected the 9600 baud input directly across R21 (33 kilohm) on

the Generator Unit Board via a 10 microfarad tantalum capacitor.

The + of the tantalum is connected to the junction of C23,R21 and R22.

The - of the tantalum goes to the conductor of the shielded cable, and

the earth shield to the immediately adjacent earth terminal.

I carefully scraped off the 'insulation' on the top of R21 (approx 5mm)

and soldered the + of the tantalum to it.

The 10 microfarads was basically for isolation. I originally intended to

match the load as per James Millers article by modifying C34, however

I just adjusted VR1 and because it works like a charm I have never bothered

to attempt matching.

I have also been meaning to run the JAS-1 via the varactor for Fuji-Oscar 20,

but once again have not got around to it. With the launch of all the Digital

Satellites, time just seems to fly, especially when we have daylight saving,

as the first evening pass goes by between 11pm and 12 midnight. Just lately

I have been going to bed at a respectable hour, hence the dealy in answering.

However this weekend Daylight Saving finishes, so UO_14 and AO_16 will be

here 'an hour earlier', which will be just great.

Trust the information is what you require. The 'old' TS-700a at this QTH

is a very reliable and whats more one of my 'very sentimental' pieces of

equipment. Do not think I could ever part with it. HI!!!!!

73 from "down under".             de Colin VK5HI.

Auto freq. track for UO-14.

^^^^^^^^^^^^^^^^^^^^^^^^^^^

Auto UO-14 RX frequency tracking for TS-790/FT-736

------------------------------------------------------

The doppler shift range in overhead pass reaches to 20kHz. It is essential

to tune-in RX frequency for good through put.

I assembled auto freq. tracking for UO-14 FSK signal reception and achieved

good result.

[Block diagram]

RX discri.out<-- voltage comparater--INV--NOR--switching TR---> Down control

(Q1)           (Q2) (Q2)      (Q3)

|

G3RUH board  DCD -----------------------

[Circuit connection table.]

GND         Q1:LM324 or TL084  Q2:74HC02  Q3:2SC945 etc.

|

(C:0.01)

|    ----      ----      ----              -----

*a<-(R:1M)--------3 Q1 1----6 Q2  4----3 Q2 1---(R:470)---B Q3 C---> DWN *c

+12V--(R:8k)--- 2    |   -5     |   -2    |            -E    |

|    ----    | -----    | ----             | ----

|            |          |                  |

(VR:10K)       GND         |                 GND

|                       |

GND                      |          GND   +12V  +5V

*b<-- G3RUH board DCD line------------      Q1   11     4

Q2    7          14

*a: to RX discri out (=G3RUH board RX IN)

*b: to G3RUH board DCD line (U10 pin 13)

*c: to down control pin of mic terminal (TS-790 mic pin 3)

setting of VR:10k

The discrimiator IC output level is 5.8V+-2V.

Whitout RX signal, set the VR to Q1 pin 1 level turn HIGH->LOW.

In my case, I assembled the circuit on a small universal PCB and enclosed into

TINY-2 with NB96 board.

In the case of FT-736, DL signal polarity is different from TS-790 so

change as follow

------

3 Q2 1------|>------> DL signal to mic terminal

2    |    Diode (1S1588 etc)

-----

73s      sueo asato    JA6FTL

(9600BD) DL2YEN de DB0EAM>

MODS @EU         de:OZ2USA 18.07.92 05:54   0   1837 Bytes

ALINCO DR1200 9600 MODS

*** Bulletin-ID: 59803_OZ6BBS ***

920718/0539z DB0BQ , 920718/0529z DB0SHG, 920717/2313z DB0OBK

920717/2313z DB0CL , 920717/2301z DB0AHA, 920717/2142Z PI8DAZ

920717/1240Z PI8HWB, 920717/1216z PI8EAE, 920717/1129z PA0SCH

920715/2357Z SM5BKI, 920715/2249Z SM0ETV, 920715/0948Z SM7DLZ

920714/0533Z OZ5BOX, 920714/0529Z OZ6BBS

Alinco mods to use mic connector for 9600 baud data

TRANSMITTER

Remove R31

Remove C40 and replace with one end free and other end

reconnected to pin 7 of the VCO board. Connect a jumper between

the now free end of C40 and the hot end of C35. The Mic input is

now connected directly to the VCO.

RECEIVER

Disconnect the pink wire that goes to data output on the

mic connector from the PCB and reconnect it to pin 2 of the IF

board connector. This eliminates the low pass filtering on the

discriminator.

Jon KB6ZBI (Info from Alinco)

========================================================================

Note: I haven't tried or verified this, proceed at your own risk. WA2ISE

(9600BD) DL2YEN de DB0EAM>

9600BD @EU       de:IW4CEA 24.09.92 20:57   0   2818 Bytes

Kenwood TM721/731 9600 baud mods

*** Bulletin-ID: 52966_IW4CEA ***

920924/2030z DB0SIF, 920924/2026z DB0GV , 920924/2004z DB0LJ

920924/1822z DB0GE , 920924/1732z DB0IZ , 920924/1728z ON4UBO

920924/1712Z PI8AIR, 920924/1711z DK0MWX, 920924/1515Z SM5BKI

920924/1348Z PA0SCH, 920923/0904Z IK7GFP, 920922/2029Z IK2DFZ

920922/0834Z IN3TUR, 920920/0144Z IW4BVX, 920920/0020Z IK4OMO

920919/2302Z I4FP  , 920919/2251Z I4UJB , 920919/2249Z IW4CEA

The mods to do 9600 baud packet using a K9NG or G3RUH Modem are fairly simple

to do with the Kenwood TM721/621.

Look at your schematics.

RX DATA: is taken from the discriminator output.

Look for the 3.3k resistor coming off pin9 of the last IF unit stage.

Connect a 1k series resistor from the modem RX input to the junction

of the .001 and 3.3k.

PTT:     is taken off pin2 on the microphone connector.

TX DATA: is fed through  a 1k series resistor to the top of VR3 (deviator ctrl)

This mod has been tested and works quite well on 440 and 220 mhz.

The mod haven't been fully tested on the 2m. portion of the radio.

The TM731 is different enought to cause problems.

What need to be done is to add another RX Mute transistor.

(look the schematics and see how the TX deviation is shorted to ground during

receive.)

Use a plain NPN and a pair of 10k resistor (copy the one used on VR3 in the

schematics.)

Take the TX DATA and feed it to the collector through a series 1k resistor, and

then feed that through another 1k resistor to the wiper on the deviator pot.

This was a result of Kenwood changing the deviation pot value from 10k to 100k

(it's wasn't broken, but they fixed it anyway.)

It is full of surface mount devices.

Article of Jeff WA6FWI @ WB6YMH.#SOCA.CA

From: CQ USA - June/92

73 de Giampaolo IW4CEA @ IW4CEA

(9600BD) DL2YEN de DB0EAM>

9600BD @WW       de:SV1UY  09.11.92 09:55   0   2210 Bytes

IC 3200 and 9600bps

*** Bulletin-ID: 77099_SV1IW ***

921109/0951z DB0ERF, 921109/0936z DB0FP , 921109/0943z DB0GV

921109/0940z DB0LJ , 921109/0919Z PI8HRL, 921109/0858z ON4UBO

921109/0943Z ON1AEO, 921109/0710Z ON6AR , 921109/0645Z ON1BWP

921109/0607Z ON7RC , 921109/0603z LX0PAC, 921109/0611z DB0GE

921109/0659l DB0FRB, 921108/0547z DB0CZ , 921107/1452Z DB0KCP

921107/1453z DB0AAB, 921107/1646Z DB0PV , 921107/1444Z DK0MUC

921107/0847Z 4X1RU , 921106/1713Z SV1IW , 921106/1714z SV1UY

Hello and thanks for reading this message.

Has anyone used IC3200 radio for 9600bps packet. If yes please let me

know if it is any good. Especially if I can use it with a short TXDELAY.

73 Demetre SV1UY

+------------------------------------------------------+

|                 Demetre Ch. Valaris                  |

| AX25 address    : SV1UY@SV1IW.ATH.GRC.EU             |

| Amprnet address : sv1uy@sv1uy.ampr.org (44.154.1.23) |

| Internet address: dvalar@leon.nrcps.ariadne-t.gr     |

+------------------------------------------------------+

(9600BD) DL2YEN de DB0EAM>

KENWOOD @EU      de:KB2LPW 10.12.92 12:29   0   3457 Bytes

9600 Bd MODS for TM721/731 R here

*** Bulletin-ID: 13585_KB2LPW ***

921210/1116z DB0SGL, 921210/1111z DB0LJ , 921210/1110z DB0GE

921210/1144l DB0FRB, 921210/1019z DB0CZ , 921210/0920z DB0KCP

921210/1019z DB0AAB, 921210/1035Z DB0MFG, 921210/0902Z DB0PV

921210/1002Z DK0MUC, 921209/1608Z 4X1RU , 921209/0852Z I5SGG

921209/0643Z 5B4TX , 921208/2211Z OZ5BOX, 921208/2203Z OZ2BUL

921208/1333Z SR1BOX, 921208/1318Z SP1NQN, 921206/1228Z SP4KTO

921205/1954Z SP4DGN, 921202/2214Z SP5OXO, 921202/0921Z SP5PBE

The mods to do 9600 baud packet  using a K9NG or G3RUH modem are  fairly simple

to do with the Kenwood TM-721/621 radios.  Look at your schematic.

Receive  data  is  taken  from  the dicriminator output. Look for the 3.3K

resistor comming off pin 9 of the  last IF stage. Connect a 1K series  resistor

from the modem RX input to the junction of the .001 and 3.3K.

PTT  is taken  off pin  2 on  the microphone  connector. TX  data  is  fed

through a 1K series resistor to the top of VR3 (the deviation conltrol).

This method has been tested and works  quite well on 440 MHz and 220  MHz.

The mods haven't fully been tested on the 2 meter portion of the radio.

The TM-731 radio is different enough  to cause problems. What needs to  be

done is to add another RX Mute  transistor. (Look at the schematic and see  how

the TX deviation  is shorted to  ground during receive.)  Use a plain NPN and a

pair of 10K resistors (copy the one  used on VR3 in the schematic). Take the TX

data and feed it to the collector  through a series 1K resistor, and then  feed

that through another 1K resistor to the  wiper of the deviation pot. This was a

result  of Kenwood  changing the  deviation pot  value from  10K to  100K.  (It

wasn't broken, but they fixed it anyway.)

As  with  any  modifications,  take   reasonable  care,  and  I  won't  be

responsible if you  trash your radio.  BE CAREFUL; it  is full of surface mount

devices.

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

This MOD file was written by Jeff, WA6FWI @ WB6YMH.#SOCA.CA.USA.NA

Schematics and layouts are available by request through the author.

Good Luck!

73 de Chris KB2LPW @ KB2LPW.#NYC.NY.USA.NA

SYSOP of PDSNET ARC and the Helping BBS's

==============================================================================

(9600BD) DL2YEN de DB0EAM>

9600BD @WW       de:IT9EFW 05.05.93 13:16   0   1962 Bytes

TM741E KENWOOD AT 9600 B

*** Bulletin-ID: 9646_IT9ZWS ***

930505/1259z DB0GV , 930505/1256z DK0MTV, 930505/1249z DB0RBS

930505/1248z DB0AAA, 930505/1233z DB0MWE, 930505/1345z DB0LAN

930505/1226Z DK0MUC, 930505/0625Z 4X1RU , 930505/0601Z I5SGG

930504/2013Z IW2GKO, 930504/1916Z IK2TTU, 930504/1839Z IW2FPO

930504/1823z IW2DNI, 930504/1433Z IK2DFZ, 930504/0735Z IK7GFP

930504/0726Z I7XQX , 930504/0558Z IK7BFM, 930504/0022Z I8WYA

930503/1451Z IW8PHV, 930503/1437Z IT9LCY, 930503/1417Z IT9LCP

930503/1054Z IT9XIX, 930503/0813Z IT9ZWS

From: IT9EFW@IT9ZWS.PATH

To  : ALL@WW

HELLO,  I  AM LOOKING FOR A MODIFY FOR TO WORK AT 9600 BAUD WITH MY KENWOOD

TM741E. PLEASE CAN ANYBODY HELP ME?

73 DE IT9EFW  @IT9ZWS-8.ITA.EU

(9600BD) DL2YEN de DB0EAM>

ICOM @ALL        de:PA3CII 28.05.93 05:07   0   1111 Bytes

9k6 mods. for IC449a

*** Bulletin-ID: 275306ON4UBO ***

930528/0453z DB0GV , 930528/0443z DB0LJ , 930528/0437z DB0GE

930528/0333z LX0PAC, 930528/0331Z ON7RC , 930528/0328Z ON1BWP

930528/0049Z ON6AR , 930528/0152Z ON1AEO, 930527/2104z ON4UBO

de PA3CII @ ON4UBO.LG.BEL.EU

to ICOM @ ALL

Hello, and tnx for reading this message.

I'am looking for the modifications to run the IC449 with 9600 Bd in

Packet Radio.

Any help is welcome... Thanks in advance.

PA3CII ampr.org. 44.137.24.54

or via ax25 pa3cii@on4ubo

73's

(9600BD) DL2YEN de DB0EAM>

ALL @WW          de:IK2QED 03.06.93 08:09  14   2505 Bytes

IC32 9k6 modification

*** Bulletin-ID: 5659_IW2FPO ***

930603/0749z DB0SIF, 930603/0745z DB0GV , 930603/0733z DK0MTV

930603/0726z DB0GE , 930603/0723z HB9EAS, 930603/0710z OE9XPI

930603/0709z DB0KCP, 930603/0704z DB0MWE, 930603/0619Z DK0MUC

930603/0345Z 4X1RU , 930602/2315Z I5SGG , 930602/2306Z IK4MGV

930602/2301Z IK4NZD, 930602/2250z IW2DNI, 930602/2246Z IW2FPO

My IC32(AT) now work very well with 9600 baud G3RUH modem. How ??

INTERFACING ICOM IC-32(A/AT/E) AND 9600 BAUD MODEM

--------------------------------------------------

IC706 and IC707 are locate on the bottom of then MAIN UNIT (first card you

see after removing of the back alluminium-cover of the IC32).

Componenst must be located reading the reference on its package.

C798 (.0022)      |       |

____||______________|6     3|__

R765(82k) |   ||              |       |    IC706 (M5218FP)

____/\/\/\__|____/\/\/\_________|5     4|__

R766 (82K)  |   |_______|

/|\            __|__

|             _____ C800 (120)

|               |

|               |

|              GND

In 9k6 (out from modem) to the VCO varactor.

Discriminator output is taken directly from pin 11 of IC707 (TK10420M)

and put into 9K6 modem input.

PTT is connected directly to the ptt key (J710).

Enjoy with 9600 baud packet.

+---------------------------------------------------------------+

|  best 73's de IK2QED, Stefano.     (IK2QED @ IW2FPO)          |

+---------------------------------------------------------------+

(9600BD) DL2YEN de DB0EAM>

9600BD @EU       de:PE1DNA 12.06.93 10:46   0   1658 Bytes

CP PE1DNA: msg DL3MEW : ICOM 2400 & 9600bd

*** Bulletin-ID: 36989_PI8ZAA ***

930612/0959z DK0MWX, 930612/0952z DB0IZ , 930612/0957z ON4UBO

930612/0931Z ON6AR , 930612/0921Z PI8HWB, 930612/1517Z PI8ZAA

Original from PE1DNA to ICOM@ALL

Hi ICOM 2400/2500 users,

In my humble opinion, the ICOM 2400 is totally unusable for packet radio, let

alone 9600 bd operation !. The delay between keying the PTT and actually

producing RF power is so long that many collisions will be produced.

This delay is 300-500 ms. In this 'dead time' any other station can decide to

key the transmitter, and thereby producing collisions.

I asked in a ICOM @ EU bulletin a few months ago if anybody knew a solution

for this problem, but all i got where "if you got the solution, please telle me

about it" responses.

Why is there so little concern about the switching times from transceivers ?

Everybody tries te find the right audio tap points for tx and rx, but the

switching time (PTT to RFout, and *PTT to rxready) gets no attention.

Especially at 9600 bd this very important.

Anyway, if someone knows how to speed up the ICOM 2400 PTT to RF time,

i am very interested.. Even with phone operation it's a pain to operate.

73 Joop, pe1dna@pi8zaa

(9600BD) DL2YEN de DB0EAM>

9600BD @WW       de:LU7AKC 13.08.93 07:52   0   3161 Bytes

TR7800 & 9600 Bps

*** Bulletin-ID: 12266_LU7AKC ***

930813/0750z DB0SIF, 930813/0750z DB0AIS, 930813/0742z DK0MTV

930813/0746z DB0HOM, 930813/0743z F6GIA , 930813/0741Z LX0PRG

930813/0637Z F5LBL , 930813/0715Z F6KIF , 930813/0714Z FF6PTT

930813/0633Z F5MSQ , 930813/0445Z F8REF , 930813/0338Z F5GHV

930812/2053Z F1HAQ , 930812/2038Z F1MAC , 930812/2358Z F1OYA

930812/1530Z F1ONT , 930812/1336Z F6CNN , 930812/1322Z F6FBB

930812/1334Z F5JGK , 930812/1258Z F1PKI , 930812/0231Z EA2SG

930811/2144Z EA2KJ , 930811/2124Z EA2AAA, 930811/2113Z EB3ETC

930811/1932Z EA3DTF, 930811/1841Z EA3FHW, 930811/1731Z EB3ERZ

930811/1057Z EA3AHM, 930809/0252Z LU6DYD, 930809/0235Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

TR-7800 & 9600 Bps.

Hello everyone,

Finally I decided to modify my old Kenwood TR7800 vhf (2m) radio to work with G3

RUH modem. It is all right and so here you are the details.

About the reception (RXA) I made the connection exactly on a lead of the R69 ( 2

2K) after the discriminator (D8-D7) located on the mother board.

About the trasmission (TXA) I made the connection on T1 test point;that is on th

e central lead of the trimmer VR3 also on the mother board.

The PTT signal was made on the rear of the Micro Jack on pin4. Naturally the wir

es are shielded and I made them passed through an hole that I made on the rear p

anel near the power supply jack.

If you like having the possibilty to switch through data mode- voice mode, you c

an utilize the Alert switch.

That is all.

Bye and have good time with 9600 bd packet radio.

Lino IK3NGU@I3XTY in Treviso Italy

9600BD @WW       de:LU7AKC 13.08.93 08:45   0   2646 Bytes

TR9500 & 9600 Bps

*** Bulletin-ID: 12268_LU7AKC ***

930813/0843z DB0GV , 930813/0842z DB0AIS, 930813/0832z DK0MTV

930813/0836z DB0HOM, 930813/0832z F6GIA , 930813/0830Z LX0PRG

930813/0639Z F5LBL , 930813/0718Z F6KIF , 930813/0716Z FF6PTT

930813/0636Z F5MSQ , 930813/0535Z F8REF , 930813/0457Z F5GHV

930812/2102Z F1HAQ , 930812/2040Z F1MAC , 930813/0006Z F1OYA

930812/1540Z F1ONT , 930812/1344Z F6CNN , 930812/1339Z F6FBB

930812/1338Z F5JGK , 930812/1303Z F1PKI , 930812/0237Z EA2SG

930811/2145Z EA2KJ , 930811/2125Z EA2AAA, 930811/2115Z EB3ETC

930811/1936Z EA3DTF, 930811/1844Z EA3FHW, 930811/1732Z EB3ERZ

930811/1057Z EA3AHM, 930809/0253Z LU6DYD, 930809/0235Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

TR-9500 & 9600 Bps.

Interconexiøn del modem de 9600 Baud de G3RUH al Kenwood TR9500.

El audio del receptor se toma de la uniøn de R42/R43 en IF-Unit.

El audio hacia el transmisor se conecta en la uniøn de C18/C19 en CAR-Unit.

El PTT al punto habitual de donde lo toma el micrøfono.

27-01-93

73 de Rafael, EA4RJ

9600BD @WW       de:LU7AKC 13.08.93 09:52   0   3441 Bytes

FT211 & 9600 Bps

*** Bulletin-ID: 12248_LU7AKC ***

930813/0949z DB0SIF, 930813/0950z DB0AIS, 930813/0842z DB0HOM

930813/0836z F6GIA , 930813/0831Z LX0PRG, 930813/0721Z F5LBL

930813/0742Z F6KIF , 930813/0732Z FF6PTT, 930813/0640Z F5MSQ

930813/0539Z F8REF , 930813/0501Z F5GHV , 930812/2105Z F1HAQ

930812/2043Z F1MAC , 930813/0014Z F1OYA , 930812/1554Z F1ONT

930812/1348Z F6CNN , 930812/1346Z F6FBB , 930812/1341Z F5JGK

930812/1304Z F1PKI , 930812/0239Z EA2SG , 930811/2145Z EA2KJ

930811/2126Z EA2AAA, 930811/2116Z EB3ETC, 930811/1942Z EA3DTF

930811/1847Z EA3FHW, 930811/1723Z EB3ERZ, 930811/1057Z EA3AHM

930809/0243Z LU6DYD, 930809/0234Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

9600 BPS with Yaesu FT-211RH

The FT-211RH has been shown to be suitable for 9600 baud packet operation.

There is no major surgery required just two screened leads are required, one

for TX AF the other for RX AF connected as follows:

RX AUDIO

On the RX IF Unit (sub board F2869104) connect the screened lead inner to the

TK10420 IC pin 9 with the outerscreen to pin 15.  Caution should be taken to

solder these if soldered to the underside of the board.

TX AUDIO

On the rear of the main PCB a small potentiometer will be seen (peak deviation

adjuster).  Unsolder the leg nearest to the rear of the set, and connect the

inner of the screened TX leads to this point, the outer to the earth plane

adjacent to this point.  To ensure that the 9600 baud modem is terminated in

the required impedance, it is essential that a suitable terminating resistor be

placed across the screened lead inner/outer, eg a 560 ohm resister; this may

usefully be done at the FT-211RH potentiometer connection.

No further adjustments need to be made to the radio.  On the tested modem a

suitable level of 2.5 kHz deviation was achieved with one third rotation

clockwise of the modem TX AF level potentiometer.

9600BD @WW       de:LU7AKC 13.08.93 09:59   0   2958 Bytes

TM731 & 9600 Bps

*** Bulletin-ID: 12262_LU7AKC ***

930813/0957z DB0GV , 930813/0956z DB0AIS, 930813/0949z DB0HOM

930813/0946z F6GIA , 930813/0934Z LX0PRG, 930813/0821Z F5LBL

930813/0849Z F6KIF , 930813/0749Z FF6PTT, 930813/0643Z F5MSQ

930813/0635Z F8REF , 930813/0550Z F5GHV , 930812/2106Z F1HAQ

930812/2041Z F1MAC , 930813/0009Z F1OYA , 930812/1547Z F1ONT

930812/1346Z F6CNN , 930812/1337Z F6FBB , 930812/1338Z F5JGK

930812/1303Z F1PKI , 930812/0235Z EA2SG , 930811/2145Z EA2KJ

930811/2125Z EA2AAA, 930811/2114Z EB3ETC, 930811/1935Z EA3DTF

930811/1844Z EA3FHW, 930811/1729Z EB3ERZ, 930811/1057Z EA3AHM

930809/0250Z LU6DYD, 930809/0235Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

TM-731 & 9600 Bps.

The TM731 is different enought to cause problems.

What need to be done is to add another RX Mute transistor. (look the schematics

and see how the TX deviation is shorted to ground during receive.)

Use a plain NPN and a pair of 10k resistor (copy the one used on VR3 in the

schematics.)

Take the TX DATA and feed it to the collector through a series 1k resistor, and

then feed that through another 1k resistor to the wiper on the deviator pot.

This was a result of Kenwood changing the deviation pot value from 10k to 100k

(it's wasn't broken, but they fixed it anyway.) It is full of surface mount

devices.

Article of Jeff WA6FWI @ WB6YMH.#SOCA.CA

From: CQ USA - June/92

9600BD @WW       de:LU7AKC 13.08.93 10:42   0   3365 Bytes

FT708 & 9600 Bps

*** Bulletin-ID: 12250_LU7AKC ***

930813/1040z DB0SIF, 930813/1039z DB0AIS, 930813/1034z DB0HOM

930813/1026z F6GIA , 930813/1007Z LX0PRG, 930813/0858Z F5LBL

930813/0917Z F6KIF , 930813/0838Z FF6PTT, 930813/0720Z F5MSQ

930813/0658Z F8REF , 930813/0630Z F5GHV , 930812/2109Z F1HAQ

930812/2039Z F1MAC , 930813/0000Z F1OYA , 930812/1536Z F1ONT

930812/1338Z F6CNN , 930812/1325Z F6FBB , 930812/1334Z F5JGK

930812/1258Z F1PKI , 930812/0232Z EA2SG , 930811/2144Z EA2KJ

930811/2124Z EA2AAA, 930811/2114Z EB3ETC, 930811/1934Z EA3DTF

930811/1844Z EA3FHW, 930811/1725Z EB3ERZ, 930811/1057Z EA3AHM

930809/0243Z LU6DYD, 930809/0234Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

9600 Mod for FT-708-R

Hello Ham's,here is the modification for use the FT-708-R with a G3RHU modem

for 9600 bd packet operation.

Audio output from FT 708 R receiver's Discriminator for 9600 bauds

------------------------------------------------------------------

by Andrea Borgnino IW1CXZ

-------------------------

Here is a very simple way of taking audio direct from the discriminator.

Connect a flying lead to pin 9 Q 107 (MC3357P) that is located at the opposite

side of the CPU.

You can also take the signal from the GREEN cable that goes to the TONE SQUELCH

optional unit.

TxAudio input into FT 708 R Varactor diode for 9600 bauds

---------------------------------------------------------

You must find the D 209 Varactor diode (FC 53) locate near the output DC

connection,or near the VR 202 trimmer in the PLL Unit.Then you must insert the

TXA signal in the catode of D 209 and the 9600 modification is made.

My FT-708 can run 9600 operation very well with a TX DELAY value of 17.

Best 73 from IW1CXZ Andrea @I1LYM.#TO.ITA.EURO

9600BD @WW       de:LU7AKC 13.08.93 10:42   0   3537 Bytes

FT790 & 9600 Bps

*** Bulletin-ID: 12253_LU7AKC ***

930813/1040z DB0SIF, 930813/1039z DB0AIS, 930813/1035z DB0HOM

930813/1035z DB0GE , 930813/1008z LX0PAC, 930813/1011Z LX0PRG

930813/0906Z F5LBL , 930813/0944Z F6KIF , 930813/0937Z FF6PTT

930813/0938Z F5MSQ , 930813/0826Z F8REF , 930813/0724Z F5GHV

930812/2115Z F1HAQ , 930812/2046Z F1MAC , 930813/0024Z F1OYA

930812/1600Z F1ONT , 930812/1353Z F6CNN , 930812/1349Z F6FBB

930812/1343Z F5JGK , 930812/1306Z F1PKI , 930812/0242Z EA2SG

930811/2146Z EA2KJ , 930811/2127Z EA2AAA, 930811/2117Z EB3ETC

930811/1943Z EA3DTF, 930811/1850Z EA3FHW, 930811/1726Z EB3ERZ

930811/1057Z EA3AHM, 930809/0244Z LU6DYD, 930809/0234Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

9600 bauds mods for FT790r

From : G4WFQ @ GB7PET.21.GBR.EU

Audio output from FT790R receiver's Discriminator for 9600bauds

---------------------------------------------------------------

Here is a very simple way of taking audio direct from the discriminator.

Connect a flying lead to pin 9 QO8 (MC3357P). (To save you having to burrow,

just solder a lead on the IC pin itself). Drill a 1/4 hole in the case between

standby and speaker sockets. Insert phono socket here and connect the flying

lead and suitable Gnd. Thats it, job done.

TxAudio input into FT790R Varactor diode for 9600 bauds

-------------------------------------------------------

Here are the details to input 9600 buads into the 790. Firstly make up the

following circuit.

470n

Txaudio----->---] ]------------------22k----->To FC53M

:           :       :

:          22k     ---

:           :      --- 10p

:           :        :

TxAgnd--->----------------------------------->Chassis Gnd

Notes:-

When you make up the above circuit, you must use very short leads!!!!

FC53M is a Varactor diode which is D1031 on the FT790r diagram.

73 de Dave G4WFQ @GB7PET 30th May 1990

9600BD @WW       de:LU7AKC 13.08.93 12:05   0   3999 Bytes

TM531 & 9600 Bps

*** Bulletin-ID: 12260_LU7AKC ***

930813/1202z DB0SIF, 930813/1203z DB0AIS, 930813/1146z DB0HOM

930813/1135z DB0GE , 930813/1120z LX0PAC, 930813/1111z ON7RC

930813/1152Z ON1KGX, 930813/1046Z ON1KEZ, 930813/1025Z F6KIF

930813/1014Z FF6PTT, 930813/1006Z F5MSQ , 930813/0848Z F8REF

930813/0830Z F5GHV , 930812/2302Z F1HAQ , 930812/2131Z F1MAC

930813/0126Z F1OYA , 930812/1627Z F1ONT , 930812/1411Z F6CNN

930812/1358Z F6FBB , 930812/1349Z F5JGK , 930812/1313Z F1PKI

930812/0346Z EA2SG , 930811/2147Z EA2KJ , 930811/2130Z EA2AAA

930811/2121Z EB3ETC, 930811/1946Z EA3DTF, 930811/1859Z EA3FHW

930811/1739Z EB3ERZ, 930811/1057Z EA3AHM, 930809/0250Z LU6DYD

930809/0235Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

TM531<>G3RUH 9600bd

Hello, Ed.

I received your mail today morning,on Dec., 15.

I'm using TM-531 on G3RUH type 9600bps modem.

We ha local aria network using 9600bps modem.

It is working well,now.  But, beginning of this network is not easy.

You can connect G3RUH type modem as follow.

1. TXA

Take off the shielding plate under the main PCB.

Search for VR3 on main PCB.

VR3 is indicated on PCB "MAX DEV" for adjust maximum deviation.

The slider of this VR is connected to MD pin of VCO through C55

capacitor.

Connect TXA to MD pin of VCO using resister(56k to 100kohm) and

capacitor 0.1uF,connected in series,like follow.

TXA ---II------/\/\/\----MD(VCO)

0.1uF    56k to 100kohm

If you get weary to take off shielding plate,you may connect the TXA

to hot end of VR3, using the resister and capacitor as aforesaid.

TM-531 & 9600 Bps.

2. RXA

Search for IC2 on main PCB.

IC2 is a 16 pin hybrid IC for IF AMP and discriminator.

You can discriminate pin number of this IC from printed symbol on

the main PCB, and grand pin.

Pin 2 and 16 is connected to grand.

pin 5 is output for ceramic filter(CF1),and pin 7 is input from the

filter.

Connect RXA to pin 12(DET) directly.

9600bps on 1200MHz band is very difficult.

Because,frequency stability is not enough.

And, IF band width is not enough,too.

We are developing SMC transceiver for G3RUH modem on 144,430,1200MHz

band.

It has high stability PLL OSC(about 1ppm) and good linearity

modulator (voltage controlled X'tal OSC)

Sammy JE1RLA @ JE1RLA

9600BD @WW       de:LU7AKC 13.08.93 12:05   0   4366 Bytes

TM721 & 9600 Bps

*** Bulletin-ID: 12261_LU7AKC ***

930813/1203z DB0SIF, 930813/1204z DB0AIS, 930813/1146z DB0HOM

930813/1136z DB0GE , 930813/1120z LX0PAC, 930813/1111z ON7RC

930813/1153Z ON1KGX, 930813/1047Z ON1KEZ, 930813/1027Z F6KIF

930813/1017Z FF6PTT, 930813/1012Z F5MSQ , 930813/0903Z F8REF

930813/0840Z F5GHV , 930812/2303Z F1HAQ , 930812/2231Z F1MAC

930813/0144Z F1OYA , 930812/1640Z F1ONT , 930812/1418Z F6CNN

930812/1406Z F6FBB , 930812/1351Z F5JGK , 930812/1315Z F1PKI

930812/0354Z EA2SG , 930811/2148Z EA2KJ , 930811/2130Z EA2AAA

930811/2125Z EB3ETC, 930811/1947Z EA3DTF, 930811/1900Z EA3FHW

930811/1742Z EB3ERZ, 930811/1057Z EA3AHM, 930809/0250Z LU6DYD

930809/0235Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

TM-721 & 9600 Bps.

The mods to do 9600 baud packet using a k9ng or g3ruh modem are fairly

simple to do with the Kenwood TM-721/621 radios. Look at your schematic.

Receive data is taken from the discriminator output. Look for the 3.3K

resistor coming off of pin 9 of the last IF stage. Connect a 1K series

resistor from the Modem RX input to the junction of the .001 and 3.3K.

PTT is taken off pin 2 on the microphone connector. TX data is fed through

a 1K series resistor to the top of VR3 (The deviation control). This has

been tested and works quite well on 440 MHz and 220 MHz. The mods haven't

been fully tested on the 2 meter portion of the radio.

The TM-731 radio is just enough different to cause problems. What needs to

be done is the addition of another RX Mute transistor. (Look at the schematic

and see how the TX deviation is shorted to ground during receive). Use a

plain npn and a pair of 10K resistors copy the one used on VR3 on the

schecmatic. Take the TX data and feed it to the collector through a series

1K resistor, and then feed that through another 1K resistor to the wiper

of the deviation pot. This was a result of Kenwood changing the deviation

pot value from 10K to 100K. (It wasn't broke, but they fixed it anyways.)

This message is being uploaded to the local BBS with the modified TM-731.

As with any modifications, take resonable care, and I won't be responsible

if you trash your radio. BE CAREFUL it is full of surface mount devices.

TM-721 & 9600 Bps.

The interface to my 721 was guessed at and seems to work quite well.  Audio

directly from pin 9 of FM IF chip, and PTT on pin 2 of mic connector.  TX input

to the collector of muting transistor of PLL audio input. The local 9600 node

seems to be quite busy and that may be the cause of seemingly poor operation.

A local friend of mine is about to get another 9600 card for his 1278 TNC so we

will have a clear frequency to do testing between our rigs.

73 & God bless from Chris/AB6FA  Lompoc CA.

9600BD @WW       de:LU7AKC 13.08.93 12:07   0   5387 Bytes

TNC220 & 9600 Bps

*** Bulletin-ID: 12265_LU7AKC ***

930813/1204z DB0SIF, 930813/1205z DB0AIS, 930813/1147z DB0HOM

930813/1138z DB0GE , 930813/1122z LX0PAC, 930813/1113z ON7RC

930813/1205Z ON1KGX, 930813/1102Z ON1KEZ, 930813/1038Z F6KIF

930813/1022Z FF6PTT, 930813/1016Z F5MSQ , 930813/0911Z F8REF

930813/0849Z F5GHV , 930812/2305Z F1HAQ , 930812/1711Z F1ONT

930812/1432Z F6CNN , 930812/1426Z F6FBB , 930812/1401Z F5JGK

930812/1324Z F1PKI , 930812/0425Z EA2SG , 930812/0018Z EA2RCP

930811/2306Z EA2KJ , 930811/2241Z EA2AAA, 930811/2240Z EB3ETC

930811/1949Z EA3DTF, 930811/1903Z EA3FHW, 930811/1746Z EB3ERZ

930811/1057Z EA3AHM, 930809/0252Z LU6DYD, 930809/0235Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

TNC220

From: VE3PAZ@VE3JF.ON.CAN.NA

Subject: pac-comm problem with UO-14

Following message has been reformated from a binary file dump collected from

FO-20 by the automatic AMSAT station operated by ve3paz in Ottawa. Hope this

helps anyone over this way who is having the same problem

Hello All @ AMSAT

The following modification to the 220 will allow your Tnc to operate at 9600

bauds in Fullduplex. For those of you who thought you already could I suggest

you try it.

It must be mentioned at this stage that the version of my 220 is 1.3. If yours

is a different version then the story may be different, but I doubt it. The

reason for this mod is to allow my station to operate on UOSAT3 FULLDUPLEX.

---------------------------------------------------------

Previous to this mod I have been ripping my hair out as to why I could not

digipeat via UOSAT3. When one day I discovered that when I changed the baud

speed to 9600 and then put Fullduplex on the txbaud rate changed back to 1200

whilst the rx rate remained at 9600. Most perculiar I thought.

So, I got in communications with the main Agent in the UK for Pac-comm to see

if anyone else had this same problem. No came the reply, obviously no one else

has tried it yet I thought. So here I am stuck with a TNC that is no use to me

for UO3. Can you help me then please I asked Agent. Yes we'll see what we can

do. In the meantime Rep has been to Paccomm across the pond and I am told that

they work alright over there. Cant be I said! So anyway he suggests I try

changing the thirmware which I did but still no good. Ok we'll send them

another Fax they said. Fine! Another phone call to Agent 2 weeks later, no info

or reply.So I put pen to paper and addressed the same question to the Gentleman

concerned at Pac-comm.  To date Ive no reply, letter sent some 4 months ago.

Not impressed!

With UO3 software now on release, and uploading and testing of UO3 bulletin

software imminent I figured I had better do something. The result is a crued

[sic] cure.

===============================================================================

9600 FULLDUPLEX on the Pac-Comm TNC-220

by D. Hulatt G4WFQ 19th Aug 90

------------------------------

Locate 74HC4020 and remove from IC socket

CAREFULLY bend pins 1 and 12 up

Solder flying leads to pins 1 and 12 on 74HC4020

Solder flying lead to pin 12 on SIO 8530 chip

Use a SPDT switch to switch to 2 signals from 4020 to SIO

Replace 4020

Drill 1/4 inch hole in rear plate above RS232 slot

Insert switch and secure.

Thats it.  Throw switch for 9600 FULLDUPLEX operation, revert for normal.

NB: You still have to change buad rate in s/w.

I hope this information will be of use to some. I would like to thank Uk Agent

for trying to get info from Pac-Comm. It must be like trying to get blood out

of a stone.

Thanks Pac-Comm, no need to bother now I've done it myself!

73 de Dave G4WFQ @ GB7PET AUK.2404 19 Aug 90

0664 08/19 15:44  G4WFQ   ALL     TNC220 ON 9600 UO3

9600BD @WW       de:LU7AKC 13.08.93 13:37   0   3676 Bytes

IC22A & 9600 Bps

*** Bulletin-ID: 12255_LU7AKC ***

930813/1335z DB0GV , 930813/1334z DB0AIS, 930813/1331z DB0HOM

930813/1331z DB0GE , 930813/1318z LX0PAC, 930813/1309z ON7RC

930813/1339Z ON1KGX, 930813/1246Z ON1KEZ, 930813/1228Z F6KIF

930813/1222Z FF6PTT, 930813/1136Z F5MSQ , 930813/1035Z F8REF

930813/0907Z F5GHV , 930812/2305Z F1HAQ , 930812/2049Z F1MAC

930813/0034Z F1OYA , 930812/1616Z F1ONT , 930812/1404Z F6CNN

930812/1352Z F6FBB , 930812/1345Z F5JGK , 930812/1306Z F1PKI

930812/0331Z EA2SG , 930811/2146Z EA2KJ , 930811/2128Z EA2AAA

930811/2118Z EB3ETC, 930811/1944Z EA3DTF, 930811/1853Z EA3FHW

930811/1729Z EB3ERZ, 930811/1057Z EA3AHM, 930809/0245Z LU6DYD

930809/0234Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

Modifying the IC22A for 9600 baud operation

by Mike Curtis, wd6ehr and Dave Shalita, w6mik

This is one of the more difficult rigs to modify - most are even simpler!

This mod will make your IC22A into a dedicated 9600 baud packet radio.

Components with * are added to the crystal board.

Remove the trimmer capacitor for the transmit crystal socket you'll be using

for 9600 baud.  Replace with the above circuit.  The 4-20 pF trimcap should be

a good quality ceramic or other low-drift trimmer.  (Sorry - Radio Shack

doesn't have anything suitable.)

Connect 2 10K resistors to C-53 (both sides) and tie the other ends together.

This junction is where you'll pick off your RXA.

Turn R-67 (dev control next to P-1 and P-2) completely counterclockwise. This

kills signals that may get into the phase modulator.

Using a service monitor, or oscilloscope connected to the detector of a

receiver, adjust the varactor bias for the best eye pattern.

Alternate bias adjustment:

Use the k9ng's 4800 Hz "idle" tone to adjust your modulator for best linearity.

Using a service monitor, or a receiver with a fairly wide passband, adjust

varactor bias for the "best" sine wave.  Thanks to Brian Kantor wb6cyt for this

tip.

MODEM @WW        de:VK4TEC 12.08.93 23:34   0   1841 Bytes

FT290 MK II / 9600

*** Bulletin-ID: 6_VK4TEC ***

930812/2318z DK0MWX, 930812/2238Z PI8AIR, 930812/2258Z PI8VNW

930812/2255Z PI8MID, 930812/2251Z ON4AWP, 930812/2242Z ON1CED

930812/2326Z ON4TOR, 930812/2209Z ON1KPU, 930812/2303Z ON1KGX

930812/2150Z ON1KEZ, 930812/2127Z F6KIF , 930812/2118Z FF6PTT

930812/2034Z F5MSQ , 930812/1959Z F8REF , 930812/1817Z F5GHV

930812/1805Z F6AIM , 930812/1945Z FF1LEQ, 930812/1744Z FE6CNB

930812/1611Z VK3BBS, 930812/1148Z VK4WIA

From: VK4TEC@VK4WIA.BNE.QLD.AUS.OC

To  : 9600@WW

VK4TEC/TPK 1.80 Msg #:6  Date:12-08-93  Time:21:51:42

DO YOU OPERATE AN FT-290R MKII ON 9600 ?

IF SO COULD YOU TELL ME WHERE THE MAGIC

CONNECTION POINTS ARE ?

CHEER'S 73'S

ANDREW

DIVERSES @WW     de:LU7AKC 14.08.93 14:03   7   6217 Bytes

IC3220 & 9600 Bps

*** Bulletin-ID: 12256_LU7AKC ***

930814/1352z DK0MWX, 930814/1339z DB0IZ , 930814/1334z DB0OVN

930814/1314z ON5VL , 930814/1307z ON7RC , 930814/1356Z ON1KGX

930814/1257Z ON1KEZ, 930814/1236Z F6KIF , 930813/2024Z FF6PTT

930813/2020Z F5MSQ , 930813/1916Z F8REF , 930813/1528Z F5GHV

930813/0600Z F1HAQ , 930812/2241Z F1ONT , 930812/2040Z F6CNN

930812/2036Z F6FBB , 930812/2056Z F5JGK , 930812/2011Z F1PKI

930812/0427Z EA2SG , 930812/0209Z EA2RCP, 930812/0206Z EA2KJ

930811/2341Z EA2AAA, 930811/2324Z EB3ETC, 930811/1950Z EA3DTF

930811/1929Z EA3FHW, 930811/1809Z EB3ERZ, 930811/1057Z EA3AHM

930809/0245Z LU6DYD, 930809/0234Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

How to iface Ic-3220 & G3RUH

Hi everybody,

The following instructions tells you how to make the right connections to an

ICOM IC-3220 in order to be connected to the G3RUH 9600 bauds modem.

------------------------------- WARNING !! -------------------------------------

The following steps are not very dificult to follow, but are to be carried

by a person with a minimum experience in electronic radio equipment.

If you are not such a person, ask to a friend with the required knowledge

to do the job for you. Otherwise, permanent damage to your rig can result.!

Also, (evidently), this manipulation will invalidate the rig warranty, so

perhaps you prefer to wait till the end of its validation period...

The author of this note is in no way responsible for any damage that can be

caused to your equipment by doing this modification to it.

Do it at your own risk, and think twice before begin !

--------------------------------------------------------------------------------

Modification instructions

-------------------------

(n) = Number of screws.

Material required: Screwdriver, Solder Iron, Coax cable (see text)

1 - Remove (4) the BOTTOM cover. (the one without the speaker).

2 - Remove (4+2) the shielding plate covering the P.A. output module.

3 - Remove (2) and discard the bottom small plate covering the hole

behind the antenna coaxial cable. Restore the two screws in place,

just to store it for the future.

4 - Locate connector J2 (The white big one, near to front panel) which

is used to interface the two main boards. (16 pins).

5 - Locate pins 1, 2, and 15 of J2. Pin 1 is the nearest to the box side

of the rig. Pin 15 (the one before last) is located about the middle

of the main board.

Pin description:

1 - UHF TX stage modulator voltage input.

2 - Ground.

15 - UHF RX detector output.

If in doubt about pinout, see schematic diagram (Main B unit, left hand)

6 - Take two pieces of SMALL diameter coaxial cable, (Z:50 ohms preferred

by me, but ordinary AF coax cable will perform pretty well).

Aproximate diameter MUST be no more then 4 mm, and lenght as required.

Following Murphy's law be generous ...

7 - Join the two (outer) shield parts of coax cables and solder them to

pin 2 (ground) at the upper (visible) part of J2. Solder the inner

(active) part of one cable to pin 1 and the other to pin 15.

Soldering should be performed to the upper part of J2 with

EXTREME CAUTION, as the plastic support structure of J2 connector,

can be seriously damaged !!!.

8 - Route the two cables trough the main board, up to the hole near the

antenna cable, routing them in such a way, that traverses the small

square hole at the bottom part of the plate covering the final P.A.

9 - Secure the cables at the output hole with a BIG plastic strap.

10 - Reinstall (6) the cover of final P.A.

11 - Reinstall (4) the bottom cover of the rig.

12 - Solder appropiate connectors at the other end of coax cables.

End of installation.

---------------------------------------------------------------------

Notes: With this modification, only the UHF part of the rig is affected,

i.e. the mod will only work for the 432 Mc/s. band.

Also, some noise has been observed at RX detector output line, which make

the DCD led of the TNC blink, but this does not affect the overall

performance of the assembly.

As a guide, using the french TPK program, a serial (rs232) speed of

19200 bds, the well known G3RUH modem, and a standard TNC, a file of

1.5 Mbytes has been transmited in one hour, obtaining and average speed

of 364 cps. (TPK measured).

73's QRO of Andreu.    EA3CQY @ EA3MM       Barcelona.Catalunya.Spain

DIVERSES @WW     de:LU7AKC 14.08.93 18:30   7   9902 Bytes

MFJ1270 & 9600 Bps

*** Bulletin-ID: 12258_LU7AKC ***

930814/1828z DB0GV , 930814/1814z DK0MTV, 930814/1756z DB0HOM

930814/1740z DB0GE , 930814/1724z LX0PAC, 930814/1720z ON7RC

930814/1729Z ON1KGX, 930814/1628Z ON1KEZ, 930814/1604Z F6KIF

930814/0025Z FF6PTT, 930813/2237Z F5MSQ , 930813/2008Z F8REF

930813/1736Z F5GHV , 930813/0837Z F1HAQ , 930813/0808Z F1ONT

930813/0309Z F6CNN , 930813/0255Z F6FBB , 930812/2215Z F5JGK

930812/2036Z F1PKI , 930812/1613Z EA2SG , 930812/1607Z EA2KJ

930812/1545Z EA2AAA, 930812/1526Z EB3ETC, 930812/1542Z EA3DTF

930812/1340Z EA3FHW, 930812/1248Z EB3ERZ, 930811/1057Z EA3AHM

930809/0248Z LU6DYD, 930809/0235Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

MFJ-1270 & SAT.

Hello Mark,

Name is George. QTH: Peachtree City, GA. Saw your message and thought I would

tell you about the 19200 mod. to the 1270 terminal port. I just got a 1270 and

just completed this mod. It is very easy and only takes a few minutes. The

modification is in the 1270 "Fast Start" handbook that I got with my 1270, but

if you have an older 1270 it might not be in the book. The following mod. is in

the Appendix section of the 1270 book here:

1. Remove motherboard out of the chassis .

2. Locate the dip switch SW2 ( dip switch for selecting the computer port rate

and radio baud rate ) and IC U1.

3. Cut the trace between SW2 and U1. ( Note: rather than do this, I simply bent

the pin # 1 on U1 out of its socket to prevent a destructive mod. ).

4. Connect a jumper from U1 pin # 10 to SW2 pin # 1 ( this is where the dip

switch has its 300 baud position ). Be sure to connect this jumper wire to

the "ON" side of the switch ( when the switch is in "ON" position it will

close connection ).

5. Ensure that U3 is an LM384 or TL084 or equivalent.

6. Now when you select a terminal baud rate of "300" on the dip switches, you

will get a baud rate of 19,200 instead.

Hope this helps, Mark. Let me know if you need more info.

BEST 73, George, AB4KN @ UO-22.

===============================================================================

Hi there Mark, the handle here is Clarence, QTH Albert Bridge, Nova Scotia.

I am sending you a file I have had for some time that I picked off AO-16 or

LO-19 some time ago. It is the 19200 baud mod for the 1270, I have not tried it

myself but it makes sense to me what is being done and I think it is one of the

mods incorporated by a lot of the other chaps. I am using an MFJ-1278 and I

have modified it using a mod I got off one of the birds and it works fine. I

called MFJ and put some heat on them for a mod and they came up with a mod

after many calls, that was very cumbersome. It might be worthwile to give them

a call anyhow and remind them that you are using their equipment and maybe if

enough people call thay will become aware that they should get into the 20th

century and put 19200 baud in their TNCs.

Enough soapboxing here is the mod. I have left the callsigns in there as I

don't take credit for it I am just passing it on.

Hope it works for you and all the best.

"  MOD ONE "

MODIFICATION FOR MFJ 1270  TNC

THIS MOD ALLOWS YOU TO USE 19200 BETWEEN THE TNC AND COMPUTER .

THIS MOD WAS PROVIDED BY HB9AQZ...HANSPETER

THE DIAGRAM IS BY KB8HAV...CHARLIE

1) replace U3 (LM324 or LM348) with a TL074, TL084 or MC33079P

2) Cut trace from U1 pin 2 to SW2 pin 1 (300 bps position!) see ** below .

3) Wire U1 pin 10 to SW2 pin 1 , see *** below .

__________________________

When you set SW2 position 1 to on, you get now 19200 bps terminal rate,

instead of 300 bps. (300 bps is no longer available!)

I placed miniature relays on the pcb near SW2 and J-4 to switch the radio

and terminal rates between 1200-9600 for packet radio and 9600-19200 for

UO-22, otherwise you must change the switches in the back every time you

change the mode of operation!

original trace

**

///////////( cut here )///////////

/                                 |

/                    8         pin 2  1

x------------------   |-------------|--|

sw2 >     | * * * * * * * * |        NEW    |   |     U1        *

| 8 7 6 5 4 3 2 1 |        WIRE   |   |-.--------------|

-----------------         ***    |   9 10             16

* * * * * * * *                 ------x

pcb 9  11  13  15 16 pcb

10  12  14

_______________________________________________________

pcb = pc board connections (pin 9 to 16 are shown for reference only)

sw2 > = Has the  numbered switch positions for setting the baud rate.

These numbers can be seen from the rear of the tnc .

x-----------x = New wire that is added from sw2 pin1 to U1 pin 10 . This is

the only wire added .

///////////// = Original trace that must be cut . It ran from sw2 pin1 to

U1 pin 2 . This is the only trace that must be cut .

U1 is a MC14040b in my 1270 .

The only mod to U3 is to replace it . ( no trace to cut here )

-------------------------------------------------------

THE TL084CN WAS ONLY $1.50 HERE IN KENTUCKY .

NOTE : This drawing is not to scale . You will find sw2 and u1 on back edge

of the pc board next to 12 volt input jack.

All pin positions are shown from the TOP VIEW  of the pc board and the tnc

LED's positioned at your chest.

All cuts and connections are made on the bottom side of pc board ,

keep this in mind when you are identifying the needed pins.

I would like to thank....Hanspeter  HB9AQZ ....for this mod.,

it has make a hugh difference in the down load bps from UO-22 at my station.

CHARLIE

KB8HAV

LOUISVILLE KY.

" MOD TWO "

Hi Clarence,

Sorry to be so late with the answer, I finally found the mod!

I got it from VE8DX, Bob...so here she comes back up North agn, hi!

73 Dave WB6LLO

This is a relatively easy modification that can be performed to MFJ 1270-74 to

allow 19.2 asynchronous communications between the TNC and your computer.

First **** replace U3 (LM324) with a TLO84cn or TLO 94 FET op-amp. the LM324

craps out at 9600 Baud and is dead at 19.2k

(my LM324 works fine at 19.2 k ---Dave)

Cut the Pc trace that goes to Pin #2 of U1 (divider) about 1/8" from the IC

itself.  Solder jumper from pin 10 of U1 to the trace you have just isolated.

Solder to the side of the trace that goes to pin 1 and pin 6 of the sw2 baud

rate selector switch.

This supplies 307.2 khz clock (19.2/16) to pin 1 whic is now 19.2 k to the

modem instead of 300 baud.  Turning on sw1 and sw7 will allow 19.2 k baud on

the serial port and 1200 on the radio port.

The 19.2k baud rate is unreliable with the pms code.  We are using the TAPR

1.1.6 code (or lower without PMS and having excellent results on the serial

port.  The radio port is currently running at 9.6k and is connected to a Paccom

nb96 modem.

***********************************************

Clarence, the above is verbatim from ve8dx.  I made a few additional mods on my

1274.  I mounted  a power switch outboard on an "L" bracket mounted to the

right hand side of the modem.  It is difficult to access the back of my modem

the way my panel is arranged.  I also mounted a three position switch that is

in parallel with the dip switch on the back that selects the rate.  There is

room just above the con and power leds.  I can now control the power and select

1200, 9600 or 19.2k from the front panel. Sori I couldn't get this to you

sooner, but had a problem finding the mod. It is now filed properly, hi!

73 Dave WB6LLO San Diego, CA

I suddenly realized I had two mods for the 1270/1274 so I have sent both as you

are aware I am sure the only basic difference between the 1270 and the 1274 is

the fact the 1274 has the tuning indicator.

Have fun ... 73 ... Clarence ... VE1HD -=>

GERAETE @WW      de:DL1YDD 18.08.93 20:56 180   2410 Bytes

FT780 & 9600 Bps

*** Bulletin-ID: 188313DB0IZ ***

930818/2053z DB0SIF, 930818/2051z DB0GV , 930818/2050z DB0AIS

930818/2047z DB0HOM, 930818/2044z DB0GE , 930818/1949z DB0IZ

de DL1YDD @ DB0IZ.DEU.EU

to GERAETE @ WW

Modification of the Yaesu FT-780/FT-480 for 9600 bps use

========================================================

The necessary modifications of the FT-780 R are quite simple. All

connections can be made without removing the main PCB.

The receiver AF *MUST* be taken directly from the discriminator. A

suitable point for this is the cathode of D02. This part is soldered

in with the ring to the top. If the transceiver is so positioned that

the tuning knob faces towards yourself, D02 is located in the lower

right of the PCB, near the ceramic discriminator, a small grey box,

abt. 5 mm x 5mm big).

The transmitter AF is to be fed to the upper left pin (refer to the

circuit diagram!) of the modulator unit. This point is also acessible

without removing the PCB. Just above of J9 there is the resistor

R 265 (labelled C 265 in my unit, 10 kOhm instead of 4.7 kOhm). You

have to remove the varnish of the upper lead of this resistor. The

TX AF is to be fed to the upper lead of this resistor.

The normal voltage level from the microphone amplifier at this point

is about 2 Vpp (just not clipping). The value should be adjusted by

VR1 of the G3RUH-FSK-modem.

Both points can be found in the circuit diagram, part "MAIN CIRCUIT".

The only question that should be left is how to get these two points

to the outside of the radio. I decided to give out the possiblity to

switch the 1750 Hz tone from the microphone (not used in US versions!).

I gained two line at the microphone plug.

For testing purposes, a 60 kB text files was transferred. There were

no retries. Reception of UO-22 and KO-23 is possible without any trouble.

Even if signals are weak (AOS/LOS), decoding works well.

vy 73 de Hartmut DL1YDD @ DB0IZ.DEU.EU

P.S.: Be careful when using this information on "Sommerkamp" radios.

They may be different (like a FT480 I saw recently).

MODS @WW         de:IK6SBE 20.08.93 10:22   0   3888 Bytes

TM702&9600 BPS

*** Bulletin-ID: 271_IK6SBE ***

930820/1015z DB0KG , 930820/1005z DK0MAV, 930820/0959z DB0DNI

930820/0943Z DK0MNL, 930820/0438Z 4X1RU , 930819/2342Z I5SGG

930819/2057Z IK4MGV, 930819/1434Z I4FP  , 930819/1402Z I4UJB

930819/1116Z IW4BYK, 930819/1437Z IK6NUK, 930819/1022Z IK6FED

930819/1121Z I6BNW , 930819/1016Z IW6BET

IK6SBE/TPK 1.81 Msg #:271  Data 19/08/93  Ora 08:24Z

KENWOOD TM702 -> 9600 BAUD

TRADUCTION BY IK6SBE

The following description refers to the modification  of RTX double band

KENWOOD TM702E with the pourpose of enabling the operation  at 9600 Baud

with the use of modem G3RUH.

The radio functions very well and herebelow the details.

Please  note  that i have  made  the  necessary  connections on the main

grid, souldering side, on minus of the PTT.

For  the  reception  i have connected the audio plug for the  two meters

directly on the PIN 12 of IC 11, and for the 70cm directly on the PIN 12

of IC 3.

For the trasmission on 2 meters  i made the contact between the resistor

SMD R70 of value 0  and the  trimmer  VR4 while  for the 70cm i made the

contact between the resistor SMD R67 with value 10K and the trimmer VR3.

I souldered the plug on  PTT directly on the back of the microphone plug

on  PIN 2.

For the  connections i have used earthed 5 poles cable and earthed it on

the  MIC AMP earth, thus on PIN 1 but earthing can be done anyhow.

Initially i have tried to keep together the connections of the two bands

thus using the same conductor for the  RX of the the 2 meters and the RX

of the 70 cm as well for the TX.

However i have obtained etter results during transmission with  separate

connections. This explains  the  use  of  5 poles cables. Eventually the

afore  mentioned  cable was  been inserted through  the same hole of the

power supply cable.

Of course i have removed the cut protection rubber on the hole.

For the utilization of one band or the other , i commute the connections

from the radio at the inlet of the modem in the TNC. That is all.

I remain at your disposal for further clarification.

Ciao from Lino ik3ngu@i3xty of Treviso

╔═══════════════════════════════════════════════════════════════════════════ª█

║                               * IK6SBE *                                  ║█

║                               ==========                                  ║█

║ ýýýýýý▄  ýýýýýý▄  ALDO * PORTO RECANATI (MC) * JN63UK                     ║█

║  ▀▀▀ýý█   ▀▀▀ýý█  PACKET ADDRESS: IK6SBE@IW6BET.AN.ITA.EU  [TPK_1.81 FWD] ║█

║     ýý█   ýýýýý█  TCP/IP ADDRESS: [44.134.240.27]                         ║█

║     ýý█    ▀▀ýý█  I.N.O.R.C.  (ITALIAN NAVAL OLD RHYTHMERS CLUB)   N° 293 ║█

║     ýý█  ýýýýýý█              * I T A L Y *                               ║█

║      ▀▀   ▀▀▀▀▀▀              =============                               ║█

»═══════════════════════════════════════════════════════════════════════════º█

▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀▀

YAESU @WW        de:IK2QED 21.08.93 02:56   0   5249 Bytes

FT-767 mods for 9600 BPS !!!

*** Bulletin-ID: 218313OK0PHL ***

930821/0254z DB0SIF, 930821/0254z DB0AIS, 930821/0242z DK0MTV

930821/0237z DB0GE , 930821/0056z HB9EAS, 930821/0049z HB9OS

930821/0048z DB0KCP, 930821/0148z DB0AAB, 930821/0048z DB0FSG

930821/0045z DB0LNA, 930821/0059z DB0RGB, 930821/0034z DB0WGS

930821/0012z OE1XGR, 930821/0046l OK0PHL

De IK2QED @ OK0PHL

R:930730/0724z @:OE1XGR.AUT.EU

R:930730/0810Z @:HA1VH.HUN.EURO [KOSZEG] #:127359 Z:9730 HF/VHF/UHF FBB5.15

R:930730/0313z @S50BOX S50BOX.SVN.EU Ljubljana Slovenija

R:930730/0030Z @:S50BBS.SVN.EU [KOPER, SLOVENIJA] FBB #:67881 B:43433_IW2GKO

R:930729/1448Z @:IW3QQV.UD.ITA.EU [Palazzolo] #:119121 Z:33056 FBB5.15

R:930729/1443Z @:IV3AVQ.UD.ITA.EU [ARIUD] [UDINE] FBB5.14d #:27432

R:930729/1441Z @:IV3VAV.PN.ITA.EU [Aviano] FBB5.14d #:105682 Z:33081

R:930729/1435Z @:I3XTY.TV.ITA.EU Luigi in Paese (TV) [SW di J.P. F6FBB v.5.14D]

R:930729/1433Z @:IW3GRX.TV.ITA.EU [Farra di Soligo] FBB5.15 #:28040

R:930729/1422Z @:IW4CQH.RA.ITA.EU [Massa Lombarda] #:109169 Z:48024 FBB5.15

R:930729/1416Z @:IW4CEA.RA.ITA.EU [Ravenna] Z:48100 FBB5.15 43433_IW2GKO

R:930729/1407Z @:I4UJB.FO.ITA.EU [Cesena] #:54998 Z:47023 FBB5.15

R:930729/1349Z @:I4FP.FE.ITA.EU [Ferrara, JN54TU] FBB5.15 #:28799 Z:44100

R:930729/1353Z @:IK4MGV.#PR.ITA.EU [Parma, JN54ET] FBB5.14b #:98532

R:930729/1343Z @:I5SGG.ITA.EURO [Florence] FBB5.15 #:138196 $:43433_IW2GKO

R:930729/1327Z @:IW2GKO.MI.ITA.EU [Muggio',JN45oo] FBB5.15 $:43433_IW2GKO

From: IK2QED@IW2GKO.MI.ITA.EU

To  : YAESU@WW

FT-767GX MODIFICATION FOR 9600 BPS OPERATIONS

---------------------------------------------

- Remove the bottom cover of the FT-767

- Remove the bottom cover (8 screws: 4 underneath, 2 on each side)

- Locate the IF-UNIT board. Is the biggest one of the two cards.

APPROXIMATIVE LAYOUT OF THE IF-UNIT

REAR OF 767

---------------------------------------------------------------

P86   C   P7101    j21  DATA IN/OUT

_    _   ____  _   _   +------+

| |  (_) |____|(_) | |  |      |

(D)-> |_|           c242 |_|  +------+

|

(A)------->|    _  (_) R383

(|  |  -(_)L21

Q65 |<---------------- Cut here (half position)

|    | <------+---- scrape varnish and cut

|   (_)R251   |     near the body of R251

|

|        _____

+---->  |     | <-- cut here

+---+   |

detail of  |   |   | <-----(C)

R251       |   |   |

|   |   |

+---+   |

|     |

-------------------

| | | | | | | | |

+-+-+-+-+-+-+-+-+-+-+

|  Q07  MC3359     <|

+-+-+-+-+-+-+-+-+-+-+

| | | | | | | | |

(B)---> | <----------------- scrape varnish from

()  (_)  ()c40            the resitance lead

c42

R39

- Once locate the components (jumper (A), resintances R39 (B) & R251 (C) and

the connector P86 (D) connect the point (A) to the point (B) and the point

(C) with the point (D).

Once you have make this simple modification you can connect yor TNC

to the DATA IN/OUT jack, to the rear of the FT-767 as follows.

+------------+

stereo jack  |            |-----=---=--\

3,5 mm    |            |-----=---=--/

+------------+ |     |   |

|     |   +--- Data in (from tnc)

|     +--- data out (to tnc)

+--- ground

Well that's it.  You are now ready for 9600 bps operation on your

FT-767GX.  If you have problems, reply; I will try to answer your

questions or solve your problems about this modification.

Best 73's de IK2QED - Stefano

+----------------------------------------------------------------------+

|  73 de IK2QED, Stefano  |  IK2QED @ IW2FPO  or  IK2QED @ IW2GKO      |

|  QTH Bovisio(MI) JN45NO |                                            |

+----------------------------------------------------------------------+

MODS @WW         de:IK0IZW 21.08.93 11:42   0   1955 Bytes

FT230R - 9K6

*** Bulletin-ID: 310_IK0IZW ***

930821/1136z DK0MAV, 930821/1123z DB0DNI, 930821/1105z DB0FP

930821/1101z DB0GV , 930821/1050z DK0MTV, 930821/1054z DB0GE

930821/1028z LX0PAC, 930821/1031z ON7RC , 930821/1118Z ON1KGX

930821/0620Z ON1KEZ, 930821/0600Z F6KIF , 930820/2304Z TK0KP

930820/1943Z IW0UHD, 930820/1833Z IS0NUP, 930820/2015Z IW0CFV

930820/2005Z IK0GDH, 930819/2103Z I0VNR

From: IK0IZW@I0VNR.RM.ITA.EU

To  : MODS@WW

IK0IZW/TPK 1.81 Msg #:310 Data:19-08-93 Ora:23:03Z

Hey OM, many thanks for reading this. I would like to try the

9600 Baud with my Yaesu FT230R. Is there someone who wants to

tell me the way to modify the RxTx? Thanks for all infos!

------------------------------------------------------------

Ciao es 73 de Renzo - IK0IZW - Aprilia (LT) - [TPK_1.81 FWD]

Home BBS: I0VNR  <------->  TCP/IP  Address: [44.134.32.182]

------------------------------------------------------------

DIVERSES @WW     de:LU7AKC 19.08.93 20:43   7   3264 Bytes

TM531 & 9600 Bps

*** Bulletin-ID: 12660_LU7AKC ***

930819/2012z DB0SIF, 930819/2006z DB0AIS, 930819/1952z DK0MTV

930819/1958z DB0HOM, 930819/1952z DB0GE , 930819/1943z LX0PAC

930819/1927z ON7RC , 930819/1645Z ON1KGX, 930819/1546Z ON1KEZ

930819/1519Z F6KIF , 930819/1347Z FF6PTT, 930819/1149Z F5MSQ

930818/2052Z I6BNW , 930818/1953Z SM2IRZ, 930818/1947Z SK2AT

930817/0658Z OH6RBG, 930817/0654Z OH6RBV, 930817/0732Z OH6AAO

930817/0615Z FE6CNB, 930817/0201z K3WGF , 930809/0235Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

TM531<>G3RUH 9600bd

Hello, Ed.

I received your mail today morningon Dec., 15.

I'm using TM-531 on G3RUH type 9600bps modem.

We ha local aria network using 9600bps mode..

It is working well,now.  But, beginning of this network is not easy.

You can connect G3RUH type modem as follow.

1. TXA

Take off the shielding plate under the main PCB.

Search for VR3 on main PCB.

VR3 is indicated on PCB "MAX DEV" for adjust maximum deviation.

The slider of this VR is connected to MD pin of VCO through C55

capacitor.

Connect TXA to MD pinof VCO using resister(56k to 100kohm) and

capacitor 0.1uF,connected in series,like follow.

TXA ---II------/\/\/\----MD(VCO)

0.1uF    56k to 100kohm

If you get weary to take off shielding plate,you may connect the TXA

to hot end of VR3, using the resister and capacitor as aforesaid.

TM-531 & 9600 Bps.

2. RXA

Search for IC2 on main PCB.

IC2 is a 16 pin hybrid IC for IF AMP and discriminator.

You can discriminate pin number of this IC from printed symbol on

the main PCB, and grand pin.

Pin 2 and 16 is connected to grand.

pin 5 is output for ceramic filter(CF1),and pin 7 is input from the

filter.

Connect RXA to pin 12(DET) directly.

9600bps on 1200MHz band is very difficult.

Because,frequency stability is not enough.

And, IF band width is not enough,too.

We are developing SMC transceiver for G3RUH modem on 144,430,1200MHz

band.

It has high stability PLL OSC(about 1ppm) and good linearity

modulator (voltage controlled X'tal OSC)

Sammy JE1RLA @ JE1RLA

DIVERSES @WW     de:LU7AKC 18.08.93 15:38   7   1435 Bytes

TR7800 & 9600 Bps

*** Bulletin-ID: 1037_LU9ECW ***

930818/1534z DB0SIF, 930818/1535z DB0AIS, 930818/1534z DB0HOM

930818/1532z DB0GE , 930818/1521z LX0PAC, 930818/1524z ON7RC

930818/1618Z ON1KGX, 930818/1451Z ON1KEZ, 930818/1424Z F6KIF

930817/2243Z FF6PTT, 930817/2253Z F5LO  , 930817/2054Z EA3FHW

930816/1026Z EB3ERZ, 930816/0918Z EA3AHM, 930814/0002Z LU6DYD

930813/0354Z LU5EO , 930811/2028Z LU7DW , 930811/1122Z LU2ABK

930810/2119Z LU9ECW

From: LU7AKC@LU9ECW.#EVITA.BA.ARG.SA.

To  : NET9K6@WW

·°

‗L

DIVERSES @WW     de:LU7AKC 18.08.93 00:44   7  14170 Bytes

TAPR & 9600 Bps

*** Bulletin-ID: 12259_LU7AKC ***

930818/0042z DB0GV , 930818/0041z DB0AIS, 930818/0039z DB0HOM

930818/0035z DB0GE , 930817/2327z HB9EAS, 930817/2140z HB9OS

930817/2114z DB0KCP, 930817/2048z DB0EIC, 930817/2145z DB0AAB

930817/2045z DB0FSG, 930817/2034z DB0LNA, 930817/2124z OE3XBS

930817/2019  OE1XIB, 930817/1603z OE1XGR, 930817/1636Z HA1VH

930817/0358Z HG1KZC, 930817/0331Z HA3PMF, 930817/0253Z HG3FAV

930816/2159z S50BOX, 930816/0902Z S50BBS, 930816/0809Z IW3QQV

930816/0806Z IV3AVQ, 930815/1509Z IV3VAV, 930815/1458Z I3XTY

930815/1230Z IW3GRX, 930815/1218Z IW4CQH, 930815/1207Z IW4CEA

930815/0517Z I4UJB , 930815/0520Z I4FP  , 930815/0111Z IK4MGV

930814/2230Z IK4NZD, 930814/1919Z IW2DXI, 930814/1851Z IW2FPO

930814/1823Z IW2FHH, 930814/1846Z IK2DFZ, 930814/1620Z IK7GFP

930813/0747Z F6FBB , 930813/0804Z F5JGK , 930813/0637Z F1PKI

930813/0620Z EA2SG , 930813/0608Z EA2KJ , 930813/0506Z EA2AAA

930813/0456Z EB3ETC, 930812/1655Z EA3DTF, 930812/1442Z EA3FHW

930812/1257Z EB3ERZ, 930811/1057Z EA3AHM, 930809/0250Z LU6DYD

930809/0235Z LU7AKC

From: LU7AKC@LU7AKC.#COL.CF.ARG.SOAM

To  : NET9K6@WW

TAPR FULL-DUPLEX (NEW) 9600 BAUD MODEM MODS BY KC6HPN

APRIL 23, 1992

These modifications correct several problems that can seriously degrade

the performance of the new (not the K9NG) TAPR 9600 baud modem.  They have

significantly improved the usability of my modem.  DCD (Data Carrier Detect)

operation, in particular, went from intermittent to rock solid.  All mods have

been implemented on my (first production run) TAPR modem board, which is

installed in a PK232MBX.

I welcome any comments and will answer any questions on these

modifications.  Just drop a message in my BBS mailbox,

KC6HPN @ WB6YMH.#SOCAL.CA.USA.NA.

73, Brian, KC6HPN

MODULATOR SIDE MODIFICATION:

1.  HOW:    Remove capacitor C5 (0.001 uf).

WHY:   Op-amps tend to oscillate when driving a non-isolated capacitive

load to ground.  U22B, a TLC274 op-amp is driving C5 to ground through C31.

U22B may oscillate at approximately 400 kHz due to the pole created by C5. This

oscillation will be superimposed on the modulating waveform. To diagnose this

problem, examine the eye pattern at TXA.  The eye should be sharp, if it is

fuzzy, the op-amp is oscillating.

NOTE:  C5 normally functions as part of an RFI filter to prevent noise from

entering the modem via the TXA line.  I have found that in my situation C2 and

R2 alone are sufficient.  Those with RFI problems should not remove C5 Instead,

place a 220 to 470 ohm resistor in series with C31 to provide resistive

isolation from C5.  This resistor will cause a slight loss in output drive.  If

necessary, you may increase output drive by changing R39 to 300 k.

NOTE:   The first TAPR kits used a TL084 op-amp for U22 which may be

slightly more resistant to oscillation.  However, the TL084 may still

oscillate.

DEMODULATOR SIDE MODIFICATION:

1.  HOW:    Change U4 from a TL084 to a TLC274 op-amp. Replace D2 (1N4148) with

a germanium diode (Bvr > 15 volts) or if not available with a

1N3600 silicon diode.

WHY:   Noise margin.  The data slicer output is extremely dirty.  An

increase in noise margin represents improved bit recovery

performance.

The TLC274 can pull its output nearly to ground, the TL084 cannot.  This

becomes important in U4A, where the op-amp is used as a schmitt-trigger

comparator (data slicer).  The op-amp output drives a digital input to ground

through diode D2.  D2, a 1N4148, adds 0.9 volts offset from ground, and a TL084

will add from 0.5 to 0.8 volts to this offset. Since a 74HCxx series part must

see less than 1.4 volts (VCC=5.0 v) to  sense a logic 0, the resulting noise

margin may be as low as 1.4-(0.8+0.9)= -0.3 volts.  This will cause bit

recovery errors and intermittent dropout of DCD (i.e. many retries) With the

TLC274 and 1N3600, noise margin is 1.4-(0.05+0.6)= 0.75 volts - much better-.

Germanium diodes, now somewhat rare, add only 0.35 volts offset and will boost

noise margin even further to about 1.0 volts.

A trade off is that the TL084 slew rate is twice as fast as the TLC274, making

it a faster comparator.  However, increased noise margin seems to be the

critical item in my modem.

2.   HOW:   Remove capacitor C24 (330 pf) and replace resistor R34 (2.2 k) with

a wire (zero ohm) jumper -BUT ONLY IF YOU PUT IN MOD#3 -

WHY:  Capacitance and series resistance are the kiss of death to digital

circuits.  Unless U19F is a schmitt-triggered gate, such as the 74HC14, (which

it is not), its output will glitch due to the slow risetime of the input

waveform.  Since U19F is in the data path, this will cause an increase in bit

errors, even when the modem is receiving a strong, well formed signal.

Ironically, the RC circuit formed by R34 and C24 may have been intended to

filter out glitches from the bit recovery circuit (U7B, U11D) which has

problems of its own (see mod#3).  Or it may have been intended to solve a

critical race condition by providing an RC delay (never use an RC for

this...use a flip-flop instead).  In the case of my modem, removing this

circuit once mod#3 was made improved performance.

3.   Okay, the easy part is over.  This mod is the most critical  and the most

difficult.  I'm also going to do "WHY" first this time, and "HOW" later.

WHY:   It is very, very necessary to add a double-buffering circuit

directly after the data slicer (U4A, D2).  A double buffer circuit consists of

two flip-flops connected in SERIES and operated off of the same clock (fig.1).

Its purpose is to synchronize external, asynchronous, signals to an internal

clock without causing glitches.

+5 v

_

|

D o------------------

|                 |

\                 |

10K  /                 |

74HC74 Dual D Flip-Flop            \                 |

/                 |

|                 |

---------------------------o                 o---

|10              |4        |                 |   |

----o----        ----o----     |                 |14 |

12 |  PR   | 9    2 |  PR   | 5   |                VCC  |0.047 uf

A -------| D   Q |--------| D   Q |---------- B              ---

Slicer    11 |       | 8    3 |       | 6   |  Buffered          ---

data in   ---|>CLK QN|-    ---|>CLK QN|-    |  data out      GND  |

|  |  CL   |     |  |  CL   |     |                 |7  |

|  ----o----     |  ----o----     |                 |   |

|      |13       |      |1        |               E o---

16x    |      -----------------o----------                 |

clock in  |                |                                 ---

C ----------------------                                  -

Figure 1:  DOUBLE BUFFER CIRCUIT

If the input signal to a flip-flop is in transition when the flip-flop is

clocked, the flip-flop will briefly become metastable.  That is, its output

will hover between logic 0 and logic 1.  In a short period, the flip-flop will

resolve the metastable state and settle to a 1 or a 0.  Two identical flip-

flops connected to in PARALLEL to the same asynchronous input (as are U7B and

U16) may not only produce glitches on their outputs, but will often settle to

DIFFERENT logic states.   Ooops!  Half the modem thinks it sees the bit, half

doesn't.  This causes bit errors and faulty DCD.

What the double-buffer  does is this: 1. On the first clock edge, the first

flip-flop clocks in the asynchronous data and resolves the metastable state. 2.

On the second clock, the second flip-flop clocks in the settled data and

provides a stable, glitch-free output.  Propagation delay and setup and hold

time requirements prevent the two flip-flops from clocking the glitchy data

straight through, even though they are using the same clock edge.

A deglitched, synchronous output is then passed to U16 for DCD and bit clock

generation, and to U7B for bit recovery.  Glitches formerly generated in U7B by

metastability and passed to U19F through U11D are eliminated, removing the need

for a (bad) glitch filter formed by R34 and C24.

This circuit improved both DCD reliability and data recovery considerably on my

modem.  If your DCD indicator (on the modem) flickers even during a strong

signal and you get many retries, this circuit will help.

HOW: The general method is to construct a circuit assembly using an IC

socket and mount the assembly "dead bug" style (upside-down) in the vacant U6

space.  If you are using the optional internal clock generator and U6 is

present, use any other convenient location.  Once assembled and connected, the

assembly is secured to the modem PC board by double-backed tape.

(A) Obtain a 14-pin PC mount IC socket, preferably with an integral VCC

decoupling capacitor, such as those made by AUGAT.  Turn the socket over. Using

fine insulated wire or resistor lead stubs, etc., construct the assembly as

follows:

I.   Wire pins 1, 4, 10, and 13 together

II.  Wire pins 3 and 11 together

III. Wire pins 2 and 9 together

IV.  Solder a 10k resistor between pins 1 and 14

V.   If you are not using a decoupled socket, solder a 0.047 uf capacitor

between pins 14 and 7

(B) Once the assembly is completed, place it to one side and examine the modem

board.  Using a soldering iron and tweezers, lift the non-banded end of diode

D2 clear of its mounting hole.  Next, using the same procedure, lift the end of

R19 nearest to U5, the 7805 voltage regulator.  Bend the hanging ends of D2 and

R19 together so that they touch.

(C) Using thin, flexible wire, connect point A (pin 12) on the double buffer

assembly to the joined ends of D2 and R19.  Secure with solder

(D) Using thin, flexible wire, connect point B (pin 5) on the double buffer

assembly to the mounting hole vacated by R19.  The area around the empty R19

hole is crowded, so solder carefully.

(E) Solder one end of a thin, flexible wire to U18, pin 11 on the back side of

the modem board.  Bring the end of the wire up and around to the top of the

board, routing it alongside U18.  Connect the wire to point C (pin 11) on the

double buffer assembly.

(F) Next, connect a wire to point E (pin 7), ground, of the double buffer

assembly.  Solder the other end of the wire to the center pin of U5, the 7805

voltage regulator.

(G) Connect a wire to point D (pin 14), +5 volts, of the double buffer

assembly.  Solder the other end of the wire to the pin of U5 furthest from the

modem PC board edge.

(H) Carefully turn the assembly upright and insert a 74HC74 dual D flip-flop IC

into the socket.  Make sure the notch on the IC is on the same side as the

notch on the socket.

(I) Apply a thin strip of double-backed tape to the IC, invert the IC and

assembly and stick it to the modem PC board in the unused U6 location.

(J) Carefully power up the TNC and check for normal operation.

HINTS:  Use heat shrink tubing wherever possible to minimize exposed

connections.  Carefully premeasure all wire lengths before soldering.  Wires to

the assembly should be long enough to allow access to the IC, for insertion and

removal, but no longer.

ADDITIONAL 9600 MODS

Collected by WÝTN

Date: 16 Sep 1994 08:20:16 GMT

From: ihnp4.ucsd.edu!dog.ee.lbl.gov!agate!howland.reston.ans.net!EU.net!uknet!signal.dra.hmg.gb!arrow.dra.hmg.gb!djch@network.ucsd.edu

Subject: MAXON 4020N and 9600

To: ham-digital@ucsd.edu

In article <MAPI.0016.006c696e657320203635393930303031.mlines@sni.co.uk>

mlines@sni.co.UK (Martin Lines) writes:

Has anyone got a MAXON 4020N UHF txer working on 9600?

Sponsored by the Midlands AX25 Packet Group (MAXPAK) I've now converted 3

4020N's for 9k6.

Quick answer:

Change the 2 21.4 8KHz filters to 15KHz (21M15 type)

Change the 455 filter to a CFW455E

Rx Audio from pin 10 MC3359

Tx Audio to junction R16/R17, .85 V p/p (could vary with different xtals)

Adjust T1,T2 for best eye pattern

Long answer:

Mail me for (18K) paper on Maxon 4020 (and also MAXPAK's G3RUH clone board).

Mail RICHARD@g1nzz.demon.co.uk for details of the MAXPAK board.

--David G8SQH

--------------------------------------------------------------------------

Hello, I got a package from Mel Whitten, K0PFX that contained his

modification tips for putting a Mitrek radio to work as a 9600 baud node.

He included a $5.00 bill to cover my expense for sending him the Bdale

Garbee work on the same subject. I had to again turn on my Mosaic system

and call the place in Colorado where I got it from. I again printed out

the information for Mel. I have an Optical Character Reader and decided

to see if I could convert Bdales printed file to a simple ascii file.

It worked and here it is:

Garbee and Garbee                            August 1994

Motorola Mitrek Radio Modifications for

9600 Baud Service

The Motorola Mitrek radios are readily available through

commercial radio surplus channels, and provide a very reasonable

platform for building robust 9600 baud packet links.

Various groups have published modification instructions for the

Mitrek series, however the modifications we found all involved

sufficiently significant changes to the transceiver circuitry

that the normal, analog tune-up procedures were invalidated.

Since we are also using Mitrek radios for our VHF and UHF voice

repeater RF decks, we were highly motivated to come up with a

set of mods for 9600 baud packet that preserved a single tune-up

procedure.  We believe that the point of diminishing returns for

tweaking the radio's IF path is low, therefore these mods are

simple but effective.

These modifications were developed by Derek Toeppen, WAOZTI,

with inputs from Bdale Garbee N3EUA, and John Conner, WD0FHG.

The work was done on behalf of the Pikes Peak FM Association.

The following instructions assume that you have a Mitrek in

front of you, and that you have a Mitrek manual to refer to for

component placement, etc.  Our local Motorola field service

office had no trouble obtaining reprints of the manual for us at

a reasonable price.  All mods have been thoroughly tested with

UHF Mitrek radios of the 3OW and 5OW variety, they should work

on VHF radios as well, but we haven't tried it.

Modifications for Simplex 9600 Baud Operation

To feed the modulation signal directly into the TX channel

element, we chose a method that does not require any

modification of the PCB traces.

Remove R50 1.

Remove C502, C503, and C504.

Remove R513, and R515.

Jumper MIC HI Audio to Channel element pin 4.

Short Jiunper from R501 through hole to R515 through hole.

The modulation signal can now be fed to the MIC HI pin (pin 1)

of the front panel control head connector.

No receiver modifications are required.  Just make sure the

channel elements are enabled.  For a single frequency radio,

just add jumper JU61 I to hard enable F 1.

Receiver audio is taken from the DET pin on front panel

connector pin I 1.

We make no modifications to the RF input/output, just let the

existing TX/RX relay do it's thing, and use the front-panel UHF

connector for your antenna connection.

Modifications for Repeater Operation

We have built one digital repeater using a 3OW UHF Mitrek

chassis.  We have built several audio repeaters with both VHF and

UHF Mitrek chassis with both 3OW and 5OW chassis.  The following

mods are for repeater operation, and are based on the 1200 baud

packet repeater we built.  If you want a 9600 baud packet

repeater, combine the above mods with the following mods.  If you

want either a 1200 baud packet repeater, or an audio repeater,

the following will be sufficient.

On the TX side, we remove R501, which prevents 9.5 volts from

appearing on the mic input.  The modulation signal can then be

fed in to the MIC HI pin (pin 1) of the front panel control head

connector.  Remember that the pre-emphasis and gain of the audio

path depends on the source impedance of the circuit driving it.

This impedance should be about 360 ohms.

On the RX side, the project is a bit more ambitious:

Remove CRI.  This keeps RX on when PTT is low (TX is on).

Remove CR403.  This prevents audio muting when the TX is on.

Install JU61 1. This hard selects channel element #1 for both

TX and RX.

Remove CR604 and CR608.  Install a jumper between node E

(collector of Q406) and the cathode through hole of CR608.

This makes a carrier detect signal available on the channel

element #4 select line.

Remove CR404.  This prevents the muting of receiver audio when

no carrier is present.

Receiver audio is now taken from one of the speaker outputs.  The

audio will have de-emphasis.  The audio level is determined by

the volume control.

To complete the project, we need to modify the RF cabling.  Note

that the TX cable is short, don't waste any length when making

these modifications!

Cut the RX and TX coax as close to the TX/RX relay as

possible.  Remove the TX/RX relay.

Add SMA connectors to the TX and RX coax.

We chose SMA's because they fit the cable well, and we have

through-panel SMA to N connector widgets that make for a nice

installation when the Mitrek is mounted inside a rack-mount box.

Drilling holes in the top cover of the Mitrek to feed the SMA'ed

cables out at the appropriate places makes for a neat

installation.

The radio can be tuned using standard Mitrek procedures.  Nothing

in these modifications alters those procedures.

---------------------------------------------------------------------

On Wednesday, September 28, 1994 12:11, Klarsen <klarsen@kazak.NMSU.Edu>

wrote:

> [some stuff deleted]

> Motorola Mitrek Radio Modifications for 9600-Baud Service

> [mods deleted]

Just FYI, Bill Lyman, N1NWP, of Wallingford, Connecticut

(lyman@a3bee2.radnet.com), has modified a number of Motorola Mitrek T-74 UHF

transceivers to make the local 9600-baud TCP/IP repeaters (Switches). I have

no idea specifically what his modification/construction procedures are, but

he might have additional ideas, if anyone is interested.

I know the Bethany (Connecticut) Switch/Repeater (44.88.3.1, I believe)

cranks out the TCP/IP packets at 9600 with ease, and running as a duplex

repeater is mighty quick! He uses two Mitreks (back-to-back), a set of cans

and an antenna, and away it goes!

--------------------------------------------------------------------

In article <05437.15021573@RAMail> you wrote:

: What is the prices you guys have seen them for?

I've paid anywhere from $75 - $125 each for UHF and VHF Mitreks in the 30W

or 50W variants, plus shipping, for untested units.  I figure $50 - $60 to

get the channel elements recrystalled and temperature compensated, and

currently am using Bomar for UHF and ICM for VHF, based on experience with

their ability to get the temperature compensation right the first time.  May

be less important to some of you, but my sites tend to be harsh.

Last time I checked, the local Motorola field office could get us manual

reprints for under $20 each, and their secretary handed me a couple of keys

out of her drawer when I walked in for the manual and asked.

Find a copy of Mobile Radio Technology and troll the ads in the back.  Make

sure you ask for a set of channel elements, you may want a cut-off cable which

they'll probably want $5 or so for (the mating connector is a bit odd), and

for some mounting schemes getting the baseplate is important, and typically no

charge if you remember to ask for it.  Can't think of anything else offhand.

73 - Bdale, N3EUA

--------------------------------------------------------------

We got a bunch of 30 W UHF Mitreks, cut for Canadian band (406-420 MHz), for

$140 each from C.W. Wolfe in Montana.  They also have 40W VHF (136-148 MHz)

units for about $90.

I'm told that this price is too high for the UHF ones, but on the other hand

they're bench tested and include a set of channel elements.  And all seven of

the ones we've bought have been very clean.

John

--

John Ackermann   AG9V

Internet:  jra@ag9v.ampr.org

Packet:    AG9V@N8ACV.#DAY.OH.USA

------------------------------------------------------------------

Hi, I ve played with the Mitreks for 9600 baud, among other radios. They are

great, once u get the IF set up right. The stock filters are for 12.5 KHz

Channels. This is the two way standard. And since the optimun BW for K9NG

modulation, with a non-dispersive filter is equal to the Baud Rate, in hertz,

and since radio IF filters have all sorts of group delay, especially at the

edges of the passband, where the K9NG signal spends most of its time, it should

be apparent that the stock filters wont work well. And, not too suprising,

that s been my experience with the stock MITREK recievers. Since to have another

station 12.5 Kc away from ur reciever, ur IF BW needs to be down around 9 Kc.

And that s about what they are, and the K9NG modulation needs a wider IF

filter than 9 Kc. If u had a FIR filter, 9.6 Kc plus the frequency error be-

tween transmitter and reciever (or the sum of the maximun expected error of

the reciever and the transmitter). Since the frequency errors at UHF , for

good radios, like these Mitreks is around a Khz, with a FIR filter u would

need a 12 Kc IF BW. But, the crystal filters in these Radios are very disp-

ersive, with loads of group delay right on the band edges, right during the

time where the K9NG signal resides, and puts most of its energy. So, with

the crystal filters, the BW needs to be even bigger. 15 Kc is a good number

from my experience.

Luckily, u can get such filters for the Mitrek. Details on this are in the

K0PFX notes, which I colaborated on.

The difference experienced here in the Packet Probability of Reception were

on the order of 10 to 1, using the stock filters, and the replacement 25 Kc

channel filters. This was over a 10 mile path non-line of site path to the

N4PCR qth, from my old qth. I had a 19 element Beam, PCR s station had a yagi

pointed generally in K9VXW's and my direction. The N4PCR station was 2 watts.

That is, N4PCR had a short Yagi, 6 or 7 elements.

K0PFX concurred that their links , which they had pulled the MITREK s out of

due to poor performance, did work with the 25 Kc Channel IF Modified MITREKS.

Now, this is not to say if u have line of site, and more signal than u need to

quiet the reciever, u might not get 80 or 90 percent Packet Probability of

Reception. But if ur signals are down there with that Fresnel loss on them,

like most people's signals are, than you will want to do these things right,

and make the appropriate IF filter modifications. We also tried one of the

IF filter mods published in PSR, that recommended taking out some of the IF

cans. The BW did improve, but the poles of the 8 Kc BW If Strip, whcih the

remaining IF filter can(s) were part of were still causing too much disper-

sion on the 9600 baud signal. A good way to judge these things is to run

two radios on the bench with shielded dummy loads. There should not andy

missed packets at all, ever, with this situation. Try it for 100 s if not

thousands of 256 byte packets. There should not be ANY misses. In this sit-

uation the misses are not going to be due to noise. They can only be due to

signal corruption by IF and other circuits.

73, Don. wb9mjn%wb9mjn.ampr.org@wb9uus.bradley.edu

------------------------------------------------------------

There are a number of uhf mitreks around here operating at 9k6 with

minimal mods performed.  However these are all mountain-top line-of-sight

sites and they seem to work well.  For weak signal paths I agree that the

K0PFX/WB9MJN IF mods are desirable.  I have modified three of the uhf

mitreks here so's to get the extra margin.  The 25 KHz wide filters from

Digi-key cost approximately $7 ea in lots of less than 10 and 4 filters

are required for each radio.  This is a simple mod to perform, just

unsolder the original filters and solder in the new ones, then perform

an alignment touch-up.

I have converted a low uhf mitrek for full duplex operation.  This is NOT

a trivial task and requires a good spectrum analyzer.  Unlike the older

micors which have separate rx and tx boards, the mitrek uses a single

board.  On full duplex, nasties from the rx oscillator chain gets into the

tx injection and creates fairly high level spurs around the transmitter

output.  These can be tamed with trial and error bonding and grounding

while observing the spectrum analyzer.  Not a quick process.

The difficult/costly part is in spliting out the antenna ports.  I removed

the antenna relay and installed two runs of mini-hardline, one to the rx

input and one to the tx output.  I used type N fittings with SMA adaptors

going to the mini-hardline.  This is expensive.  But I felt for the shielding

and isolation it provided within the cramped confines of the mitrek it was

worth it.

The easy solution for full duplex with the mitreks is to simply use two

radios.  The separate radios provide good isolation and shielding.  The

radios are small, so they don't take up much space.  This approach also

gives the advantage of having a standby spare in case of rx or tx failure.

73 de Jack

--------------------------------------------------------------------------------

In article <m0qqkfO-00021jC@dptspd.sat.datapoint.com> you wrote:

: I have converted a low uhf mitrek for full duplex operation.  This is NOT

: a trivial task and requires a good spectrum analyzer.

Interesting.  We've converted perhaps 4-5 Mitreks for full duplex operation,

including at least one low uhf, one high uhf, and a some VHF units.  In every

case, we had none of the nasties you describe, and we *do* have access to good

test equipment (working for HP has value from time to time!).  The only RF

problem we've had is one low uhf unit which put a smallish but annoying spur

on the 2m 1200-baud radio next to it on the shelf at a user's house.  He's

swapped things around in the meantime, and I hope to get time to go poke at

that radio before too long.

I have to assume there must be some variability in the radios.

: The difficult/costly part is in spliting out the antenna ports.

Agreed.  However, we didn't think it was all that bad.  We cut the two pieces

of coax off at the relay, drill holes in the top plate to fish them out through

rubber grommets near to where they originate on the boards, and put SMA's on

the ends.  We found some really neat chassis-mount N to SMA widgets somewhere,

and have used those to present N connectors on the rear panel of the rack

chassis we mount the Mitreks in.  Oh, that probably deserves comment too, what

we do is to put screws through the bottom mounting plate, nuts on the outside,

then let the screws project down through holes in the rack chassis, and put

wing nuts on the botton of the box.  Ergo, to swap a Mitrek out of the rack

box, we pull the two SMA's loose from the rear-panel feedthrough adapters,

undo the mating connector with audio/DC from the front, reach under and undo

the three wingnuts, and pop the Mitrek out.  Best of both worlds, in that we

have a very clean rack-mount module, but it's easy to swap the radios in a

couple of minutes when we're in the oxygen-starved environment above 14k feet.

: But I felt for the shielding

: and isolation it provided within the cramped confines of the mitrek it was

: worth it.

The fact that we exit the coax near where it originates on the board through

the top plate achieves a similar result.  Your approach makes equal sense to

me.

: The easy solution for full duplex with the mitreks is to simply use two

: radios.

We're too cheap.  :-)

73 - Bdale, N3EU

----------------------------------------------------------------

FT-708

Audio output from FT 708 R receiver's Discriminator for 9600bauds

Here is a very simple way of taking audio direct from the discriminator.

Connect a flying lead to pin 9 Q 107 (MC3357P) that is located at the

opposite side of the CPU.

You can also take the signal from the GREEN cable that goes to the

TONE SQUELCH optional unit.

TxAudio input into FT 708 R Varactor diode for 9600 bauds

You must find the D 209 Varactor diode (FC 53) locate near the output

DC connection,or near the VR 202 trimmer in the PLL Unit.Then you

must insert the TXA signal in the catode of D 209 and the 9600

modification is made.

My FT 708 can run 9600 operation very well with a TX DELAY value of 17.

Best 73 from IW1CXZ Andrea @I1LYM.#TO.ITA.EURO

Email iw1cxz@radio-gw.cisi.unito.it

Andrea Borgnino IW1CXZ

-------------------------------------------------------------------------------------

Date: 31 Oct 94 21:57:00 GMT

From: steve.diggs@totrbbs.atl.ga.us (Steve Diggs)

Subject: GE Master II mods?

-> Newsgroups: rec.radio.amateur.digital.misc

-> From: grsk@vectorbd.com (Gary Skuse)

-> Subject: GE Master II mods?

-> Date: Mon, 31 Oct 1994 19:19:28 GMT

-> Message-ID: <CyJxoG.2x6@vectorbd.com>

->

-> Our local club just acquired a GE Master II for use on the 2M user

-> port. Can anyone point me toward information to modify this radio for

-> both 2M use (i.e. how do I tune it for the 2M band) and use at 9k6

-> bps?  All help will be greatly appreciated.

->

-> 73, ka1njl

Glad to hear other folks beginning to use this excellent radio for

packet use. I have a GEMII of the 70cm variety that I am modifying for

9K6 packet repeater use, and have been working on the mod. since

acquiring the unit at Dayton this year.

First thing to keep in mind: the MII comes in MANY different

manufacturing versions, and it is quite easy to inadvertantly pick up

the WRONG docs for the exact unit you have. There will be a paper tag on

each PCB assembly section; go by THAT number to match docs. Disregard

any other info! Next, a simple question: in the control head area up

front of the radio, you will find a bank of pins labelled P904. Is one

of those pins marked "DISC" ??

On the receive side, my interface worked the first try. The DISC pin

noted above comes right off the IF. To improve bandpass characteristics

of the IF receive audio, the final RC filter may have to be removed, but

anyway, I was able to copy beacon text without ANY modification.

On the transmit side, the general approach is to use the temp.

compensation curcuit in the ICOMs as a way to impose 9k6 modulation on

the transmit signal. Only problem is that there is a 5 volt bias on that

line, and I'm just now getting around to constructing an OP AMP curcuit

to take the 9k6 audio and add the proper DC bias. A high performance OP

AMP is needed to not distort the 9k6 audio; I'm using the NSC LMC-6082.

Anyway, maybe between us we can come up with a "cookie cutter" MOD file

for this excellent radio.

Regards,

Steve Diggs

----

Top Of The Rock BBS - Lilburn, GA         SYSOP: Steve Diggs

UUCP: totrbbs.atl.ga.us               Snailmail: 4181 Wash Lee Ct.

Phone: +1 404 921 8687                           Lilburn, GA 30247-7407

-------------------------------------------------------------------------

In article <Cyn6F2.GA7@nntpa.cb.att.com>

jkbe@lena (John Bednar) writes:

> Gary Skuse (grsk@vectorbd.com) wrote:

> : Our local club just acquired a GE Master II for use on the 2M user port.

> : Can anyone point me toward information to modify this radio for both 2M

> : use (i.e. how do I tune it for the 2M band) and use at 9k6 bps?  All

> : help will be greatly appreciated.

> : 73, ka1njl

>

> For our GE ExecII I use the following Tx modulation circuit at 9600bps:

> (PacComm Tiny II Mark 2 TNC)

>

> - Short out C8 in the PacComm TNC (it's on the main board).

> - I use 174U coax between the TNC and radio. Connect a 1uf non-polar

>   ceramic capacitor and a 10K resistor in series. Attach the resistor directly

>   to the varactor diode in the channel element. Connect the coax center

>   conductor to the capacitor. Ground the shield of the coax on the channel

>   element.

> - Set the PacComm modem to high audio level TX with the jumper

>

> This ExecII has been in service for over a year now.

>

> Has anyone played around with capacitors across the GE IF filters

> to widen the RX response? I had limited time last weekend and tried

> several values (3pf, 5pf, 30pf). All produced results that were

> worse than the stock receiver.

>

>

> John, WB3ESS

>

-----------------------------------------------------------------------

Date: 27 Nov 94 11:57:49 GMT

From: healy@ee.unr.EDU (Bill Healy)

Subject: 9600 mods for micor?

>

>     I was wondering if anyone has had any succes in using VHF

> micors for 9600 baud packet. I have a couple of them either to

> modify or to use as boat anchors......

>

>  Randy ve2nrn@ve2noq.ampr.org

>        VE2NRN@VE2NOQ.#NOQ.QC.CAN.NA

>

Here's a message that went around on packet early this year that explains

the mod for UHF micors, I think it will also work on VHF. You can contact

the author and see if he knows more.

Bill N8KHN

healy@moriah.ee.unr.edu

Revised UHF Micor Modification...

R:940515/2054z @:N7MRP.AZ.USA.NA #:57647 [FBB-Internet-Gate-Phx] Z:85016

From: N7MRP@N7MRP.AZ.USA.NA

To  : MICOR@USA

This is an updated text file containing the MICOR 9600 Baud operation

modification.  The only major changes are the X-TAL filter changes and the

wiring of the first stage of the exciter board "hot."  The changes are

noted with a > in front of the text.

First, the MICOR was modified for "stand alone" operation (without a

control head) - on P901 (the big connector in the front), pin 11 is

jumpered to pin 1 (Gnd), pins 8,3 & 22 are connected together and to a

2amp fuse connected to the big A+ feedthru going into the PA.  All these

jumpers are done on the control(interconnect) board, except for mounting

the fuse itself.  Unless you want speaker audio to listen to, there is

no need to install volume or squelch pots, and the audio amps are not

used in this application.

Second, we did a couple mods that seem to help prevent problems before

they arise: add a capacitor (at least 20 microfarads, I use 220 uf)

across C903 on the control board (this is the tubular electrolytic

located just to the rear of the 9.6v regulator transistor heatsink). Add

a diode (1N4001 or better) from P901 pin 16 to any of the A+ pins

(3,8,18), with the banded end to the A+ pin.  (this is a "spike"

suppression diode)

Third, one mod on the Audio/Squelch board: replace C203 with a 1-10uf

electrolytic/tantalum- I recommend at least a 20v rating.  This improves

the low frequency end of discriminator audio.

If your radio has a CTCSS encode board plugged into the exciter,

remove it.  Install JU304, which is located immediatly adjacent to the

encoder board connection on the exciter.  Depending on which exciter you

have, the encoder will have either 7 or 11 pins - in either case the pin

you will use to inject transmit "audio" is the one adjacent to the big

square hole, towards the outside edge of the radio (pin 6 for 7-pin, or

pin 10 for 11-pin ones).  Connect a shielded lead from this pin to an

unused pin on P901 (I use pin 27).  On the exciter, trace the land from

the pin you just connected to and you will find an 82k resistor -

replace it with a 10k resistor (1/4w) and a 10uf capacitor in series.

(Some exciters already have a 6.8uf cap in series - if you are SURE

yours has the cap, you don't need to add another!

>   On the receiver RF/IF board, remove the SENCOND (2nd) and FOURTH (4th)

> crystal filter "cans" that come after the IF amp, and replace them with

> 1000pf disc caps (not critical).  Note that the caps DO NOT connect to

> the center ground pin!  This mod is CRUCIAL to making the receiver work at

> 9600!

>

>   The previous mod required the removing of the LAST TWO crystal filter

> cans and after about 6 months of operation,  the receiver IF began to

> oscillate badly and thus retries went through the ceiling.  The data being

> received was  quite jittery on  a scope and  thus not decodeable.  The

> above new modification has been stable for the past year now, and there

> have been absolutely NO problems since then.

>

>

>   The next step is to wire the first stage of the exciter "hot."  Find

> regulated continual 9.6v and apply it to the first stage of the exciter.

> The has allowed for a much quicker key up time.  Before this

> modification it was necessary to have longer TXD's to be certain the

> data was decoded on the other end completely.  You will find by completing

> this modification, you can operate with a TXD of 8ms and I've even made

> it work at 5ms.

> This is pretty amazing.  On some of the longer paths like I have that

> contain path fades etc.  I have found that with the MICOR radios,  that a

> TXD of 12 or 13 is just right and I have continued to maintain a retry rate

> of 0-1% for the past 2 years now on all links that contain MICOR/MITREK

> mixes.

Thats ALL there is to it - naturally, you need to tune the radio up,

but I figure you already knew that!  Connect your transmit audio to

whichever pin you connected the jumper to, receive audio (discriminator)

is on pin 20, PTT is pin 16, and 12v goes to the two big pins in the

middle (pins A and B).

I would advise obtaining a service manual for these radios if you plan

on using them - it is Motorola part # 68P81015E70, and you should be

able to order one through any Motorola service shop.

The original mod was written by Hartley J. Gardner, W1OQ@N7MRP.AZ.USA.NA

and implemented by Daniel J. Meredith, N7MRP@N7MRP.AZ.USA.NA

The revision to this mod has been worked on by both of the above mentioned

parties and has been revised by Daniel J. Meredith, N7MRP@N7MRP.AZ.USA.NA

<<<-------------------------------------->>>

Daniel J. Meredith

Arizona Packet Coordinator

Voice/Fax: (602) 956-2566

Data     : (602) 912-0225

Ax.25    : n7mrp@n7mrp.az.usa.na

Internet : daniel.meredith@aznetig.stat.com

[Entered at 13:53 MST on 15-May-94]

----------------------------------------------------------------------------