THE GRAPES 56 Kbps MODEM

And Related Information

Collected by WÝTN

Rev 2, 11-7-94

Date: Mon, 18 Jul 1994 13:32:20 GMT

From: ihnp4.ucsd.edu!swrinde!gatech!kd4nc!ke4zv!gary@network.ucsd.edu

Subject: Grapes modem

To: ham-digital@ucsd.edu

GRAPES is the Georgia Radio Amateur Packet Enthusiast's Society, i.e, the Georgia state packet networking group. GRAPES distributes

kits of the WA4DSY 56 kb RF modem described in the ARRL 6th Computer

Networking Conference Proceedings. Since it's introduction in 1987,

GRAPES has shipped thousands of units worldwide, and uses them internally

for the state network, which now extends into Tennessee, Alabama, and

North Carolina. They are used in many countries, including Britain,

Sweden, Japan, and the Netherlands. The largest group of amateur users

outside Georgia is in Canada where they have setup a full duplex network.

GRAPES has also sold units to governments and research agencies where they

are used for telemetry of remote sensing data, and for remote control.

The modem uses MSK (Minimum Shift Keying) and occupies 70 kHz bandwidth

for a 56 kbit/sec data rate. The ARRL bandplans allocate 100 kHz channels

for their use on several bands. The proposed 219 MHz band will all be

100 kHz channels for 56 kb trunking operation. The modems have a tracking

data detector circuit that will continue to deliver full sensitivity error

free data if the carrier center frequency drifts +/- 5 kHz. It will

work with larger amounts of drift, but error rates climb if signal

strength is not high enough.

The modem is a true RF modem. It *is* the radio, so the hassles of

trying to modify radios to work at these speeds is absent. It outputs

and inputs in the 10 meter band and a transverter is required to shift

the signal up to the UHF band of choice. Units such as those made by

Microwave Modules, Sinclair, and SSB Electronics will work. Even the

Hamtronics XV4 transmitting converter and receiving converters will

work. The only requirement of the transverter is that it be a *linear*

transverter since MSK has an amplitude component of 3 db. If a class

C transverter or amplifier is used, the modem will still work, but

the occupied bandwidth increases significantly.

The modem is, of course, a synchronous modem and outputs Tx and Rx

clocks to the digital interface. While you can modify a TNC2 to

work with the modem, and GRAPES includes instructions to do that,

it's recomended that you use the Ottawa PI card or the Gracillis card

in a PC instead. Since these cards have direct DMA with the PC's main

memory, they are better suited to support long 56 kb data bursts.

If you use the TNC2 approach, you're limited to an effective

thruput of 19.2 kb because of limitations in the TNC. The radio

side will run at 56kb, but only in bursts of less than the TNC

buffer length since it can't handle the RF side and the serial

link to the computer both at full speed at the same time. However,

you can use a Kantronics Data Engine as your TNC or switch. It has

the horsepower to keep up with continous 56kb data streams. You

can also use a Mac directly. It's serial port is capable of

synchronous operation at 56 kb.

Basic pricing on the kit is $250. This includes boards, all parts,

and instructions, less case and power supply. You have to furnish

your own enclosure and +/- 5 volts. You can also buy bare boards

and waveform ROMs for $125, but the full kit has the parts priced

at GRAPES' cost, so it's unlikely to save you any money to go that

way.

Note: these modems do not interoperate with lesser modems at lower

speeds, so unless there is already 56kb activity in your area, you

should arrange with another ham to buy pairs of them. I've built

a half a dozen of these modems, and assembly and tuneup are not

difficult. However, I would caution that these are not Heathkits,

and some experience with digital and RF circuitry is helpful. The

minimum test equipment you will need is a dual trace oscilloscope

capable of XY operation to at least 100 kHz. (That's almost any

dual trace scope.) The usual RF test equipment, signal generator

and monitor receiver, are also helpful, but you can just loopback

the transmitter to the receiver for tuneup and testing.

Gary

--

Gary Coffman KE4ZV          |    You make it,     | gatech!wa4mei!ke4zv!gary

Destructive Testing Systems |    we break it.     | uunet!rsiatl!ke4zv!gary

534 Shannon Way             |    Guaranteed!      | emory!kd4nc!ke4zv!gary

Lawrenceville, GA 30244     |                     |

------------------------------

Heatherington WA4DSY 56KB RF Modem

----------------------------------------

GRAPES (a Non-Profit Volunteer Organization of Georgia Amateur Radio

Operators) is selling full kits of the WA4DSY 56KB RF Modem for $250.

The proceeds from the kits support our packet network building activities.

You will need a transverter with a 28 MHz I.F. for the band of your

choice, a case, connectors, power supply and frequency determining

crystals.  The printed circuit boards, documentation, State EPROM,

support diskette and all other parts are supplied.

The transverter chosen should switch as quickly as possible in the

5-15 msec range to be effective.  We use the Microwave Modules MMt 432-28S

units; however, these are getting difficult to find except as used ones at

hamfests.  The last few we purchased new were also pretty expensive ($389).

We believe that their 220 MHz units were quite a bit cheaper, but we've never

bought any new.  On the hamfest market, we have been paying $125 to $185

for used, normally working units.

Maple Leaf Communications (Bob Morton, VE3BFM) is shipping a 220 transverter

which has good charactertistics and an even better price tag ($279).  We

have two of these units and they seem to work fine.  Contact Bob at

705-435-0689 or RR #1, Everett, ONTARIO, Canada, L0M 1J0.

The transverter must be linear as the modem has amplitude variations as part

of the converyed modulation, which must be reproduced by the transverter.

Usually any transverter capable of SSB operation is suitable if it switches

quickly enough.

We also sell a printed circuit board set that includes the State EPROM,

documentation and support diskette for $110.  Documentation sets are

available separately for $20 and we have a VHS (beta by special request)

Video tape featuring Dale Heatherington explaining the Beta modem design

for $20.

Doumentation sets are creditable towards the full parts kits or board sets

and the board sets are also creditable towards the full parts kits.

We pay U.S. shipping, however, we do require that the approximate difference

between domestic ground delivery and premium shipping be sent in advance.

Overseas second day air can cost up to $60 per kit!  The kits weigh about

2.5 pounds each, and we can put 3 to 4 kits in a single box to save on

shipping.

Please send orders (a letter is fine) and your check payable in U.S funds

to:

GRAPES, Inc.

PO Box 636

Griffin, GA  30224-0636

We're sorry but we cannot accept Purchase Orders nor Credit Cards.  Please

allow four to six weeks for delivery.

THANKS!!   Bob  KA4BYP,     email:  ka4byp@netcom.com

=========================================================================

[Additional information related to the GRAPES modem]

From: ka4byp@netcom.com (Bob Merritt)

Newsgroups: rec.radio.amateur.digital.misc

Subject: Re: 56k+ Packet System

Organization: NETCOM On-line Communication Services (408 261-4700 guest)

References: <9410040039.AA02497@enterprise.chinalake.navy.mil>

<1994Oct4.141109.27381@ke4zv.atl.ga.us> <36s91n$90c@hardees.rutgers.edu>

Tim Hayes (thayes@hardees.rutgers.edu) wrote:

: gary@ke4zv.atl.ga.us (Gary Coffman) writes:

: >Well then you need a GRAPES 56kb RF modem. It's $250, and with the

: >necessary transverter and digital interface, it's still under $600.

: >Most of the voice radios being used for 1200 baud packet cost nearly

: >that much. 46 times the throughput for about the same money is an

: >unbeatable deal.

: From what I've heard there isn't a ealisy available transverter that

works

: reliably with the GRAPES hardware.

: How much work is it to put one of these systems together? I'd be

intersted in

: running something like this, but its hardly what you could call

: "plug and play"...

Try Bob Morton of Maple Leaf Communications for transverters at

705-435-0689 or RR1, Everett, ON, Canada L0M 1J0.  GRAPES has bought a

couple from him and ALANET has bought several.  I believe he gets about

$279 each for his 8 watt units.  I also understand he's working on a 35

watt model.  The 8 watt units ought to get you 30-40 miles easily on

220MHz, maybe even 50 or 60 miles on a clear point to point link.

73/Bob ka4byp

--

*******

Bob Merritt  KA4BYP  ----->  ka4byp@netcom.com  <-----

------------------------------------------------------------------

Date: 5 Nov 94 21:17:00 GMT

From: steve.diggs@totrbbs.atl.ga.us (Steve Diggs)

Subject: GRAPES (56Kbps) setups

-> We'd like to know is what people who actually have GRAPES modems

-> have.  Since the distance between us is 6.5 to 10 miles, I'd

-> particularly like experiences from people who actually use GRAPES

-> over this distance.

We make extensive use of the GRAPES modem for our backbone in North

Georgia. We use a less powerful transverter to go <60> miles, so I would

think that the equivalent of two Dixie cups and some string would do

your job. <grin> We are about to try the Down East Micro unit for the

first time ourselves, but we expect acceptable performance from them.

Regards,

Steve Diggs, President

East Atlanta LAN

----

Top Of The Rock BBS - Lilburn, GA         SYSOP: Steve Diggs

UUCP: totrbbs.atl.ga.us               Snailmail: 4181 Wash Lee Ct.

Phone: +1 404 921 8687                           Lilburn, GA 30247-7407

------------------------------------------------------------------------

In article <39kprg$uh@qualcomm.com>

karn@unix.ka9q.ampr.org (Phil Karn) writes:

> In article <39f1ht$8vv@news.cc.utah.edu>, val@cs.weber.edu (Val Kartchner) writes:

> |> We've been looking at the GRAPES modems.  Since the output is on 10MHz, we

> |> need a transverter.  We've got the catalog from Down East Microwave, and

>

> I assume you mean 10m (28 Mhz)? That's where mine are.

>

> I have several of these units, although they've been on the shelf since I

> moved from New Jersey to California over 3 years ago. Inertia and lack

> of time...

>

> Anyway, in New Jersey I ran them heavily on 220.55 Mhz until we lost

> that part of the band. I used Microwave Modules transverters, which

> were about the only units available at the time.  They worked

> reasonably well once you tune them up -- assuming no QRM. The receiver

> is an FM discriminator, and because of its wide bandwidth (100 Khz vs

> 20 Khz for ordinary NBFM) you will need proportionately more (7dB)

> power and/or antenna gain over that required for ordinary NBFM over

> the same path.

>

> I recommend testing the path with a pair of HTs. If you can make it

> reasonably well with one watt, then you should have at least 5 watts

> on your GRAPES modem, and so forth.

>

> These units are somewhat QRM sensitive.  Part of the problem is their

> "stealthy" nature. A GRAPES modem cannot be heard at all on a NBFM

> receiver (though it may move an S-meter) because it looks like

> broadband noise and will not open a conventional noise squelch. So

> when everything suddenly stops you may find that some FM simplex users

> have camped out on your channel.

>

> Around here (San Diego) the GRAPES users on 70cm are plagued by Navy

> radar. I've had a project going for some time to add forward error

> correction that should deal with this problem. It may even lessen

> transmitter power requirements. Some day...

>

> GRAPES modems *can* be heard as bursts of noise on an AM or SSB

> receiver, or on a wideband FM receiver (e.g., Icom R-7000), and these

> are useful monitors when setting up your links.

>

> I strongly recommend a transverter with a built-in PIN diode

> switch. Packets at 56kb/s can be pretty short, and you don't want a

> slow T/R relay stretching things out. This can be a problem if you

> need a lot of transmit power (to get you that extra 7dB). It's better

> to spend the money on antenna gain and try to stay within the

> capability of your transverter rather than add an external amplifier,

> most of which use T/R relays. And this will help keep RF out of your

> monitor, stereo, TV, etc.

>

> Phil

------------------------------------------------------------------------