NETWORKS, NODES, AND SWITCHES

X1J, PI2, ROSE, NetROM, TNC-95, TCP/IP, HAPN-2, etc.

Also Related PacComm Products

Miscellaneous Info Collected by WÝTN,

Rev. 9, 1-3-95

MISCELLANEOUS

Hi N4ULS and others. You can pretty much do the wide area stuff with either

ROSE or NETROM equivalently. I see no real advantage for ROSE, over a properly run NETROM network. I personally do not like the ROSE user interface. But that may just because i was exposed to NETROM first.

NETROM networks typically do not achieve wide area success. This has nothing

to do with the features in Netrom. Everything needed to do a successful job is

there. The success is usually blocked by personal ego's and lack of cooperation

or capability to educate.

ROSE has a phsycological advantage as i see it. Being technically more meager,

and usually understood by one "Guru" in an area, the cooperation is automatic.

Either the "Guru" is allowed to do his thing, or u can t even get between

neighbor nodes. So, the owners tend to let the "Guru" do all the operating, and

the cooperation between nodes is then automatic. Also, since there are not

"Tom, Dick and Bozo" neighboring ROSE system owners, there is no politics ex-

erted on the "Guru" to make things work to Bozo's system, when Bozo hasn t

got the faintest idea what s going on, and wont let anybody fix it for them,

on an ongoing basis.

Now, you may wonder why i m bringing this stuff up. My experience is that

u can do the wide area job technically, easily, with ROSE, IP, NETROM, FLEXNET,

and probably a few other Level 3/4 systems. It don t matter what u pick, only

thing that matters is if the hams in the comunity of the area to be covered will

let u get on with the endevour.  These things are all pretty much equivalent

in what u can achieve with them. The problems in Ham Radio Packet Networking are cooperation, community, system design and site procurement.

I personally like IP because of its integration of applications. This gives

it a future. Oh, yea, the IP addressing scheme is the most versatile, too.

IP Routing is kinda up in the air at the moment, but in the long term that

will shake out. Discussion here have been where do we automate? Below the

horizon or above, or both and if both, together or split? For the time being

split, and fixed can work, with cooperation. And locally the cooperation

is not happening at the moment.

73, Don wb9mjn%wb9mjn.ampr.org@hamgate.cc.wayne.edu

------------------------------------------------------------------------------------

>Subject: [NETSIG:408] Re: Looking for information.

>

>

> Hi N4ULS and others. You can pretty much do the wide area stuff with either

>ROSE or NETROM equivalently. I see no real advantage for ROSE, over a properly

>run NETROM network. I personally do not like the ROSE user interface. But that

>may just because i was exposed to NETROM first.

>

Due to many requests I am working on a NODE style interface for ROSE, it

has been delayed a bit due to a greater demand for UI support for IP,

which I am also working on.

> NETROM networks typically do not achieve wide area success. This has nothing

>to do with the features in Netrom. Everything needed to do a successful job is

>there. The success is usually blocked by personal ego's and lack of cooperation

>or capability to educate.

I have been told by some NetRom network managers that due to the way nodes

are propagated through the network there is a limit on the size a single

network can grow, since the node names fall off the edge. Is this true?

>

>73, Don wb9mjn%wb9mjn.ampr.org@hamgate.cc.wayne.ed

--------------------------------------------------------------------------------------

WB9MJN said:

>You can pretty much do the wide area stuff with either ROSE or NETROM

>equivalently.

I think that if you will observe some of the large TheNet/NetRom networks

such as the one in New England, you will find that though you may be able

to cover the same distance, you must do a good part of that manually.

I believe that NEDA only allows nodes to propagate 3 deep.

>From my vantage point here in Florida I can connect to other users on

the ROSE network that we are interconnected to, some of these users are

as much as 40 or more switches distant from me, and I only issue ONE

connect request.

>I see no real advantage for ROSE, over a properly run NETROM network.

I suppose you do not since you probably have not dealt with a large ROSE

network. But from where I set here in Tampa Florida I can use a large

NetRom network, up in Minnesota, and a large ROSE network, TX, OK, AR,

LA, FL and East NM. I find the difference is like night and day.

I can not find any real INFO on the MN network, I am told the get a

nodes list... I get something that looks like a crypto sheet, and give

up after trying to decode it.

But, I can connect to say INFO at say 214411 and I get back a pretty good

descripton of what I can access in the 214 area over around Dallas. 817411

gives me simular data on that area. We also use 555 for wide area INFO,

and it has become standard over most of the ROSE type networks. Makes

network trolling very easy.

NetRom looks like a series of old step by step exchanges with manual

routing between them, ROSE looks more like a nice clean network that

lets me get to where I want to.

Another advantage being that ROSE is PID transparent, it can transport

IP, NetRom or any other protocol (inside AX.25 frames) transparently.

As a counter example, in order to transport IP over NetRom you first

must fool the NetRom into thinking you are sending it something with

either a F0 PID or a NetRom PID. Some people have done this by writing

drivers (NOS as a example) in order to send IP over the network, that

is a little like having to go out and learn some strange language in

order to user your telephone, because the TELEPHONE can not carry

English or Spanish or whatever language you are going to use.

The NETWORK should be transparent, when it is not it limits what others

can do. So I do not think that NetRom has any advantage here.

>I personally do not like the ROSE user interface. But that may just

because i was exposed to NETROM first.

To each his own, I personally see no need to have to connect to SOMETHING

before I connect to my target, I am trying to connect THROUGH a network TO

my target. But again, to each his own.

>NETROM networks typically do not achieve wide area success... [cut]..

I think it has much to do with the design of NetRom, it was designed for

small networks, and a large network totally taxes it's operation unless

you limit the propagation of nodes data, and thereby fall back to the

manual routing...

> ROSE has a phsycological advantage as i see it. Being technically more

meager...

This one is interesting, I am not sure what you mean by "meager" if you

mean that the level 3 overhead is lower (3-5 bytes/level 3 frame as

compared to 20 bytes/level 3 frame for NetRom) then indeed it is much

more meager, but with rf paths being mostly "thin" and slow why load them

up with all that overhead, we need the bandwidth for other applications

that are more fun to play with than NetRom overhead.

As far as tools to work with, there are tools being developed right now,

There is a Network Manager program written by NX2P which will allow the

network sysops to build and add to the network as it grows. There is also

a tool that allows network sysops to go inside a switch and change parameters

on the switch, it does it neatly and without having to deal with cryptic

rows of numbers ect.

There are others that are presently being developed. There is even a PC

based version of ROSE that has been developed by a fellow switch op here

in Florida. So meager in terms of tools I think is not quite the description.

>Either the "Guru" is allowed to do his thing, or u can t even get between

neighbor nodes.

Here in Florida we are trying to educate others help sysop the switches,

finally we are beginning to get some takes, guru NOT, we would like to

have a lot of sysops to help run the network makes it a lot easier to

go off on a trip or not worry if something happens at the switch site.

Most users are not interested in sysoping, all they are interested in

is a operational network.

ROSE does make the sysops work together and that is very important, we

have to PLAN the network, and a planned network is a much better plan

than a hodgepodge "me too" network.

>The problems in Ham Radio Packet Networking are cooperation, community,

system design and site procurement.

This is a very true statement, and we have found the same problems here

in Florida as in other areas. But I have found that by being forced to

work as a group and not as independent "node ops" we are able to achieve

more lasting and reliable network parts.

>  I personally like IP because of its integration of applications.

I too like IP, but I believe that IP was designed to link machines and

the machine applications, not to be the whole network. Look at the dis-

cussion going on on the TCP list right now about that very idea.

ROSE addressing is based on X.121 addressing and that addressing scheme

was designed to network a whole planet, and allow such systems as IP

to flow through it and do what they do best.

ROSE meets many of the problems that have been voiced on the TCP list

as to handling the AX.25 links. And the author Tom Moulton is working

to make ROSE operate even more transparently right now.

I personally believe that a network built using both is a very effective

way to deal with the problems that both systems have, and the combination

of the two gives a very powerful set of communications tools that we can

use to make our RF networks operate most effectively.

I have tried NetRom and found it wanting, it does work and works well for

small networks, but when you want a large network you need a addressing

system that can deal with a network of any size, you need a routing system

that too can handle whatever size of network you design. And you need a

network protocol that has low overhead and at the same time is robust and

will carry your data across the network with a minimal impact on bandwidth.

Why burden your already low bandwidth RF paths with more and more overhead

when you do not need it??

I am sorry, if I sound biased but I suppose I am, I have tried NetRom and

even tried to operate a network based on it many years ago, and due to

user disgruntlment changed back to ROSE. I found it wanting, and today

still find it to be so.

------------------------------------------------------------------------

Chuck Hast

No Dumb Operating System here, only OS/2. Soon to add LINUX.

Delphi:   wchast@delphi.com

AX25 BBS: kp4djt@kp4djt.tpa.fl.usa.noam

ampr.org: kp4djt.ampr.org

AX.121:   3100813077

------------------------------------------------------------------------

Hi Tom, and others. Yes, because NETROM has an autorouter which functions

in a determinstic manner, the range of a NETROM network is less. There are

ways to operate around this. If ur users will cooperate and not put up per-

sonal Nodes, this lessens the memory requirements, increasing range. Of course

with ROSE there are few, if any personal nodes, so this point is mute with

ROSE, but is this because just because NETROM is much more available, and has

an autorouter while ROSE is much less available, and does not have an autorouter.

Larger memory in the NETROM nodes will also aleviate the practical effects

of this problem, too. NETROM has the "slime trail" feature as well. And before

there was a PC version of NETROM this feature was generally enabled, and worked

well. But with the uncooperative technigue of using the "slime trail" as a way

to hide on a backbone one is not coordinated on, "slime trail" gets turned off.

Again, not a technical fault, but a cooperation problem in the ham packet

community. The flaw here is that autorouter's determine routing to specific

nodes, and thus memory required for a route to all those specific nodes out

there gets out of hand fast. NETROM's programmer's did figure this out, how-

ever, and put in the "slime trail", but Ham's corupted its usage, and made

it unusable. This is really no different an effect, than a ROSE operator who

does not do the routing tasks that are required, for whatever reason, since

ROSE does not have an autorouter. The cause's are different, but the effects

are the same.

The impression i get from the pro-ROSE comments I find somewhat nieve of

the operating tweaks NETROM can be put thru. The reason everybody just doesn t

jump over to ROSE, is because the people that are using NETROM successfully

, do not see any technical advantage to ROSE, and the ROSE commentors just

don t understand how NETROM is used successfully. Where ROSE has succeeded

is pretty much a map of the areas where understanding of how to make NETROM work was non-existant. Not to say that all the places using NETROM know what they are doing with NETROM, however.

ROSE is nice, its just not a socalled "killer" solution. TCPIP, less its

router developement hole, is just so much moreso a "killer" solution over

both NETROM and ROSE. And the versatility of IP addressing fills the router

developement hole, for the meantime.

73, Don. wb9mjn%wb9mjn.ampr.org@hamgate.cc.wayne.edu

--------------------------------------------------------------------------------------

Don,

The autorouter in Netrom is nice for small networks, but for a Network that

covers hundreds of miles the level 4 timers will need to be set quite long

to insure that the entry node does not retry transmissions too often.

This has a significant impact on performance.

ROSE has no level 4, besides level 4 and above belongs in the End User's

Terminal.

Larger memory may reduce the need to drop nodes off the list, but it would

not have any impact on the required level 4 timers.

I would like to help people understand the technical advantages of ROSE.

It was designed to be a National network, not one to connect NOCAL and SOCAL.

TCP/IP is *NOT* a stand alone "killer" solution. Phil's has stated that it

needs a fully connected ax.25 network layer. (i can get the exact quote)

ROSE is currently working on fixing the routing hole.

73, tom w2vy

-------------------------------------------------------------------------------------

> I would like to help people understand the technical advantages of ROSE.

> It was designed to be a National network, not one to connect NOCAL and SOCAL.

>

> TCP/IP is *NOT* a stand alone "killer" solution. Phil's has stated that it

> needs a fully connected ax.25 network layer. (i can get the exact quote)

The gist of it was that there is a need for a network sublayer between the

ax.25 link layer and IP.  This sublayer would use a dynamic routing protocol

to pull together the rf fragments and provide the illusion of fully connected

subnetworks to the IP layer.

> ROSE is currently working on fixing the routing hole.

I'm not convinced that an adaptation of existing routing protcols at the

IP layer can't do the job.  We've had pretty good success with RIP on our

rf network here.  We have our datagram-based regional subnet spread across

about a dozen different frequencies, and yet IP routing still works.

Interposing another networking sublayer under IP would be superfluous in

this case.

OTOH, Phil's solution might be cleaner (more universal), as long as it

doesn't add a bunch more overhead, and it doesn't take another 10 years

to arrive.

So tell us more about how (and when) ROSE will fix 'the routing hole'...

Let's face it: the *real* problem in amateur packet networking is that

amateurs continue to try to use underpowered hardware (slow modems,

radios with lousy turnaround time, TNC-based routers, etc), and poorly

engineered rf network nodes (hidden transmitters, intermod, etc).  No

amount of software cleverness can work around these fundamental problems.

Unfortunately, the knowledge and expertise (and, yes, the money) needed

to do it right are scarce commodities these days.

> 73, tom w2vy

Barry

--

Barry McLarnon  VE3JF/VA3TCP   |  Internet: bm@hydra.carleton.ca

Ottawa Amateur Radio Club      |  AMPRnet:  barry@va3tcp.ampr.org

Packet Working Group           |  FreeNet:  aa187@freenet.carleton.ca

--------------------------------------------------------------------------------------

[Note: next message not a thread to message(s) above, rather to material.]

>You seem to be suggesting that the network be designed to facilitate the

>most mundane of applications: keyboard-to-keyboard chat between two users

>running the equivalent of dumb terminals.  That's just not where it's at

>anymore.

>

>Barry

But, any network, no matter how advanced, should always support the simplest

hardware configuration.  This is necessary for some of the most basic "work"

we do as hams.  I don't want a network that locks out the "keyboard to

keyboard chat between two users running...dumb terminals".  I don't want a

network that will make it impossible for the user with a Model 100/TNC/radio

portable packet station to respond to the local hospital during a drill or an

actual emergency and set up a packet link to the county EOC.  To me, that's

one of the real neat things about packet.  If I feel like it, I can connect

my small TNC to an HP calculator and have a fully functional packet station,

provided I have a convenient entry point into the network.

TCP/IP has tremendous potential and will bring a lot of "nifty" applications

to the packet users. But let's not lock out the basic users in the process.

Let's not force them to find some local TCP/IP system running a gateway just

so they can get into the network. The TNC-Radio Node/Switch, regardless of the

networking technology used, is easy to build, easy to place at a typical radio

site, and easy to equip with standby power that can run the system for days or

even weeks. During a real emergency, do we want to rely on complicated

gateways running on PC's, many of which will rely on commercial power for

their operation?  Do we want to rely on the users operating these gateways,

who may become bored, fed up, divorced, relocated, etc., thereby changing the

topography of the network, sometimes with little or no warning?

What I need is a network that meets the requirements of the TCP/IP users AND

the basic, "dumb terminal" users.  From what I've see so far, a full blown

dedicated TCP/IP network falls short of this need, since the user must either

run a full blown TCP/IP station or be close enough to a gateway to get

access.  And who knows how he establishes a "keyboard to keyboard" session to

his buddy with a dumb terminal on the other side of town once he finds a

gateway he can use.

I think the answer might be some combination of a bit regenerator high-speed

packet repeater linked to a node/switch.  All users within range of the

repeater hear each other without causing hidden transmitter problems. The bit

regenerator is a dumb device, it just retransmits whatever it hears on the

input.  This is great for TCP/IP and "other" users.  The stations within range

of the bit regen repeater become a "lan".  No NET/ROM, ROSE or KA-NODES would

be needed (or wanted) on the repeater.

Attached to the repeater, and monitoring, would be a node or switch linking

it to a wide area backbone, with other bit regen repeaters serving other LANS.

So, if a user needs to establish a link outside of the LAN, they issue the

appropriate command to the node/switch via the repeater.  The node/switch

establishes the connection to the other end. If the appropriate networking

technology is used, "connections" by "basic" users, and TCP/IP frames could

both be supported.

Food for thought...

-----------------------------------------------------------------------------------------

The discussion of the past few days has been excellent.  Lots of

ideas brought up, problems noted and questions raised.

Here's some thoughts I had:

I disagree with Hank's statement #3 - that we should never use non-amateur

facilities.  There's just too much good stuff out there.  If we artificially

isolate ourselves from it we're the ones who will miss out.

But I agree whole-heartedly with Hank's belief behind the statement.  We

need to continue to work on and invest in our own network.  To get

too comfortable using the "others" will only lead to stagnation

of our system.

This discussion has shown me some of the areas where work and innovation

can still be used to make some big steps in the digital communications

arena.

We need digital regenerating repeater satellites in orbit...  A couple

high orbit 56K channels would do a lot to tie our local nets together.

And speaking of satellites - we should be working with our fellow

communications experimenters in the FIDO network.  With the "Planet

Connect" style of bulletin forwarding it would be trivial for the Amateur

Bulletins to be forwarded across the country and to the FIDO bbs's.  And

then fed to the local packet systems.  Blasphemous?  Maybe.  But it's

an alternate communications system that is available and not being used.

It would be better than relying on the landline..

And what about HF?  We have the frequencies.  Let's use them.  Only

lets use them better than we have been.  Two ideas - is there some way

to "capture" bulletins being broadcast between other systems.  Something

like that would work ideal for me in Montana where I have the equipment

but not the "connections" into the forwarding system.  Besides - having

to resend a bulletin time after time is a terrible use of resouces.  The

satellites have this technology - why aren't we using it on HF.  A few

super-stations should be able to easily and efficiently feed most of the

country.

The second idea - let's get some better digital links between local networks

going on HF.  It seems to me that given some revolutionary new digital

concepts there's no reason we couldn't have a nationwide network that allowed

real time connections anywhere at least during certain times of the day.

There are a couple hot HF packet frequencies with lots of gateways - but

their usefulness is almost null due to the inappropriate nature of standard

CSMA AX25 technology.  There must be a way to do it right - let's find it

and implement it!

--

Bill

Reply-To:       vodall@bigsky.dillon.mt.us

Amateur Packet: wa7nwp@n7see.mt.usa.noam

---------------------------------------------------------------------------------------

> >No doubt, but it doesn't have to be that way.  We're talking about the

> >future network, not the status quo, right?

> >

> My point is that the local net should not be required to have an IP guru

> to gain access to the long haul network.

Better he should need a ROSE guru?  Seriously though, the need to have

guru status in order to set up an IP router tends to be greatly exaggerated.

Anyone running NOS is potentially a provider of gateway services for plain

vanilla ax.25 users, and I know lots of people who are successfully running

NOS and are far from being gurus.

> >> It is interesting that every time TCP/IP is mentioned the concept of

> >> 'wondrous applications' pops up.

> >

> >Yes, and with good reason.  Without interesting applications, what's the

> >point of having a network?

> >

> The point of having a network is Communications. and the idea of an

> application is not linked to TCP/IP.

Like it or not, most of the new networking applications that people want to

use nowadays run on top of TCP/IP.  Maybe not out in the hinterlands of

packet yet, but give 'em time...

> >> The primary feature of a backbone should be Communication.

> >

> >Absolutely.  I don't recall saying otherwise.  And backbone nodes should

> >not have a bunch of Level 4-7 cruft, they should be routers.

> >

>

> That's right the level 4 and above, if used, is up to the user, not the

> network to select. If we have a Level 3 network then it IS left up to the

> user to select the higher layer protocols. If the netwotk is IP then we

> have forced their hand.

But IP *is* a Level 3 network!  Users have a choice: they can use TCP, UDP,

etc... they can even run ax.25 on top of IP if they're perverted... :-)

And the ax.25 Level 2 is still there for those who prefer to run in 'dumb

terminal' mode and let the applications run on other systems...

> >Heh... no shock at all.  So what's your point?

> >

> An IP backbone will force people to run IP, whereas the features that are

> in ROSE will allow any existing or new protocol to be used, let's keep

> the system open for all to use.

I said it before and I'll say it again: an IP backbone does not force

people to "run IP".

> >I'm not telling anyone what they should do or want.  What I do know is a lot

> >of people, newcomers and oldtimers alike, whose interest in packet has been

> >sparked (or rekindled) by those very applications.  Perhaps you can explain

> >why you're so antagonistic towards them...

>

> Great the people who want to use IP should be able to, but the flip side is

> that if some don't want to they should not be forced to.

I don't use force... I prefer to show by example something which I think

is better.  The Ottawa area network is strongly IP-oriented, but there are

still lots of people using plain old ax.25.  I don't think they feel

pressured to 'run IP' (but many want to after seeing it demo'ed)... nor

or they left out in the cold if they don't.  Should I gather some

testimonials? :-)

> >Sigh... you're not paying attention.  The operative word in the above is

> >'facilitate'.  I want a network which is optimized for what I consider to

> >be (IMHO, of course) the most interesting applications.  That doesn't

>

> How about a network that can support ALL modes of communication?

>

> >mean that it precludes doing all that fun stuff that we did back in the

> >80's, but I think keyboard-keyboard chat and the like are steadily

> >diminishing interests... why cater to them?

> >

>

> It seems that when talking to you you want to convert the world to IP, for

> their own good or because they really want to use those killer apps.

>

> My view of the network is it should be a efficent as possible, add the least

> overhead and allow all modes of packet activity. I am going to acheive that

> in ROSE by providing a fully transparent system that accepts all protocols

> that can be supported within the confines of AX.25 with addressing and

> routing that can support enough switches to provide a nation wide network.

And my view is that we'd better base the network on something better than

10-year-old TNC technology, or it won't amount to much.  IP is no less

"open" than ROSE, but it does provide a more powerful and widely-supported

base for applications, which are the only reason for a network to exist.

> Barry, unless you have some very specific questions that I can answer I see

> no point in wasting everyone's time reading this dialogue.

Finally, something we agree on. :-)

> If nothing else I am sure that we agree that are views are different enough

> that we will not agree to much else on this topic.

I hope we provided some entertainment value, anyway.

> THE END

Hmmm... guess I've been KISS'ed off. [For the humor-impaired: that's a joke]

> vy 73,

>

> _________________________

> Thomas A. Moulton, W2VY

> w2vy@ram.net

73 to y'all too...

Barry

--

Barry McLarnon  VE3JF/VA3TCP   |  Internet: bm@hydra.carleton.ca

Ottawa Amateur Radio Club      |  AMPRnet:  barry@va3tcp.ampr.org

Packet Working Group           |  FreeNet:  aa187@freenet.carleton.ca

-----------------------------------------------------------------------------------------

>I said it before and I'll say it again: an IP backbone does not force

>people to "run IP".

But it does force the backbone to carry all the overhead of IP...

A network system based on ROSE or something similar in concept doesn't force

a bandwidth-limited network (at least until 1-100 Mbps links are commonplace) to carry all the excess baggage. If you want the extra IP functions, then put an IP

router at each end of the pipe. xNOS can be easily configured to use ROSE as

the pipe between stations (or neighboring LANs).

Mike

------------------------------------------------------------------------------------------

All this discussion about TCP/IP and ROSE is fun,  and they are both

good and I'd rather see one or both of them in place than something

like NETROM,  but there is a reason why many are not going to ROSE

right now,  and that's hardware.

I think ROSE is good at what it does, but I don't know enough about it

(there are no ROSE nodes around here, we're all TCP/IP based) and right

now people are putting up PC's at sites and  and P10 switches due to

the power.  Can ROSE run on something other than an underpowered TNC to

handle something like Barry's 56k packet network? They run PI boards at

56k and many other people are too.  It's nice to also put up a PC with a

single IP address and have multiple ports using external tnc's, DRSI

cards, PI boards, PacketTwin cards, etc...   Just curious about ROSE

on this?  Many people are running high speed with large packet sizes.

I run pretty small ones at only 512 bytes or 864 bytes on 9600, but if

I had higher speeds up I'd be running the usual 1460 byte packets that

others are running.

Like I said, I don't know enough about ROSE to know if it can handle

this right now,  but this is normal in many places n packet.  Many if

it was PC based and you can stick one address on it and put on all the

ports you want using whatever hardware you want and handle very large

packets at very large speeds it'll be considered more. Trying to convince

Barry that he needs it may not be a good idea since it probably can't

do what he's doing right now.

For those people that dont' have money to put up PC's at sites and all

they have is a couple of tnc's for a switch, personally I'd recommend

ROSE to them over NETROM anyday, but for anything more powerful it

usually ends up being a PC running TCP/IP.

--

Ron Atkinson  N8FOW

AMPRnet  : n8fow@n8fow.ampr.org

Internet : ron@chaos.eng.wayne.edu

aa011@detroit.freenet.org

-----------------------------------------------------------------------------------------

At 11:06 AM 16/12/94 -0600, <mwhittin@pen.k12.va.us> wrote:

> I hear a lot of people saying that keyboard to keyboard packet

> is obsolete.  If that's so, why do we even need to establish a

> real-time network accross the nation, or accross the world?

Keyboard-to-Keyboard is such a small percentage of packet activity that it

might as well be obsolete.  A Real-Time network has many more users than

being just for keyboard-to-keyboard type chats. World Wide Web, Gopher,

station control, Dx Clusters, and so on all benefit from Real-Time

connectivity.  There is no paradox between saying Keyboard chats are

obsolete and wanting a real connectivity.  It's just hilighting the fact

that times change.

Carl.

--

Carl Makin (VK1KCM)          "Speaking for myself only!"

makinc@hhcs.gov.au      'Work +61 6 289 8443'      Canberra, Australia

'The entire concept of daylight savings time is like trying to make

yourself taller by cutting off your head and standing on it.' - Usenet Oracle

-----------------------------------------------------------------------------------------

According to Carl Makin:

> ROSE, Netrom (and from what I know of them Texnet and Flexnet) seems aimed

> at Joe BBSUser.  Any other usage is an add-on, an afterthought.

> Functionally we're limiting ourselves to the dictates of a simple protocol

> running on limited 10 year old hardware and the expectations of people who

> still think morse is a neat idea.  I think we can do better than that.

>

If you don't make a network available for the unwash masses then in most

cases you can;t get the support to afofrd to maintain the system.  That is

with systems that depend on local user support in order to provide nodes and

operations and growth.

The key is to make the network operate in such a manner as to allow the

high-end user (1%-2%) to keep having fun.  Many of the current networks, as

you mentioned, have been out grown by the 1% of networkers and high-end users

that want to continue to experiment.  This is why there are so many

developmental, small networks around now.

Greg

-----------------------------------------------------------------------------

Instr Technology / Dept of Curriculum and Instr / Univ of Texas / Austin

-----------------------------------------------------------------------------

Work:  (512) 471-5211   | Packet Radio BBS: WD5IVD @ N5LDD.#NTX.TX.USA.NA

Fax :  (512) 794-5888   | InterNet: gjones@tenet.edu

-----------------------------------------------------------------------------------------

According to Greg Jones:

>According to Carl Makin:

>> ROSE, Netrom (and from what I know of them Texnet and Flexnet) seems aimed

>> at Joe BBSUser.  Any other usage is an add-on, an afterthought.

>> Functionally we're limiting ourselves to the dictates of a simple protocol

>> running on limited 10 year old hardware and the expectations of people who

>> still think morse is a neat idea.  I think we can do better than that.

>>

>

>If you don't make a network available for the unwash masses then in most

>cases you can;t get the support to afofrd to maintain the system.  That is

>with systems that depend on local user support in order to provide nodes and

>operations and growth.

>

>The key is to make the network operate in such a manner as to allow the

>high-end user (1%-2%) to keep having fun.  Many of the current networks, as

>you mentioned, have been out grown by the 1% of networkers and high-end users

>that want to continue to experiment.  This is why there are so many

>developmental, small networks around now.

>

Allowing the high end user to have fun is chasing the bleeding edge of

technology, I'm afraid.  In my limited packet experience, I find that the

high end users are rarely found building anything outside of their own

shacks.  Until more people are ready to fund and support hardware

that's located beyond their own property lines, I doubt we'll see anything

beyond what's already deployed.

I take umbrage to the comments I read about the 'unwashed' who are

also cast aside with 'JoeBBS' and 'keyboarders'.  Those same packeteers

are the folks I call USERS, for whom a network exists.  I, for one, don't

like pursuing technology unless I can stay aligned with my customers and

offer improvements THEY want.

--

*--------------------*----------------------------------------*

|  John Zantek WX3K  | Internet: jzantek@tcpet.uscg.mil       |

|    [44.4.40.104]   |   Packet: WX3K@WX3K.#NOCAL.CA.USA.NOAM |

|  Data 707 765 7016 |  Amprnet: wx3k@w2air.ampr.org          |

*--------------------*----------------------------------------*

|       (WWW Home Page) http://wx3k.ampr.org/index.htm        |

*-------------------------------------------------------------*

----------------------------------------------------------------------------------------

> >> ROSE, Netrom (and from what I know of them Texnet and Flexnet) seems aimed

> >> at Joe BBSUser.  Any other usage is an add-on, an afterthought.

> >> Functionally we're limiting ourselves to the dictates of a simple protocol

> >> running on limited 10 year old hardware and the expectations of people who

> >> still think morse is a neat idea.  I think we can do better than that.

> >>

> >

> >If you don't make a network available for the unwash masses then in most

> >cases you can;t get the support to afofrd to maintain the system.  That is

> >with systems that depend on local user support in order to provide nodes and

> >operations and growth.

> >

> >The key is to make the network operate in such a manner as to allow the

> >high-end user (1%-2%) to keep having fun.  Many of the current networks, as

> >you mentioned, have been out grown by the 1% of networkers and high-end users

> >that want to continue to experiment.  This is why there are so many

> >developmental, small networks around now.

> >

>

> Allowing the high end user to have fun is chasing the bleeding edge of

> technology, I'm afraid.  In my limited packet experience, I find that the

> high end users are rarely found building anything outside of their own

> shacks.  Until more people are ready to fund and support hardware

> that's located beyond their own property lines, I doubt we'll see anything

> beyond what's already deployed.

Time for a counterexample.  In this area, the so-called 'high-end' users

have funded, built and maintained most of the local network facilities.

Besides which, people's shacks can be an excellent place to put network

nodes.  Around here, it works like this: a user goes 'high-end' and

upgrades to 56kb/s, and he then uses his old low-speed equipment to put

an network access port up for users in his local area, or in some cases

to run a backbone link to an outlying area.  The high-end users have

their 56kb to play with, but other users around them get a piece of the

action, gratis.  The high-end folks are the ones running most of the

bbs's and other services, too.  This is basically a cellular system, with

a high-speed central full-duplex repeater to tie the cells together... you

also need some lower-speed repeaters to fill in the coverage gaps between

the cells.  The network nodes then end up in locations where they are

readily accessible and easily maintained, and can run on more powerful

hardware than TNCs.  It works here... why not elsewhere?

> I take umbrage to the comments I read about the 'unwashed' who are

> also cast aside with 'JoeBBS' and 'keyboarders'.  Those same packeteers

> are the folks I call USERS, for whom a network exists.  I, for one, don't

> like pursuing technology unless I can stay aligned with my customers and

> offer improvements THEY want.

But as one of the network builders, you also have to provide some

leadership.  You can stay mired in 10-year-old technology and applications,

or you can show the users something new, and sit back and see what happens.

I get tons of questions from local users, and that provides a pretty good

reading of where their interests lie... the vast majority are about

Internet mail, Internet gateway access, running NOS, running Linux,

using winsock apps over the air, upgrading to higher speed, using the

convers servers, etc... very few are about keyboard-keyboard 'qsos', pbbs

use, node hopping, and the other stuff we all used to do back in the 80's

(and I do still run a pbbs, and am readily accessible to the 'unwashed' :-).

I'm not trying to hold the Ottawa network up as a shining example, but I

think our experience provides some insight into what the users in your

area *will* be looking for, if they aren't already...

>    *--------------------*----------------------------------------*

>    |  John Zantek WX3K  | Internet: jzantek@tcpet.uscg.mil       |

>    *-------------------------------------------------------------*

Barry

--------------------------------------------------------------------------

As the originator of the now-infamous "Looking for information" thread, I want

to thank everyone who took the time to respond both here and to my personal

email address.  This has been a fascinating thread to follow and I appreciate

all of the input given.

I wish to make a few observations based on the results of my question and the

results of informal discussions with local users and system operators, and I

invite further discussion.

Many feel that TCP/IP networks are the future for amateur packet radio

networks.  I am intrigued about the possibilities of TCP/IP, but I'm not

convinced that the basic user network should be exclusively TCP/IP.  I feel

strongly that the basic network should always support users running the most

simple of hardware configurations.  This is necessary to continue supporting

some of the most basic "work" we do as hams.  I don't want a network that

locks out the "keyboard to keyboard chat between two users running...dumb

terminals".  I don't want a network that will make it impossible for the user

with a Model 100/TNC/radio portable packet station to respond to the local

hospital during a drill or an actual emergency and set up a packet link to the

county EOC.  If I want to run mobile packet for an event or just for grins,

around home or in another area, I don't want to hassle with finding out where

a gateway is, or what I should to my TCP/IP address when I travel to another

area.  To me, these are some of the real neat things about packet. If I feel

like it, I can connect my small TNC to an HP calculator and a handheld radio

and have a fully functional packet station, provided I have a convenient entry

point into the network.

TCP/IP has tremendous potential and may bring a lot of "nifty" applications

to the packet users. But let's not lock out the basic users in the process.

Let's not force them to find some local TCP/IP system running a gateway just

so they can get into the network.  I know a lot of folks who use our

current network just to talk to TNC maildrops, sending messages back and

forth between friends.  I know that some will never want to do anything more

than that, ever.

The TNC-Radio Node/Switch, regardless of the networking technology used, is

easy to build, easy to place at a typical radio site, and easy to equip with

standby power that can run the system for days or even weeks. Granted, the

technology is 10 years old, but we've seen some new TNC developments with high

speed, advanced CPUs, more memory and multiple ports, etc. This should help

maintain and improve overall network performance.

During a real emergency, do we want to rely on complicated gateways running on

PC's, many of which will rely on commercial power for their operation?  Do we

want to rely on the users operating these gateways, who may become bored, fed

up, divorced, relocated, etc., thereby changing the topography of the network,

sometimes with little or no warning? What I need is a network that meets the

requirements of the TCP/IP users AND the basic, "dumb terminal" users.  From

what I've see so far, a full blown dedicated TCP/IP network falls short of

this need, since the user must either run a full blown TCP/IP station or be

close enough to a gateway to get access.  And who knows how he establishes a

"keyboard to keyboard" session to his buddy with a dumb terminal on the other

side of town once he finds a gateway he can use.

I think the answer might be some combination of a bit regenerator high-speed

packet repeater linked to a node/switch.  All users within range of the

repeater hear each other without causing hidden transmitter problems. The bit

regenerator is a dumb device, it just retransmits whatever it hears on the

input.  This is great for both the TCP/IP and "other" users.  The stations

within range of the bit regen repeater become a "lan".  No NET/ROM, ROSE or

KA-NODES would be needed (or wanted) on the repeater.

Attached directly to the repeater, and monitoring, would be a node or switch

linking it to a wide area backbone, with other bit regen repeaters serving

other LANS. So, if a user needs to establish a link outside of the LAN, they

issue the appropriate command to the node/switch via the repeater.  The

node/switch establishes the connection to the other end. If the appropriate

networking technology is used, "connections" by "basic" users, and TCP/IP

frames could both be supported.

Since ROSE is designed to be a "designed" and "managed" network from the

first day the first switch goes online, it is an attractive option for the

backbone switching technology.  From what I've heard, and seen firsthand,

it looks like a ROSE network would be MUCH easier to maintain than a network

based on TheNET/NETROM, etc.  The planned UI application sounds like a

reasonable accommodation for the TCP/IP crowd, allowing them to pass TCP/IP

traffic transparently through the network. The UI application will also allow

APRS stations to pump unproto posits into the network and have them pop out on

the local APRS net frequency.

Not to say I haven't heard a few horror stories about ROSE.  But, the

favorable comments far outweigh the unfavorable.  It seems that if ROSE is

put up in a well planned and well managed environment, it works well.  If it

is shoved down the users' throats, they hate it no matter how well it works.

Comments?

Jack

n4uls@ram.net

---------------------------------------------------------------------------

ROSE

Doug,

Here is the short form of what ROSE is:

ROSE X.25 Packet Switch

ROSE uses AX.25 for User access and X.25 Packet Layer on top of AX.25 for

internodal links.

The addressing used is X.121, which is a Global address space.

Each switch retains all configuration information in bbRAM, which includes

routing table information to route calls to the entire network.

It was designed to be a National/Global networking solution and a requirement

was to be fullt transparent to any AX.25 device.

The routing table information is not dymanically created, it can be done by

hand or with NETMGR, a windows based network design program.

It also has the ability to upload custon applications, I admit there is not

much documentation for writing applications...

more info  ftp to lantz.com

questions as always...

_________________________

Thomas A. Moulton, W2VY

w2vy@ram.net

--------------------------------------------------------------------------------------

This is in repley to the person who was asking for response from Rose sysops.

I lost the original request, sorry.

Having been running Rose here in Hawaii for some years, I find so far since

we started it has always proven to be far superior to Netrom or thenet or

x1j for networking. Let me add here that I have run all four and am constantly

watching out for what will work best for us. Under the concepts we use, Rose

fits in the best. Your Needs May Differ.

Here are our concepts of what we want from our network here.

Total control over how and where the packets are sent, link to link.

Per packet overhead reduced to the minimum, forget this 20-40 bytes per packet

overhead.

No broadcasting.

Allow any protocol to go thru the network without having to create seperate

networks that reduce overall reliability, which allows us to create useful

larger coverage links that can be used for transparent-to-the-user rerouting

around link failures during times of emergencies.

Allow us to put up immediate drop points that allow users to initiate outbound

connections but blocks inbound requests. This is highly useful for areas under

emergency commms that want to pass traffic outward and stop inquiries coming in

which can bring the comms to its knees with redundant requests. One assumes

that outward traffic is higher priority than inbound.

Help files at each switch to bring a new user up to date on everything or at

least where to go and how to get things on packet.  A user connects directly

to a switch and within one minute they get the help file if they do nothing.

Remember when you were a new user? Usually they can monitor what is going on

frequency, see that the switch callsign is most often used, will attempt a

connection to it. Once connected, by the time they figure out how to disconnect

or if they hit their return key, the help file is sent to them. Saves time on

answering repeated new user questions.

Allow us to make link changes in sites, callsigns, frequencies, without having

to update all the users and PBBS Sysops, because it is transparent to them.

Saves us huge amount of time in this area alone when we are trying to make

these changes. It used to be a nightmare. Imagine trying to tell 30-100 PBBS

users that their PBBS has moved. Now that is avoided by assigning a address to

the PBBS itself across the network concerned. In 1992-1994 we had to move our

PBBS 7 times. Each time we told no one, including other PBBS's that forward.

They could not tell that it had moved until we said so.

Does not care if the user is an expert or VIC-20 user. They can access

every thing equally.

Allow connection to any other kind of network or end PBBS without hassle.

Even, Yuck! a digipeater (The 80's are over, right?).

Allows promotion of distributed end user services to be spread out across the

network to maximize transport capability of information everywhere. Keeps

the concept of network building has its needs (Cold Cash), and end use services

with different support needs (Colder Cash).

Allows us to get better network coordination with other network coordinators.

I would say this is the best of all things because it makes for better packet

for everyone.

I could probably say more but will stop here and say, try it, you might be

surprised at what it could do for you, and we dont make use of all the

possibilities either.

We still have a x1j up, but we use it only for its conferencing capability.

If something new comes out, we try it out. Nothing we have tried so has proven

itself to be as easy as Rose, for our situation. Your Needs May Differ.

We are hoping the dxcluster group here will get word soon from their counter

parts elswhere that using the Rose networks actually speeds up their stuff

for their users. They are on the other end of our link and so do not see how

well it works from our end when we make a connction.

Lastly, try it out for yourself. I trust no numbers stated and concepts. I set

up some tncs with different protocols. Connect a user at each end and dump

10k-100k files thru it. Then I divide the file size by how long it took to get

there. This gives me a real world numbers on how well it works based on our

setup. Perhaps this is not considered to be proper thruput testing. But it

seems to me the best reality check I can do with what I have.

The bottom line, I cannot praise W2VY enough for all his work. Thank you.

Oh yes, a disclaimer. These are my opinions, and my impressions of how it

all works. I could be stating the concepts or the workings of Rose incorrectly,

and advise that you seek other Rose network sysop opinions too.

Chris/AH6GG

------------------------------------------------------------------------------------

We are running a pretty large ROSE network here in the Tampa Bay area.

We have had some problems with getting timers set properly as we upped

the speed of the network, but we have found that it was just a question

of setting these parameters properly for the higher speeds, and off it

goes quite well.

As to ROSE it's self, the way you set up and maintain a ROSE network does

DEMAND a cooperative relationship between network switchops, but this

relationship once established means you are going to have a much more

homogenous network, and there will be much better planning to it's growth.

The PID transparencey of ROSE means that it can carry the other networking

protocols across the ROSE network and allow NetRom, IP ect. to appear as

though they were talking device to device...

The addressing scheme means that users can issue a single command connect

and will once the connection is established find themselves connected to

the target call on the far side of the link, no need to pick their way

through the network, nor need to have maps of the network, all they need

are exit point addresses, and coverage...

------------------------------------------------------------------------

Chuck Hast

No Dumb Operating System here, only OS/2. Soon to add LINUX.

Delphi:   wchast@delphi.com

AX25 BBS: kp4djt@kp4djt.tpa.fl.usa.noam

ampr.org: kp4djt.ampr.org

AX.121:   3100813077

--------------------------------------------------------------------------------------

>While it would be ideal to make direct connects to a end destination 1000

>hops away, I've found that each node has propagation delay.  there seems

>to be a practical limit due to this (as well as circuit congestion) which

>would limit the size of ANY network, regardless of protocol.  Does anyone

>have information on this topic?

>

When I picked 1000 I was thinking of a network with 1000 switches.

The point you raise is a vaild one, but the limitations you are looking at

are modem speed and processor related, not Protocol.

The technical limitations of a protocol should not limit the size and

throughput of the hardware you implement it on.

An ideal cost to coast path could use from 100 (30 miles apart) to 300

switches. A 256 byte ROSE packet at 56kb takes 0.04 sec to transmit if you

had full duplex or no contention backbones. (0.04 * 100 = 4 sec)

The real network would have txdelay's other traffic, but a delay of 60 sec is

a reasonable goal.

Many paths through the US will always have 1200 baud hops. This would cause

the delays to go up drastically, say 5-10 mins.

The point to be made for ROSE is that the routing table can support an

unlimited number of switches in the network. Which means no matter how large

the network is you can reach any switch in the network with a single network

connect command.

tom, w2vy

-----------------------------------------------------------------------------------------

>All this discussion about TCP/IP and ROSE is fun,  and they are both

>good and I'd rather see one or both of them in place than something

>like NETROM,  but there is a reason why many are not going to ROSE

>right now,  and that's hardware.

ROSE runs on many TNC's all Z80 based. There is also a verison ported to

the PC. It only supports Async ports at this time, this could change.

I also have in my hands a Z182 board, which I am in the process of porting

the code to.

What's a Z182? It is a Larger, Faster Z80 w/1MB address space, 2 Sync ports,

2 Async Ports, Timers, 1 FDX channel of DMA.

The expectation is that the DMA port will be able to run in excess of 56k,

(more like 230kb IMHO) and very likely 56kb on the other port and maybe

a pair of 9.6kb channels with the addition of a second SCC.

The board is from PacComm which they say is the begining of a new line of

Z180 based boards.

I was considering 1K packer sizes, could do 2K, I guess.

>For those people that dont' have money to put up PC's at sites and all

>they have is a couple of tnc's for a switch, personally I'd recommend

>ROSE to them over NETROM anyday, but for anything more powerful it

>usually ends up being a PC running TCP/IP.

This can change. I think that a Z182 @ 20mhz or 33mhz will leave most PC's

in the dust... time will tell since the code isn't ported to the z182 yet...

the boards are real! that is usually the hard part !

_________________________

Thomas A. Moulton, W2VY

w2vy@ram.net

-------------------------------------------------------------------------------------

>ROSE, Netrom (and from what I know of them Texnet and Flexnet) seems aimed

>at Joe BBSUser.  Any other usage is an add-on, an afterthought.

ROSE has been designed from the start to provide full AX.25 connectivity,

without repsect to the upper layer protocol being used.

This is proven by the fact that ROSE passes ALL AX.25 PID's and right now

I'm working on supporting UI frames to complete the support and while I am

at it I want to support broadcasting, which can help IP as well as others.

Almost every feature in ROSE in generic for all protocols.

tom

-------------------------------------------------------------------------

X1J / SPRINT (PacComm)

I have a Sprint which was used for a week with X1J with a 19.2 kbaud

diode matrix. (I tried it with *both* PacComm U14 mods.) On Friday night

I pulled it from service because of the large number of L1 RX

RS232 errors in the stats displays of the *other* nodes

in the stack. The Sprint display always had zero errors.

PacComm does not have a solution yet. I removed

the Sprint and replaced it with a TinyII MK2 and the stack is back

to normal operation.

This is not an isolated case because a sysop to my north is

having the same problems. I'm sure either PacComm, I, or someone

else will find a solution but I thought it was worth mentioning.

I like the TNC but the schematics are very poor. I hope everyone

complains as loudly as I have about the quality.

John, WB3ESS

-----------------------------------------------------------------------

NEWS!

I just got off the phone with Chuck at PacComm. They reproduced the

problem. The fix is to move Jumper JP17 to pins 1 & 2. The manual

tells you to use other pins. When you make this change the dual

color LED should be green most of the time. When data is being Rx'ed

this LED will go out. When transmitting data, the LED will turn

RED.

I am using X1J firmware with a diode matrix. Since CTS wasn't

being checked, this jumper change may help the ROSE users.

Soon as I can get to the node site I'll give this a try. The sysop

to the north has a node stack at home so he will be able to test this

tonight.

John, WB3ESS

-----------------------------------------------------------------------

Date: Thu, 03 Nov 1994 11:49:39 -0700

From: mmunster@qualcomm.com (Marvin J. Munster)

Subject: X1J2 PARAMATER SURVEY

The following is a list of node parameters the I am currently experimenting

with for our local node here in Escondido (San Diego, CA).  I know I am

leaving myself wide open for all kinds of comments in both directions,

however I have to respect the great tallent pool that is present on this

reflector.  I would like any and all comments and opinions regarding the

settings listed below, good and bad. What are your settings and particular

situation for your nodes?

At this time, this is a single 2 meter node with no other prots (yet).

There are a couple of neighboring nodes that I will lock in at a later date

when the parms are firmed up.  By the way, I am interested in the quality

settings of your neighboring nodes, and why you locked them in where you did.

To view this table, you should set your screen or printer font to 10 point

courier.  This will prevent wrap-around in the table.  I hope you can read

this thing.  Columns in a text file are terrible, but I did my best. :<

Column  A   represents the letters in the menue of the combining program.

Column  B   is the description

Column  C   is what this description is called in the program

Column  D   is the parameter number in version 2.06 of TheNET

Column  E   is the parameter number in version 2.08 of TheNet

Column  F   is the software default

Column  G   is what the settings I have currently set in the E-Prom

A       B                                    C               D     E     F    G

________________________________________________________________________________

TheNET Version--->     2.06  2.08  X1J2 X1J2

Parm  Parm  Parm Parm

Nr.=TheNET Parm Number--->      Nr.   Nr.  Dflt ESCN

A       Level 1 parameters

AA  PERSISTANCE (n/256)                 Persistance         16    26    64    64

AB  Slot Time (10's of ms)              SlotTime            17    27    10    20

AC  Full Duplex                         FullDuplexFlag                         0

AD  Send Flags if no data (full duplex) FullDuplexSendFlags                    0

AE  Transmit keyup delay (10's of ms)   TxKeyupDelay                          20

AF  CWID repeat period (Sec)            CwidPeriod                             0

AG  CWID keyer speed (dot spd 10's ms)  CwidSpeed                              6

AH  RS232 host mode (normal/DCD mode)   HostMode                               1

AI  RS232 corsslink protocol            CrosslinkProtocol                      0

B       Level 2 parameters

BA  Node callsign   NodeCallsign                                        WB6PKK-8

BB  FRACK (T1) time (seconds)           L2T1Timer           18     7     6     8

BC  AX.25 window size (packets)         L2WindowSize        19     8     7     2

BD  AX.25 retries                       L2RetryCounter      20     9    10     8

BE  Ack (T2) time (10's of ms)          L2T2Timer           21    28   100   110

BF  Active check (T3) time (10's ms)    L2T3Timer           22    29     0 18000

BG  Digipeat                            L2DigipeatFlag      23    10     0     0

BH  Callsign Validation                 CallValidateFlag    24    11     1     1

BI  MHeard list length (0=disabled)     MHeardTableSize                       20

BJ  Digipeat up/downlink control        L2DigiUpAndDownlinkCtrl                0

C       Level 3 parameters

CA  Node alias                          NodeAlias                           ESCN

CB  Size of destination node table      NodeListSize =       1    17   100   150

CC  Minimum auto update quality         NodeAutoUpdate       2     1    86   150

CD  HDLC (radio port) default quality   HDLCPortQuality      3     2   192   150

CE  RS232 (crosslink) default quality   RS232PortQuality     4     3   255   255

CF  Initial obsolescence count          InitalObsoles.       5     4     6     6

CG  Minimum obsolesc. count for b'cast  MinimumObsoles.      6     5     5     5

CH  Nodes broadcast interval (seconds)  B'castInterval       7     6  1800  3600

CI  Initial 'time to live' (hops)       NetwkTimeToLive      8    18    32    32

CJ  Selective nodes broadcast on ports  SelectivePortB'cast                    1

CK  Xlink port node b'cast interv. (s)  CrosslinkB'castTime                    0

CL  Node broadcast algorithm port cntl  NodeBroadcastAlgorithm                 0

CM  Hash node broadcast port control    HashNodeB'castControl                  0

CN  Control of Slime Trails

D       Level 4 parameters

DA  Transport FRACK timeout (seconds)   TransptFrackTimer    9               180

DB  Transport retry counter             TransprtRetryCounter10    20     3     3

DC  Transport acknowledge delay (sec.)  TransprtAckTimer    11    21     3    10

DD  Transport busy delay (seconds)      TransprtBusyTimer   12    19   180   180

DE  Transport window size               TransptWindowCount  13    23     4     4

DF  Transport overfill limit (frames)   TransprtOverfillLimit14   24     4     4

E       Level 7 parameters

EA  No activity timeout (seconds)       NoActivityTimer     15    25   900   900

EB  Beacon mode control                 BeaconMode                             1

EC  Beacon period (seconds)             BeaconTimer                          600

ED  Beacon digipeat callsign

EE  CQ enable flag                      CQEnableFlag        26    31     1     1

EF  Information message                 InformationMessage                  Esc*

EG  Password                            Password                           -----

EH  Connect redir. (BBS,HOST,DXClstr)   ConnectRedirector=1                  BBS

EI*     User message control flags

EIA Issue a msg when trying BBS etc     TryingToConnectMsgFlag                 1

EIB Show all available cmds to sysop    SysopSeesAllCommandFlag                1

EIC Say 'goodbye' nicely                GoodbyeMessageFlag                     1

EID Give connect text on uplink to alia ConnectWelcomeMsagFlag                 1

EIE Show nodes in routes as alias:calsn ShowNodesAsAliasClsgn                  1

EIF Pass 8 bit data in TALK command     Pass8BitDataInTalkComd                 0

EIG Make node alias handling case sens  MakeAliasesCaseSens                    0

EIH Enable/Disable TexNet intrfc hndlr

EJ  Auto reconnect to node              AutoReconnectToNode                    1

F       IP Router Parameters

FA  This node's IP (Ampernet) address   NodeIpAddress                    0.0.0.0

FB  The IP (Ampernet) b'cast address    IpBroadcastAddress               0.0.0.0

FC  The default Time To Live (TTL)      IpTimeToLive                          20

FD  AX.25 ports mode control (DG/VC)    IpPortModeFlags                        0

FE  Enable or Disable the IP router     IpEnableFlag                           0

FF* Set MTU parameters menu

FFA MTU for AX.25 Lvl 2 IP on Radio pt  IpMtuPort0                           256

FFB MTU for AX.25 Lvl 2 IP on RS232 pt  IpMtuPort1                           256

FFC MTU for Net/Rom IP on L3 port       IpMtuNetromPort                      236

FFD Max data bytes in a L2 packet       IpMtuMaxL2InfoBytes                  257

FFE Total bytes in a L2 packet          IpMtuMaxTotalL2DataByt               328

G       Miscellaneous Parameters

GA  Host command lead-in char (Escape)  HostEscapeCharacter

27

H       Meter Control Parameters

HA  Meter Control Flags Menu

HAA Enable Deviation meter              DevMeterEnableFlag                     1

HAB Enable Signal strength meter        SMeterEnableFlag                       0

HAC Show Signals as 'S1..S9+'.          SMeterAsSPointsFlag                    0

HAD Enable Voltmeter channel 1          VoltmeterCh.1Flag                      1

HAE Enable Voltmeter channel 2          VoltmeterCh.2Flag                      1

HAF Divisor is 1000 not 100, channel 1  VoltmeterCh.2Divisor                   0

HAG Divisor is 1000 not 100, channel 2  VoltmeterCh.1Divisor                   0

HAH Resolution setting fine, channel 1  VoltmeterCh.2Resolution                0

HAI Resolution setting fine, channel 2  VoltmeterCh.1Resolution                0

HB  Deviation Meter Scale Factor        DeviationMeterScale                    0

HC  S Meter Noise Floor ADC reading     SmeterNoiseFloor                       0

HD  S Meter Scale Factor                SmeterScaleMultiplier                  0

HE  dBm Meter Scale Factor              SmeterDbMultiplier                     0

HF  dBm Noise Floor                     SmeterDbNoiseFloor                     0

HG  Voltmeter Channel 1 Scale Factor    Voltmeter1Multiplier                   0

HH  Voltmeter Channel 2 Scale Factor    Voltmeter2Multiplier                   0

HI  Voltmeter Channel 1 Offset Value    Voltmeter1Offest                       0

HJ  Voltmeter Channel 2 Offset Value    Voltmeter2Offset                       0

*Information Message

Escondido Amateur Radio Society (EARS) Elev. 1,600', Mt. Whitney, San

Marcos, CA

BTEXT

CTEXT

Help  (Fill in later)

You may respond to this reflector or directly to me at the address below.

Thank you in advance for all the fine help I know is forthcomming.

Marvin Munster, WB6PKK

mmunster@qualcomm.com

-----------------------------------------------------------------------------

On Dec 12,  1:34pm, ESTEY@SKYLER.MAVD.HONEYWELL.COM wrote:

> Subject: [NETSIG:459] 'RLI BBS with X1J 'PROMS

> At a radio club meeting last week, I made the statement that:

> 1.  I would like my 'BPQ version 4.07 to do IP routing but had never

>     really understood the 'BPQ IP router nor had been successful at getting

>     it to work.

> and that I liked ....

> 2.  The ability to keep the packet switch going even if the BBS is down.

>

> Then someone said, "well,  that's simple.  Instead of using the "KISS"

> code in your TNC-supplied EPROMs he suggested using a X1J EPROM.  He says

> this will work fine with JNOS.  My application is a little different:

> DesqView running 4 copies of 17.x 'RLI BBSs and 'BPQ 4.07 with 6 ports.

> So, will 'RLI run with the TNCs using X1J firmware?

Sure, just run bpq on the computer and have it talk to the

tncs on the interconnect between them.  i.e. the bpq running on

the computer is "just another KISS link on the interconnect".

An even better solution (IMHO) is to run bpq on a data engine,

let that handle all the networking / routing, and let the computer

handle all the applications via multi-drop KISS link to the DE.

This solution may or may not work ... suspect that John has not

provided quite the right interface from the computer to the

serial port of the DE so that the DE can talk to a few tncs plus

the other copy of BPQ running on the computer.

The third solution, and (IMHO) the best one is to run an old

junker 386 as your switch, and hang it on a UPS that gets power

from the same battery bank that backs your radios.

This way you don't lose the applications OR the network when

the power goes away.  You can run any (or multiple) switch software

on the 386 ... and not be limited by the capabilities of the

TheNet proms.

Speaking of those limitations of TheNet - the fact that a multi-port

switch does not share a single node/link/route table seems to me

the main disadvantage of using anything other than KISS in the

tnc itself.  It's too bad the folks who designed NET/ROM didn't

do the inter-tnc communication a little differently, and allow a

node stack to share a single (distributed) routing table.

...  Hank

>

> A related question is has anyone got the BPQ router working well?

Or even "... got the BPQ router working at all ..."

The only time it has been tried on our network, it tended to stay

up for about 90 minutes max ...

...  Hank

--

Hank Oredson @ Mentor Graphics             Library Operations

Internet     : hank_oredson@mentorg.com    "Parts 'R Us!"

Amateur Radio: W0RLI@W0RLI.OR.USA.NOAM

----------------------------------------------------------------------------------------

In article <787235530snz@kbcomms.demon.co.uk>

bigwoodp@kbcomms.demon.co.uk (Paul Bigwood) writes:

> In article <1994Dec5.215613.29443@combdyn.com>

>            lawrence@combdyn.com "Lawrence *The Dreamer* Chen" writes:

>

> > In article <3bkn5f$eai@gort.oit.umass.edu> aswNS@hamp.hampshire.edu (Albert S

> >  Woodhull) writes:

> > >Ian Wade's NOSIntro book mentions that Net/ROM nodes of verison X1G

> > >and later can act as TCP/IP routers, but I am unable to figure out how

> > >to take advantage of this. The sysop of a local node (TheNet X1J) I

> > >wish to use forwarded to me a list of parameters to include in my

> > >autoexec.nos, and he has put my call and IP number in the node's

> > >database, so it appears in the IProute list, but I have been unable to

> > >connect to the node with TCP/IP, even though I have no problem making

> > >an ax25 connection to the same node.

> > >

>

> I missed the beginning of this thread, but having gone through setting up

> a local IP router in this part of the country,I can hopefully add some

> value.

>

> X1J does indeed support AX25 digipeating, IP routing and Net/Rom routing. It

> also has an internal BBS. All these features can be enabled or disabled as

> required by the sysop. All parameters can be set/reset remotely using AX25.

>

> You can ping the IP address of the router, but not telnet to it.

>

> Normally it is installed on a site with good coverage and setup to do IP and

> Net/Rom routing only. Any router needs to know where to forward packets to

> and from and this is setup using the IProute command, with ARP add commands

> to add the AX25 station callsign. Remember that IP traffic is encapsulated

> inside AX25 packets, so the router needs to know not only the IP address of the

> station, but also its AX25 Callsign with SSID.

>

> What you need to do is to find out what the other stations that use the

> router have as their default ip route.

>

> You need to set up your default route to chuck everything that you dont have

> a direct route to, to the router. Depending upon which version of NOS you're

> using somethine along the following lines should do.

>

> Net> route add default tnc0 44.xx.xx.xx ; where tnc0 is the name of the

> interface you have. (144 and 432 are common over here) and the ip address

> (44.xx.xx.xx) is that of the router. You also need to have an ARP entry

> for your callsign as well.

>

> ARP add 44.yy.yy.yy AX25 G3WYW-7 DG; where 44.yy.yy.yy is YOUR ip address

> and G3WYW-7 is your hamcallsign with whatever SSID you use on that port.

>

> X1J really comes into its own when is it given a range of IP addresses to

> route. If you were a local node that handled ip routing for a number of

> users, then the routing table for the X1J would be setup something like this

>

> route add 44.131.6.160/28 AX25 44.131.6.4; whereby the range of ip addresses

> from 160 to 192 (/28 says mask from bit 28 to 31) are forwarded to 44.131.6.4

>

> Hope this helps. If you need more specific help, then mail me direct.

>

> 73s de Paul G3WYW

>

> --

> Paul Bigwood G3WYW                              Karibu Communications,

> Tel/FAX: (44) (0) 1635 861981                   18 The Martins, Kennet Lea,

> E-Mail : bigwoodp@kbcomms.demon.co.uk           Thatcham, Berkshire RG13 4FD

> Specialists in Mobile Communications, - Voice & Data using Radio and Wire

--------------------------------------------------------------------------

PI2

Doug Holcombe, KB5YZI wrote:

>

> I'm looking at buying a Ottawa PI2 card and I have a few questions

> swimming around in my head. Can I put a Kantronics DE-19K2/9K6 (19.2k/9600

> baud modem) on the low-speed port and get it to work or does it have to go

> on the high speed port?

We are running Ottawa PI2 cards on the 230 kbps backbone here.  One of

the hilltop sites also has several other ports on the JNOS host.  For a

short while, we tried to run a 1200 bps radio on the slow port while

running higher speed on the fast.  We discovered that the fast port

ended up going deaf quite often, apparantly associated with traffic on

the slow port but the precise relationship and cause hasn't yet been

figured out.

I haven't seen the Kantronics radio but the PI card can either use

external clocking or do its own internal recovery from the bit stream,

even at 230 kbps if you use my s/w hacks.  I should think it would work

very well on the Kantronics radio.  Furthermore, because the cards are

DMA, the system performance isn't compromised by a PC host having to run

character by character interrupt and further slowing and loading things

down.

> they sell? if this type of modem doesn't work at all with this PI2 Card

> then what type of modems are there that will work for this card. I'm

> looking to do 9600 baud right now and then move up to 56k! later on.

> are there any other types of modems (kits or pre-assemabled) that can go

> faster than 9600?

-------------------------------------------------------------------------------------

I think you'll find that the PI card works fine for this.  I guess I'd

ask "what *don't* they work with?".At the moment, we are still

experiencing some problems which seem associated with the PI driver

software which has caused the highspeed port "deafness" and crashes.

This has required that JNOS be run with frequent h/w and/or s/w resets

and we are not now operating with the slow ports at all.  OTOH, a lot of

data is flowing across the system.

Try traceroute or hop check to santarosa.ampr.org.  Hopefully it will

terminate normally (if it stops at redwood3, the PI card on redwood2 may

be deaf) and you'll see the last three hops work quite well.  All radios

are using PI2 cards.  You may also try 'finger pishow@<host>' where

<host> is one of redwood3, redwood,santarosa,k6hsj, to see the current

status of the PI2 card.  This also works on redwood2 but please avoid

this since it is floppy based and the finger causes an extra disk write

(for logging).  I really don't want to generate any more trips to the

hill than are necessary.  It's looking like we're going to have to

spring for a hard drive there as it is.

I like the PI2 card pretty well and think it is a great choice for

higher speed packet.  Of course I'll like it even better when

simultaneous correct operation of slow (interrupt) and fast (DMA) ports

is a reality (:>) Perhaps the solution to these problems will even

enable slow port operation at 9600 bps on 'reasonable' hosts (386 class

or better ).  That would make them an awfully attractive alternative to

almost any TNC2-like solution since the fastport and DMA would then come

"for free" and allow an upgrade path as you suggest.

73

Glenn Elmore n6gn

amateur IP:	glenn@SantaRosa.ampr.org

Internet:	glenne@sr.hp.com

------------------------------------------------------------------------

> Can the PI card be used with BPQ node software?

No.  The only PI drivers that exist are for NOS and Linux.  I sent mail to

G8BPQ several years ago asking him if he would provide the programming

info necessary to write a PI driver for his stuff, and never got a reply.

I don't think there is much interest on our part now anyway, even if the info

was forthcoming... but anyone else is free to take a crack at it. :-)

Barry

--

Barry McLarnon  VE3JF/VA3TCP   |  Internet: bm@hydra.carleton.ca

Ottawa Amateur Radio Club      |  AMPRnet:  barry@va3tcp.ampr.org

Packet Working Group           |  FreeNet:  aa187@freenet.carleton.ca

-----------------------------------------------------------------------

Date: Thu, 22 Dec 1994 11:05:54 GMT

From: gary@ke4zv.atl.ga.us (Gary Coffman)

Subject: BEST TNC for NETWORKING?

In article <mmunster-2012940938080001@mmunster-mac.qualcomm.com> mmunster@qualcomm.com (Marvin J. Munster) writes:

>I don't want to start a war here, but if you had it to do all over again,

>what TNC would be best to build a new network. I want to use one "brand" of

>modem that will fit all applications.

>

>        Considerations:

>

>        X1J/2

>        1200  user port

>        9600 and _up_ network

>        Power sources (12 volt DC pref., low current?)

>        Hill top operation (not in a house - you know, temp. stability, etc.)

>        Standardization (same equipment at all sites)

>        Cost - (best bang for the $)

>        (add your own here)

>

>Lets hear it all, good or bad; should or shouldn't; do or don't.

>

>Don't forget to say why you like it or dis-like it.

>

>Thanks for your input and thoughts on the subject.

We decided that the best *switch* for mountaintop use in our network

is the lowly PC, with an Ottawa PI card or cards on board. PCs are

cheap, and replacement components are available everywhere on demand.

They have proven to be reliable in the mountaintop environment. And

because they have enough memory and processor horsepower, are able to

run more advanced networking software for our 56 kb networks. The only

TNC that came close to meeting our needs was the Kantronics Data Engine,

but because it's single sourced, and more expensive than a PC clone

assembled from parts, we opted to use the PC. We started with simple

8088 systems, but as prices have fallen, use of 80386SX systems has

become standard for us. No keyboard or monitor need be left at the

site. Those can be brought along when troubleshooting or on site

updating are required. Normally, we do system maintenance and software

upgrades over the network.

We use a locally modified version of Karn's NOS as our switch software,

but now that we've upgraded to 386s, we may switch to Linux with native

drivers.

Gary

--

Gary Coffman KE4ZV          |    You make it,     | gatech!wa4mei!ke4zv!gary

Destructive Testing Systems |    we break it.     | emory!kd4nc!ke4zv!gary

534 Shannon Way             |    Guaranteed!      | gary@ke4zv.atl.ga.us

Lawrenceville, GA 30244     |                     |

TNC-95 (TAPR)

According to Barry McLarnon VE3JF:

> > Considering you didn't list cost as an objective, the "TNC-3" from

> > Germany based on a 68302, is probably the best platform to build from,

> > at least on paper. Considering the last I heard of it was in its

> > announcement in PSR it would be hard to consider "practical".

> > Sure would be nice to packet at data rates that would make a '302

> > work up a sweat.

>

> Don't forget the Gracilis PackeTen... it's also 68302-based, and it's

> available.  It would be my first choice for unattended network nodes if

> money were no object.  Unfortunately, hams who'll spend thousands on a

> fancy hf rig tend to balk at putting up, say, 100 bucks for a 10% share

> in a P10.  I guess they figure that, like the ionosphere, a packet network

> will magically materialize for free.  Of course, there's also the little

> matter of rf equipment, but you get the idea... with a large user base,

> the per-user cost could be quite modest, if they actually shared the cost.

>

Since this will appear in the upcoming PSR, I thought I might post it here a

little early.

My guess right now without seeing a final parts list is bare baord for

$30-$40, core kit / PCB right at $90+/-, options $10 or so depending on what

they are.

I would be interested in feedback, but this design is coming out to support

the DSP-93 needs, we just had the option to do a little changing to

it in order to make it more versatile.  So - please no processor debates :-)

Greg

---

TAPR TNC-95 (this is to appear in the upcoming PSR)

It would take a long article to describe the thought and effort that has

gone into the design of the TNC-95 as described below.  Some of the more

important points that have been discussed over the few months, include the

need to support:

* a low-cost data controller for internal use in the DSP-93.

* future TAPR kits that require data controllers (i.e. PCON, METCON-II,

etc.) for those that want a standalone package.

* networkers who require a low-cost versatile multi-port network controller

that can be used in a variety of situations, but will easily allow for

porting of the existing software base.  The concept of channel element or

communications co-processor could be used in some situations.

* a replacement for the TNC-2 bare boards TAPR has been offering since 1991.

* a new OEM design that can be offered by TAPR.

The process of the design began in July 1994 when a survey was conducted

with individuals who had purchased TNC-2 bare boards.  This survey was the

results of having to make a decision in regards to having more TNC-2 bare

boards produced.  The PCB house had doubled the price and we really wanted

to know if we needed to go to the effort of doing more boards or discontinue

the product.  The result was interesting -- 80% of the purchasers were only

building the data controller section for networking and leaving the rest of

the board unpopulated.  This seemed to us as a waste of cost to produce a

board that more than 50% was not being used by 80% of the people purchasing

it.  Also, at about the same time, the beta-dsp group had determined that a

TNC inside the DSP-93 was going to be required.  The decision was made to

discontinue the TNC-2 bare board and to start design of what is now called

the TNC-95.

The TNC-95 is a design that attempts to address a wide variety of needs

without requiring "payment" for unused features.  The concept currently is

that the TNC-95 will be available as a bare board, a core kit, and the

additional options that might need to be added later for different

functionality.  The selection of parts for the unit, should make it easier

to find if the builder does not wish to purchase a complete kit from TAPR.

Parts procurement for the older TNC-2 bare board was difficult.

The core consists of a Z180 MPU, an 85C30 SCC, an EPROM and two SRAM.  The

EPROM and SRAM may be any size between 32k x 8 and 512k x 8.  The core

configuration provides two synchronous ports and two asynchronous ports with

TTL level interface.

No modem is provided on-board.  The 85C30 will support a variety of speeds

to allow a variety of modems to be supported.  Since modems are not standard

from area to area, it did not make sense to provide modems on the board.  In

addition, having the modem as an external device promotes the modular

approach that this kits strives for.  Many times, the modem and radio are

tuned to each other, so this can be a benefit to many.  No other components

are required to add a second modem channel, except the modem.

The TNC-95 is designed in such a manner as to allow it to be configured for

either external use or internal (PC Plug-in) use.  To get the functionality

of an external controller, an external modem (for desired speed) and a

MAX232 chip to get RS232 drive levels are added to the core configuration.

If you want to use the card as a plug-in to the ISA bus, then instead of

adding the MAX232, you add a kit of parts that provides the ISA bus

interface.  The card looks to the PC like a COM port.  Provisions are made

to allow I/O port addresses 268H, 278H, 2E8H, 2F8H, 368H, 378H, 3E8H and

3F8H with IRQs of 2, 3, 4, 5, 10, 11, 12, and 15.  DMA operation is also

supported.   Power can be supplied from the ISA backplane or and external

12vdc supply.

The third option that may be added is a second 85C30.  This will allow the

addition of 2 more external modems.  If high-speed RS232 speeds (TBD) are

required, then one of the 85C30 ports can be used for that function.

With the above configurations, you can begin to see the potential of this

unit.  If you are running X1J, THENET, ROSE, or TexNet then no modifications

will be required in hardware, since there is enough space for the larger size

EPROMS, after code has been ported.  In addition, one TNC-95 can replace

four TNCs for many that are networking with TNCs.  The concept of having the

TNC-95 as a channel element for future networking is important.  The TNC-95

fills a niche for the 1200-19.2K+ operations and allows other amateur PC

boards available to provide higher speed access.  No highend limit can be

set until the unit is actually finished and tested.

We have been in touch with the developers of code for the TNC-2 and will be

contacting them when beta-test units are available for them to port code on.

Hopefully we will have a wide range of code available around the

introduction.

The TNC-95 will be in prototyping in January.  Timeline after that will

depend on how the prototype comes out.  A small group of beta-testers has

already been selected.  The goal is to try to have PCBs available sometime

by the middle of 1995.  More information to follow in a later PSR.

HAPN-2

The HAPN Packet Radio Multi Mode Adapter and modems

(by Hamilton and Area Packet Network)

The HAPN-2 multi mode digital controller is unique in terms of its ease of

operation. It is designed for the IBM PC and compatibles. The IBM PC and

compatible has become a leader in the Ham world due to its attractive price

and excellent performance.

The HAPN-2 is a card that plugs directly into one of the slots of the PC. It

contains a built-in modem which interfaces to the radio with a DB9 connector.

The card is 8 inches long and contains a small prototype area. The card is

able to perform many functions, the most important being the 2 packet ports.

One high speed port for an external modem and one more conventional port for 1200/600/300 BPS packet. Hardware for additional modes such as CW, RTTY,

AMTOR, WEFAX and SSTV is included on the board.  For a full detailed

technical information see: "A Digital Multi-Mode Controller for the IBM PC"

in the 11th ARRL Computer Networking Conference papers 1992.

It has been 5 years since HAPN announced the first IBM PC plug-in TNC. This

TNC ( HAPN-1, packet only TNC ) made us known in many countries. The card was known for its easy operation and user friendly software. However technology has been progressing towards multi node units and higher baud rates. Many customers asked for additional modes, such as HF packet, RTTY, WEFAX, AMTOR, SSTV, etc... and more advanced networking such as TCP/IP.

The HAPN-2 adapter for the IBM PC and compatible addresses these requirements.  Here are some of its features:

1. Excellent software. The software provided by HAPN is user friendly, easy to

operate and very powerful.

2. There is no tying up of the RS232 ports, so it will not interfere with your

mouse or telephone modem hook up.

3. There are two simultaneous channels!

Channel A

- high Speed channel, capable of running in DMA or interrupt mode. The

DMA is primarily designed for high speed data transfers which is required

for speed such as 56K baud. The interrupt mode is used for slower modems.

- the high speed channel supports three types of external modem interfacing

via the DB25 connector. They are RS232, RS422 (high speed standard) and

TTL levels. Also medium speed modems such as 4800/9600 Bps can be readily

plugged in.

Channel B

- interrupt driven port, programmable for 1200/600/300 baud with the built

in AM7910 modem, used for VHF/UHF and HF. Other modems such as 4800 and

9600 baud could be installed via the disconnect header.

4. Friendly Software

- Updated AX25 user friendly software for DOS and OS2 (including WARP VDM)

will run on this new card. They are C25 multi connect terminal emulation,

file transfer programs ASCII, SF/RF, YAPP and XPACKET. Other programs are

PCRBBS, S25 , H2TEST, H2INSTAL and HAPNCUST. The host mode software (C25)

supports split screen, function keys and pop-up dialog boxes. The

resident driver program M25 runs in the background while you are

running other programs on the PC.

5. Hardware is compatible with programs written for the DRSI PC*Packet card.

- Examples are HAPNDRSI.SYS and the DRSI TNCTSR-L.EXE (and TNCTSR-S.EXE)

drivers.  Popular software aplications such as WA8DED host software,

HB9CVV host program THS, G8BPQ, FBBS, DXCLUSTER and ESKAY are examples

that can be run.

6. Advanced Software

- TCP/IP (NOS) is supported for networking.

Both DOS and OS2 (including WARP VDM) are available.

7. Additional Hardware

- hardware is included for CW, RTTY, WEFAX, AMTOR and SSTV. An FM

synthesizer circuit lets you generate any audio frequency for the

different RTTY shifts, SSTV, etc. The decoding of receive tones is done

with timers and software.

The new HAPN-2 card is a versatile multi port packet node with great

supporting software. In addition, it can offer other modes such as CW,

RTTY etc. with software that is currently being developed. Any Ham having an

IBM or compatible computer will be surprised by it's value.

H.A.P.N  has the following products available:

$Can    $US

HAPN-2 MULTI MODE PACKET CONTROLLER

Assembled and tested board with AX25 hostpacket software and

self test program.( 6 programs and comprehensive manual )      220.00  185.00

HAPN-2 KIT

Complete PARTSKIT with construction details and AX25 host       190.00  157.00

software with manual (as above)

HAPN-2 SEMI KIT

Contains bare board, end bracket and preprogrammed pal.         95.00   80.00

Software and manual as above.

AX25 software for HAPN-2 (OS2 beta)                                                    7.00    6.00

TCP/IP (NOS) software for HAPN-2                                                         7.00    6.00

TCP/IP (NOS) software for HAPN-2 (OS2 beta)                                      7.00    6.00

TCP/IP (NOS) software for HAPN-1 (older TNC, DOS only)                  7.00    6.00

RTTY software for HAPN-2  (DOS)                                                         10.00    8.00

Uses on screen tuning indicator.

HAPN-T KIT  (see Communications Quarterly spring 1992)          60.00   48.00

4800 baud modem kit, all parts and manual

HAPN-T bare board                                                                                 18.00   15.00

4800 baud modem board and manual

Software updates are for registered users only.

(specify HAPN-1 or HAPN-2 adapter and diskette format 3.5" or 5.25".

HAPN-1/2 update AX25 host update with manual                         7.00    6.00

HAPN-1/2 update UTILLETY SF/RF YAPP XPACKET                       7.00    6.00

HAPN-1/2 update PCRBBS (1 disk)                                                    7.00    6.00

Shipping and handling : $6 for US and Canada, $9 elsewhere.

Use checks or money orders payable to H.A.P.N. (outside Canada and US

use International checks or money orders only).

For ordering or more information contact HAPN on the new address:

HAPN                          Phone: 905-692-3802

5193 Whitechurch Rd.

Mount Hope, ONT

L0R 1W0,  CANADA

Internet: john.vandenberg@canrem.com

Packet: VE3DVV@VE3DTV.#SCON.ON.CAN.NA

PacComm PRODUCTS

PacComm Network Equipment and Accessories

The PacComm 'Node Series'

The NODE series of packet controllers are specially priced to stretch the

network budget. Quantity purchase discounts are available.

The NODE models are based on the TINY-2 MK-2 TNC which offers a dual EPROM

image capability. Put your favorite network firmware in one half of the

EPROM and standard AX.25 code, or diagnostic firmware in the other half.

Really simplifies testing of equipment at remote sites! Easily adaptable

for use by bank-switching public domain network firmware.

Several models feature a 10 MHz CPU, SIO and fast RAM and EPROM for

increased performance at busy sites. These units operate the CPU at 9.8 MHz

clock rate.

All models support RS-232 port data rates up to 38.4 kbps for efficient

communication between units in a 'node stack.' Running the 'wire side' of

the node stack at that speed can significantly improve response and

throughput. This configuration easily supports the 19,200 bps RF data rate

without strain.

PacComm DFM modems may be easily altered to operate at 4800 bps or 19,200

bps or other desired speeds. These alternate speeds are available from the

factory. Instructions for changing speeds are included in the modem manual.

RF data speeds greater than 9600 bps with the PacComm DFM modem require a

bandwidth which is only authorized for 222 MHz and above. 4800 and 9600 bps

models are compliant with FCC regulations for use on the 6 and 2 meter

amateur bands.

The DFM modems attach directly to the modulating varactor of the transmiter

and the receiver discriminator. Not all radios are suitable for use with

this type of modem. Operation at 19,200 bps will require modifications to

the IF section of almost all radios.

The TINY-2 / NODE and TINY-2/ 10MHz/ NODE

The TINY-2/NODE is the workhorse of the amateur packet network. It is

shipped without an EPROM to reduce the cost and is ready to accept the

customer's NODE EPROM. The unit is based on the TINY-2 MK-2 TNC which

supports serial and radio port speeds up to 38,400 bps.

The unit is available from the factory with the 10 MHz option installed.

User upgrade to 10 MHz operation requires only the replacement of four

socketed ICs and resetting of a push-on jumper. The 10 MHz model is

supplied with a blank 'fast' EPROM since customer supplied EPROMs may be

too slow for the 9.8 MHz processor.

The TINY-2 /9600/NODE<R> and TINY-2/9600/10MHz/NODE

The TINY-2/9600/NODE is a TINY-2/NODE with an MC-NB96 DFM modem attached

and ready to use. Supplied without an EPROM.

For busy high traffic packet nodes, the 10 MHz version of the TINY-

2/9600/NODE offers faster response and increased throughput. This model is

supplied with a blank 'fast' EPROM since customer supplied EPROMs may be

too slow for the 9.8 MHz processor.

The IPR-NB96/10MHz/NODE

The IPR-NB96/NODE is a special model of the IPR-NB96. The modem switching

features of the standard IPR-NB96 have been eliminated. The unit operates

only at 9600 bps and cannot switch to any other modem or speed. The open

squelch DCD feature is not provided. CPU operates at 9.8 MHz. A blank

'fast' EPROM is provided.

The unit is ideally suited for adding 9600/19,200 bps network links to

existing node sites. It is also a convenient way to add 9600 bps access to

a BBS system.

The NX2P EZ-Matrix

The EZ-Matrix is a diode matrix board for connecting multiple NODE TNCs

together. It uses DIODE arrays in DIP packages which greatly simplify

construction. Node cables are attached to board mounted connectors.

RF Data Transceivers

We can provide the right RF solution for your requirement.

PacComm is an authorized dealer for E.F. Johnson, Maxon, Motorola, Repco,

Standard, TEKK, and other brands.

PacComm IPR-NB96 UHF models use the TEKK KS-900L Data Transciever. Units

are stocked on frequencies of 441.050,  .075, and .100 MHz.

Xanthus Digital Regenerator

The XDR is a full duplex frame regenerator for AX.25 repeaters with the

following features:

- Operates in any TNC-2 compatible TNC.

- Full audio regeneration

- Busy Tone while carrier is detected on the input

- Only repeats valid frames

- Operation to 9600 Baud

- Provides consistent deviation on transmitter

- Transmits HDLC ID every 9.5 Minutes

You no longer have to adjust the audio transfer of your repeater to

accomodate a wide range of user audio characteristics. Will not repeat

audio, can not be used as a voice repeater.

The RADIO MULTIPLIER-II

The NX2P Radio Multiplier-II  lets you take maximum advantage of your

dedicated packet station. Designed by Bill Slack, NX2P Electronics. Allows

up to four TNCs or TNC-like devices to share one radio. Each TNC can

transmit and receive packets over the air.

- Distributes power to all the TNCs

- Allows communication between all the TNCs.

- When any one of the TNCs is transmitting data, audio feedback causes DCD on the

other TNCs preventing collisions.

- Uses standard 5 pin DIN cable connection from radios.

Take advantage of hams who live at good locations to provide those

neccessary network links by putting a network NODE/SWITCH in parallel with

their dedicated packet radio and antenna.

Experiment with new networking protocols  in a real networking environment while still using your existing network.

Place a monitor TNC and terminal in parallel with your BBS for monitoring

and personal packet operation without having to take the BBS off the air.

Run TCP/IP and AX.25 simultaneously thru the same radio.

The NX2P RADIO MULTIPLIER allows up to four TNCs to share one radio by

diode OR-ing the PTT control signal from each TNC and combining their audio

outputs. This combined audio signal is then fed to the transceiver's

microphone input as well as being amplified and combined with the

transceivers receive audio before being fed to each of the TNC's receive

audio input. The audio feedback prevents more than one TNC from keying the

radio at the same time and provides the means for all the TNCs to

communicate between each other.

All the audio inputs are high impedance and all the audio outputs are a

relatively low impedance. The RADIO MULTIPLIER connects to the radio via a

5 pin DIN socket using the standard pinout found on most TNCs. Each TNC is

connected to the RADIO MULTIPLIER via its own cable (cable not supplied) to

its own connector on the back of the RADIO MULTIPLIER.

The RADIO MULTIPLIER requires a single 12V supply for operation. Unswitched 12V from the RADIO MULTIPLIER's input is available at each of the TNC connectors to make supplying power to the TNCs easier.

The RADIO MULTIPLIER is packaged in a TNC-2 style cabinet.

PacComm/TI3DJT CoaxLAN

The PacComm/TI3DJT CoaxLAN adapter was designed to be used in ROSE

networking applications where many ROSE switches need to be linked together

using the switches serial ports.

The CoaxLAN is a small circuit board which mounts into a DE-9 cable

housing. The board has an 8 pin connector which connects directly to the

TTL serial port of a TNC.  The cable of the CoaxLAN is fitted with an RCA

jack allowing locally available parts to be used in fabricating

interconnecting cables.

Two LEDs are provided to allow monitoring data into and out of each switch.

The advantages of the Coaxlan over other means of interconnecting ROSE

switches include:

- Increased fan-out

- Convenient connections

- Lengthy cables between node TNC's allowed

- Allows 38.4 kb data rate between switches when used with the TINY-2 MK-2

Node

The CoaxLAN adapter connects directly to the TTL port of the TINY-2 MK-2.

Older TINY-2 and MICROPOWER-2 and MFJ TNCs can also be used if 5vdc is

jumpered to the TTL connector to power the CoaxLAN.

OptoPcScc (Netherlands)

Date: Mon, 19 Dec 1994 22:43:39 GMT

From: rob@pe1chl.ampr.org (Rob Janssen)

References<3co0pc$4kd@feenix.metronet.com> <D11v8E.2wo@pe1chl.ampr.org>, <3d404e$imo@feenix.metronet.com>

Reply-To: pe1chl@rabo.nl

Subject: Re: 4800 Baud ( was 2400+ Baud modem chips)

In <3d404e$imo@feenix.metronet.com> kf5mg@kf5mg.ampr.org writes:

>    Thanks for the info Rob.  Now I want to ask about the SCC Card.  :)  Is

>that a Baycom SCC card I've heard about?  Where can I get more info on the

>SCC card you like to use?  How much do those things cost, compared to a

>TNC.  In the US...  we can get a TNC-2 clone ( MFJ-1270c ) for around $120

>and a G3RUH modem is pre-built, ready to go, board for around $105.  It

>drops into the TNC-2 and plugs into the modem disconnect header.

There exists an SCC card from Baycom, which is already mentioned in another

followup.  It includes modems, which can be convenient to the end-user but

is usually not so convenient when building a node.  I think they have a

special card for that.

In the Netherlands, many people use the so-called OptoPcScc card which

has been designed by PA0HZP and myself.  It is a four-port card without

modems, but featuring optocoupler isolation of the digital signals.  This

has some advantages w.r.t. grounding loops and surge protection.

The card goes for the equiv. of about $100 as a kit.  Modems that have TTL

interface can be easily connected using a simple interface consisting of

a 74LS14 and 4 resistors to interface to the couplers.

Modems with this interface on-board are available for 1200, 4800, 1200+4800

and G3RUH-type (9600 or higher) locally.  Prices range from about $25

to over $100 (I am not sure about the G3RUH price but I think it is well

over $100.  there are quite some components on that modem, and some are

expensive.  When you see a modem for significantly less, make sure it is

a genuine G3RUH modem and not a K9NG, which does not have the digital

filtering)

For a single 1200 bps channel the above kit is about the same price as

a TNC (about $130 total), but the advantage becomes clear when one wants

to operate more than one channel.  E.g. 1200 bps on 2 meter, and 1200+4800

or maybe even 9600 bps on 70cm as well.  In that case one SCC card and

a couple of modems is a lot less expensive (note that you don't need COM

ports either, which saves as well)

The PA0HZP OptoPcScc card has vectored interrupts, and a single interrupt

can be shared with 3 cards, yielding 12 channels.  It is in wide use in

nodes in the Netherlands in this configuration.  (2 or 3 cards)

>    I haven't had 100% success with 9600b on 2 meters. I'd like to try a

>4800b link to see if through put is better of a good 4800b link compated

>to a marginal 9600b link.

The main difference between HAPN and G3RUH is that HAPN does not need the

coverage from 10Hz.  This makes it a lot easier to use with FM-only PLL

synthesized transmitters.  It can even be used with Phase-modulated

transmitters.

When you are using such a transceiver (PLL) the switchover to HAPN will

give a lot better eye-pattern and thereby better performance at lower

signal levels.  When your transmitter is already performing well (crystal

controlled or all-mode transceiver) the difference is less clear, and will

probably be offset by the lower datarate.

>    Also... I'd like to look at using a SCC card instead of a TNC-2 clone.

>I've heard that there is a 4 port, Baycom SCC card but haven't seen

>anything that talks about it, how much it cost, where to get it, etc.

>Anyway if you ( or anyone else ) can tell me where I can fing ordering

>info for SCC cards or HAPN modems, I'd appreciate knowing about it. Thanks.

Ordering the OptoPcScc card described above can be done through PA0HZP.

His address details are:

Address: H.Z.Peek

Postbox 329

1440 AH Purmerend

The Netherlands

Email:	 PA0HZP@PI8NVP.NLD.EU (ax25 BBS)

pa0hzp@pi8nos (ax25 smtp)

henkp@nikhef.nl or henkp@nikhefk.nikhef.nl (internet, eunet, surfnet)

Phone:	 +31 2990 30977

73,

Rob

--

-------------------------------------------------------------------------

| Rob Janssen                | AMPRnet:   rob@pe1chl.ampr.org           |

| e-mail: pe1chl@rabo.nl     | AX.25 BBS: PE1CHL@PI8UTR.#UTR.NLD.EU     |

-------------------------------------------------------------------------

Messages (postings) are not necessarily in chronological order or in the same section as the original thread.

See also separate file on IP routing.