Proposal

A High-Speed Multicast Capable Packet System

for the 219 Mhz band

By: Jeff King WB8WKA, jking@merit.edu, wb8wka@detroit.ampr.org

Date: 9/4/94

On August 19-21 1994 I had the pleasure of attending the ARRL

Digital Networking conference in St. Paul Minnesota. One of the

committee meetings that I attended was a joint session of the

futures mode and 219Mhz committee. One of the goals of the

219Mhz committee was to occupy the band once we obtain it. Phil

Karn KA9Q proposed the band, in part, be used for multicasting.

What I propose here (and there also) is a system that will be

multicast capable (when the protocol issues are ironed out) and

be put to immediate use today.

One of the problems with packet is that it is half duplex. Even

at medium speeds, such as 9600 bps, you are lucky if you can

utilize half the channel capacity with our current system. While

you certainly could operate full duplex, this would preclude

others use of the channel due to the full occupancy of both of

the channels (RX and TX).

Statistics on people's usage of the internet show that the

majority of the data exchanges are asymmetrical. That is, you

receive far more then you send. This factor can be to our

advantage, both from a technical as well as economical

perspective in amateur radio tcp/ip.

A system like this is already in use on the amateur Pacsats.

There are multiple low speed inputs with one high speed output.

I propose a similar system for terrestrial use. With the high

speed output on the 219mhz band (and users receiving there) you

have greater control over placement of transmitters, which will

be a requirement of the 219Mhz band that is a shared band.

The system would consist of one high speed output

(128Kbaud-256kbaud) on 219 or 222/223 MHz and multiple medium speed (9600 baud) inputs on different bands, lets say 440 for now (but it just as easily could be 2m 9600 baud). The high

speed channel could be based on a GRAPES MSK modem, transmitter

only. This would feed a omni directional antenna. Power output

would vary depending on coverage required. The user inputs could

be based on TEKK 440 data radios (or DR-1200). Antennas would be

shaped coverage (beams or otherwise) directed towards the user

area one wished to cover. Initially the user inputs (440

9600bps) could be simplex based, depending on height/coverage,

one might want to make them mini repeaters so one could get DCD

and eliminate hidden terminals. This has another advantage. Lets

say you are running 4 9600 inputs at about 1 watt ( User Areas,

North, East, South, West). You'll need to space them at least

1mhz apart if you are at the same site and running 1 watt or so.

If you run them as repeaters/DDR's (not hard at all at the 1

watt level), you can have them on adjacent channels. I.E.

441.075/446.075 441.1/446.1 441.125/446.125 441.150/446.150.

On the digital hardware end, life is simple also. The Ottawa

'PI' board would be a perfect fit for this, both from the client

(user) and server end. The PI board is a $125 (U.S) board that

has two ports on it similar to a DRSI with the exception one

port is DMA based. Meaning, you can run 56Kbaud on XT based

machines with ease. Obviously, things work much faster on better

class machines. The TEKK radio is in the 100-110 dollar range. A

TAPR 9600 baud modem would set you back around $70. The server

would be a 386+ class machine with multiple ports.

The real beauty of this plan is its scalablility, system

economics and pay as you go. Remember I mentioned it could be

done today with existing hardware. A user (client) would

initially purchase the 9600 baud equipment. With this, he could

communicate with users on his LAN as well as do standard half

duplex routing through the server/router. This is what we have

today on the 2m 9600 baud as well as 440 9600 baud channels in

the Detroit area. However, once the user wanted to upgrade, he

would just need to purchase a 219mhz receive ONLY modem/

receiver, which could consist of simply a wide band police

scanner or a low cost hamtronics receive transverter. Due to

multipath concerns, all user stations would need to use a beam

antennas for the high speed channel (pointed at the server).

System economics is where this really shines also. That is, the

system cost (add all the user costs+the server costs) is less

then with standard high speed systems. If the server system

invests in a higher power/better coverage high speed channel

(and remember, there are no hidden terminals due to the fact

there is only one transmitter on the server frequency) then the

total system cost is reduced. This is due to the fact the users

do not need to invest heavily in high antennas/towers to receive

the site in addition they only need be concerned about RECEIVING

the site at high speed, there transmitting is only done locally

with modest (inexpensive) equipment. Initially, the user station

would do symmetrical routing (as would the server), that is, out

the same port he receives on. (User: route add server.ampr.org

440, Server: route add user.ampr.org 440) When he added his high

speed 219 receive port, his routing then would be the same

(route add server.ampr.org 440) but the server would return to

him on the high speed 219 channel (route add user.ampr.org 219).

Now, how about some of the potential problems? When grabbing

stuff from the internet at high speed, there will be plenty of

'holes' on the medium speed channels to get a 'packet in

edgewise' due to the fact most acks are quite short. Since the

high speed channel is being controlled by a router (its  NOT a

DDR), it will be easy to interleave packets even if it is keyed

continuously. User stations will be able to communicate amongst

themselves as they do now (on simplex). Since the 440 (2m)

radios will be RX and TX, DCD will be maintained reducing

collisions. The only potential problem we will have is if a user

wishes to SEND a file to the internet. Then we would run into a

problem that the 9600bps channel would be continuously occupied

(with acks coming back at >56Kbaud on 219). I believe this

problem could be solved in software. Obtaining a high speed

internet feed could be a problem, but here in the Detroit area

we have two well-connected sites (WSU and MERIT) that could feed

the multicast server (which is a high site by definition).

OK, so how can we do this NOW? We can start on a 56kbaud TX

channel in the 222/223 range... we may have to be a bit

creative, i.e., horizontal polarization, overlay on top of

distant repeater outputs, but technically it is very do-able.

When (if) we get the 219Mhz band, we move the server down there

and up the speed to >128Kbaud. The user would simply need to

move his receiver as all his hardware will still be capable of

the higher speed (on AT class and above). And of course, our

medium speed user channels are already going into place.

Now I've just talked about a system that could be implemented

today. Like high speed access to Mosaic (faster then any phone

line), WWW, FTP's, Archies, you name it. These are all

client/server applications. How about multicasting?

How would you like to receive the entire USENET feed? How about

all the AX.25 bulletins? Would you like to watch NASA select

(the space shuttle liftoffs) video from you PC screen? Listen to

internet talk radio? Have multi-media (video/audio)

convers/roundtable's? All this is possible (and much of it

already happening) with multicasting. With multicasting you

don't acknowledge every transmission and multiple users can

receive the same data. This is going to be much of the future

both for the internet side as well as AX.25 side for

bulletin/information distribution. And one other possibility of

amateur multicasting, that was mentioned at the DCC, is the fact

is that a SWL (DWL?) will be able to receive most of this

information. This would also attract users (SWL's, DWL's) to the

hobby.

So folks, what do you think? It won't take a rocket scientist to

do this and it can be done TODAY with off the shelf equipment.

It will have to be a group effort though as many things would

have to fall into place, but it CAN HAPPEN if YOU wish it to.

Speak up if you think its a good idea, bad idea, questions, or

have some thoughts.. I personally want to get high speed

internet access and this seems to be the most economically way I

have run across yet. Remember, if you do nothing, nothing will

happen. Lets here from you. Comments, questions to

semcon@detroit.ampr.org

73   Jeff King   WB8WKA@detroit.ampr.org, wb8wka@wu8c.#semi.mi.usa